***Sub Engineer***

1. Pick up the correct statement from the following :

a. the principal axes of both the lenses coincide the optical axis of the telescope

b. the lens nearer the object to be viewed is convex and is called objective

c. the lens nearer the eye is called eyepiece

d. all the above.

2. Tacheometric formula for horizontal distances using horizontal sights can also suitable be employed for inclined sights through θ by multiplying

a. the constants by cos2 θ

b. the constants by cos θ

c. the constants by sin θ

d. the multiplying constant by cos2 θ and additive constant by cos θ.

3. The back staff reading on a B.M. of R.L. 500.000 m is 2.685 m. If foresight reading on a point is 1.345 m, the reduced level of the point, is

a. 501.345 m

b. 501.340 m

c. 504.030 m

d. 502.585 m.

4. The minimum range for sliding the focusing lens in the internal focusing telescope for focusing at all distances beyond 4 m is

a. 5 mm

b. 10 mm

c. 15 mm

d. 20 mm.

5. Laterite is found in

a. U.P.

b. Punjab

c. West Bengal

d. Kerala.

6. The rocks which are formed due to pouring of magma at the earth's surface are called

a. Plutonic rocks

b. Hypabyssal rocks

c. Volcanic rocks

d. Igneous rocks

7. Sea sand used in structures causes

a. dampness

b. efflorescence

c. disintegration

d. All of these.

8. Pick up the rock which is not  a sedimentary rock from the following:

a. sand stone

b. gypsum

c. dolerite

d. lignite.

9. The locus of the moment of inertia about inclined axes to the principal axis, is

a. straight line

b. parabola

c. circle

d. ellipse.

10. The load on a spring per unit deflection, is called

a. stiffness

b. proof resilience

c. proof stress

d. proof load.

11. To ascertain the maximum permissible eccentricity of loads on circular columns, the rule generally followed, is

a. middle half rule of columns

b. middle third rule of columns

c. middle fourth rule of columns

d. none of these.

12. A three-hinged arch is said to be :

a. statically determinate structure

b. statically indeterminate structure

c. a bent beam

d. none of these.

13. The property of steam function ψ is :

a. the flow around any path in the fluid is zero for continuous flow

b. the rate of change of ψ with distance in an arbitrary direction, is proportional to the component of velocity normal to that direction

c. the velocity vector may be found by differentiating the stream function

d. all the above.

14. With a clinging nappe of a weir, the excess discharge, is

a. 8% to 10%

b. 18% to 20%

c. 25% to 30%

d. 30% to 40%.

15. The value of kinetic energy correction factor (a) for a laminar flow through a circular pipe, is

a. 1.0

b. l.5

c. 2.0

d. 2.5

16. The total pressure force on a plane area is equal to the area multiplied by the intensity of pressure at its centroid, if

a. area is horizontal

b. area is vertical

c. area is inclined

d. all the above.

17. Pick up the correct statement from the following:

a. The void space between the soil grains, is filled partly with air and partly with water

b. In perfectly saturated soil, the voids are completely filled with water

c. In dry soil, the voids are completely filled with air

d. all the above.

18. The degree of saturation of the soil sample stated in Q. No, 216, is

a. 60%

b. 62%

c. 64%

d. 66%

19. Pick up the correct statement from the following:

a. When stress decreases, void, ratio decreases

b. When stress decreases, coefficient of permeability decreases

c. When stress decreases, coefficient of volume change decreases

d. When stress decreases void ratio, co-effi-cients of per-meability and volume change decrease.

20. When a cohesionless soil attains quick condition, it looses

a. shear strength

b. bearing capacity

c. both (a) and (b)

d. neither (a) nor (6).

21. In a singly reinforced beam

a. steel possesses initial stresses when em-beded in concrete

b. plane sections transverse to the centre line of the beam before bending remain plane after bending

c. elastic modulii for concrete and steel have different values within the limits of deformation of the beam

d. none of these.

22. Bottom bars under the columns are extended into the interior of the footing slab to a distance greater than

a. 42 diameters from the centre of the column

b. 42 diameters from the inner edge of the column

c. 42 diameters from the outer edge of the column

d. 24 diameter from the centre of the column

23. A continuous beam shall be deemed to be a deep beam if the ratio of effective span to overall depth, is

a. 2.5

b. 2.0

c. less than 2

d. less than 2.5

24. The transverse reinforcements provided at right angles to the main reinforcement

a. distribute the load

b. resist the temperature stresses

c. resist the shrinkage stress

d. all the above.

25. The foundations are placed below ground level, to increase

a. strength

b. workability

c. stability of structure

d. all the above.

26. To support a heavy structure in sandy soil, the type of foundation generally used, is

a. raft footing

b. pier footing

c. strap footing

d. none of these.

27. For a good concrete

a. cement should be sufficient to produce the required strength

b. water should be free from organic materials

c. mixing of ingredients should be done thoroughly so as to produce homogeneity

d. All the above.

28. The shrinkage of concrete

a. is proportional to water content is the mix

b. is proportional to cement concrete

c. increases with age of concrete

d. all the above.

29. Permanent hardness of water can be removed by

a. adding lime

b. adding chlorine

c. boiling

d. zeolite process.

30. Pick up the correct statement from the following :

a. Wrought iron pipes are lighter than cast iron pipes

b. Wrought iron pipes corrode quickly and are less durable

c. Wrought iron pipes coated with zinc, are called galvanised iron pipes

d. All the above.

31. Dry weather flow is :

a. average monthly rate of flow

b. average annual rate of flow

c. water supply allowance per capita

d. none of these.

32. The coagulant which is generally not used for treating the sewage, is

a. alum

b. ferric chloride

c. ferric sulphate

d. chlorinated copperas.

33. Pick up the correct statement from the following:

a. The full supply level of a canal should be above ground level

b. According to Lacey, regime conditions require a particular slope for a given discharge and silt factor

c. In case the ground slope is less than the required bed slope, the silt factor must be reduced by permitting the entry of coarse silt

d. All the above.

34. A current meter measures the velocity of flow, if it is held

a. on the surface of water

b. at the bottom of channel

c. at any point within the cross-section

d. none of these.

35. If A is the area of the surface, x is the depth of its C.G. from the surface of the water and ω is the density of water, then

a. total pressure on the surface is equal to ω x

b. depth of the point at which total pressure acts is equal to its moment of inertia divided by A x

c. depth of the centre of pressure is 2/3H vertically below the surface

d. all the above.

36. Attracting groynes are built

a. perpendicular to the bank

b. inclined down stream

c. inclined up stream

d. none of these.

37. Pavement is said to be flexible if it contains

a. stabilised soil base constructed of lime cement or tar

b. bitumen-bound stone layer of varying aggregates

c. lean concrete base

d. all the above.

38. If cross slope of a country is greater than 60%, the terrain is classified as

a. rolling

b. mountainous

c. steep

d. plain.

39. Formation width of a hill road is the distance between

a. outer end of side drain to outer end of parapet wall

b. outer end of side drain to inner end of parapet wall

c. inner end of side drain to outer end of parapet wall

d. inner end of side drain to inner end of parapet wall

40. If a Lemniscate curve having an angle of deflection Δ, is transitional throughout, the maximum polar angle is

a. Δ/3

b. Δ/4

c. Δ/5

d. Δ/6

41. The ground surface slopes 1 in 50 along a proposed railway embankment 150 m in length. The height of the embankment at zero chainage is 0.5 m, the width is 11 m and side slopes 2:1. If the falling gradient of the embankment is 1 in 150, the quantity of the earthwork calculated by prismoidal formula, is

a. 3250 m3

b. 3225 m3

c. 3275 m3

d. 3300 m3

42. The concrete work for the following part of the building of specified thickness is measured in square metres

a. Floors

b. D.P.C.

c. Wall panels

d. All the above.

43. While estimating a reinforced cement structure, the omitted cover of concrete is assumed

a. at the end of reminforcing bar, not less than 25 mm or twice the diameter of the bar

b. in thin slabs, 12 mm minimum or diameter of the bar whichever is more

c. for reinforcing longitudinal bar in a beam 25 mm minimum or diameter of the largest bar which is more

d. All the above.

44. Pick up the incorrect statement from the following:

a. Lead is the average horizontal straight distance between the borrow pit and the place of spreading soil

b. The lead is calculated for each block of the excavated area

c. The unit of lead is 50 m for a distance upto 500 m

d. The unit of lead is 1 km where the lead exceeds 2 km.

45. The time which results in the leasi, possible construction cost of an activity, is known

a. normal time

b. slow time

c. crash time

d. standard time.

46. Pick up the correct statement from the following:

a. Programme Evaluation and Review Technique, is event oriented

b. Programme Evaluation and Review Technique is not event oriented

c. Critical Path Method is event oriented

d. Critical Path method is event oriented.

47. The time by which activity completion time can be delayed without affecting the start of succeeding activities, is known as

a. duration

b. total flat

c. free float

d. interfering float.

48. The final selection of a construction site, is done by

a. local civil authority representative

b. representative of engineer authority

c. representative of administration

d. all the above.

49. The fuse large includes

a. passengers chamber

b. pilot's cabin

c. tail of aircraft

d. all the above.

50. The length of a runway under standard atmospheric conditions is 1800 m. If the actual reduced level of the site is 1200 m, the design length of the runway is

a. 2360 m

b. 2460 m

c. 2560 m

d. 2660 m