***Sub Engineer***

1. The bearing of line AB is 152° 30' and angle ABC measured clockwise is 124° 28'. The bearing of BC is

a. 27° 52'

b. 96° 58'

c. 148° 08'

d. 186° 58'

2. Diurnal variation of magnetic declination is

a. greater at equator than nearer the poles

b. less at equator than nearer the poles

c. less in summer than in winter

d. same at all latitudes and during different months.

3. A clinometer is used for

a. measuring angle of slope

b. correcting line of collimation

c. setting out right angles

d. defining natural features.

4. The length of a traverse leg may be obtained by multiplying the latitude and

a. secant of its reduced bearing

b. sine of its reduced bearing

c. cosine of its reduced bearing

d. tangent of its reduced bearing.

5. The portion of the brick without a triangular corner equal to half the width and half the length, is called

a. closer

b. queen closer

c. king closer

d. squint brick.

6. Which one of the following is acid resistant asbestos:

a. amosite asbestos

b. anthophylite asbestos

c. crocidolite asbestos

d. All the above.

7. The compound of Portland cement which contributes to the strength after two to three years is

a. Tricalcium silicate

b. Di-calcium silicate

c. Tricalcium aluminate

d. Tetracalcium alumino ferrite.

8. Mild steel is used for

a. structural works in beams, joints and girders

b. small sized water pipes

c. columns and struts

d. none of these.

9. The eccentricity (e) of a hollow circular column, external diameter 25 cm, internal diameter 15 cm for an eccentric load 100 t for non-development of tension, is

a. 3.00 cm

b. 3.50 cm

c. 4.25 cm

d. 5.0 cm

10. A simply supported rolled steel joist 8 m long carries a uniformly distributed load over it span so that the maximum bending stress is 75 N/mm2. If the slope at the ends is 0.005 radian and the value of E = 0.2 x 106 N/mm2, the depth of the joist, is

a. 250 mm

b. 300 mm

c. 350 mm

d. 400 mm

11. The stress in the wall of a cylinder in a direction normal to its longitudinal axis, due to a force acting along the circumference, is known as

a. longitudinal stress

b. hoop stress

c. circumferential stress

d. ultimate stress.

12. According to Unwin's formula, the diameter d of a rivet of plate of thickness t is :

a. d = 6.05 t

b. d = 1.5 t + 4

c. d = 5 t

d. d = t + 1.5

13. If a pitot tube is placed with its nose facing down stream, the liquid

a. does not rise in the tube

b. rises in the tube to a height

c. falls in the tube to a depth

d. none of these.

14. A body of dimensions 1.5 m x 1.0 m x 2 m weighs 3000 kg in water. Its specific gravity is

a. 0.8

b. 0.9

c. 1.0

d. 1.1

15. An ideal fluid is

a. incompressible

b. compressible

c. compressible and non-viscous

d. slightly affected by surface torque.

16. The discharge through a V-notch weir varies as

a. 1/H

b. H3/2

c. H7/2

d. H5/2.

17. The change of moisture content of soils, changes the

a. value of the angle of repose

b. amount of compaction required

c. cohesive strength of soil

d. all the above.

18. Pick up the correct statement from the following:

a. In hydrometer method, weight Wd per ml of suspension is found directly

b. In pipette analysis, weight Wd per ml of suspension is found indirectly

c. In pipette analysis, weight Wd per ml of suspension is found directly

d. None of these.

19. The water content of soil is defined as the ratio of

a. volume of water to volume of given soil

b. volume of water to volume of voids in soil

c. weight of water to weight of air in voids

d. weight of water to weight of solids of given mass of soil.

20. Pick up the incorrect statement from the following:

a. Below the water table, the pore water may be static.

b. The hydrostatic pressure depends on the depth below the water level.

c. The attractive forces between the particles, caused due to negative pressure of water held above the water table is called soil suction.

d. None of these.

21. A reinforced concrete cantilever beam is 3.6 m long, 25 cm wide and has its lever arm 40 cm. It carries a load of 1200 kg at its free end and vertical stirrups can carry 1800 kg. Assuming concrete to carry one-third of the diagonal tension and ignoring the weight of the beam, the number of shear stirrups required, is

a. 35

b. 40

c. 45

d. 50

22. If P kg/m2 is the upward pressure on the slab of a plain concrete footing whose projection on either side of the wall is a cm, the depth of foundation D is given by

a. D = 0.00775 aP

b. D = 0.0775 aP

c. D = 0.07775 aP

d. D = 0.775 Pa

23. By over-reinforcing a beam, the moment of resistance can be increased not more than

a. 10%

b. 15%

c. 20%

d. 25%

24. A column is regarded as long column if the ratio of its effective length and lateral dimension, exceeds

a. 15

b. 20

c. 25

d. 30

25. Which one of the following piles has a cast iron shoe even after removal of the hollow cylindrical steel casing

a. pedastal pile

b. Franki pile

c. vibro pile

d. both (a) and (d) of the above.

26. A covering of concrete placed on the exposed top of an external wall, is known as

a. cornice

b. coping

c. frieze

d. lintal.

27. Ordinary Portland cement is manufactured from

a. lime stone and clay

b. gypsum and lime

c. pozzolana

d. lime, pozzolana and clay.

28. Setting time of cement increases by adding

a. gypsum

b. hydrogen peroxide

c. calcium chloride

d. sodium oxide

29. The R.L. of ground water table on the sides of a valley is 1505 m whereas R.L. of the stream water is 1475 m. If 60° slope consists of pervious soil between R.L. 1485 m to 1500 m, the gravity spring may be expected at the point of reduced level

a. 1500 m

b. 1505 m

c. 1475 m

d. 1485 m.

30. Pick up the correct statement from the following :

a. At zero discharge, discharge valve remaining closed, the head developed is maximum which is known as shut-off head

b. At a particular speed the discharge at which efficiency of a pump is maximum, is known as normal discharge

c. The brake horse power required from the primemover to drive the pump, increases with the increase of discharge

d. all the above.

31. As compared to fresh river water, sea water contains oxygen

a. 10% less

b. 20% less

c. 10% more

d. 20% more.

32. In a residential colony, sewers of diameters 100 mm, 150 mm and 225 mm were laid with a gradient 1 in 120. Which portion of the sewage system does not choke in due course of time

a. 100 mm dia.

b. 150 mm dia.

c. 225 mm dia.

d. all of these.

33. Which one of the following statements is correct ?

a. The canal system and the drainage system are complementary

b. The canal system runs on ridges, gets divided into smaller channels and spreads its water on the land

c. The drainage system runs along the lowest line, collects drainage water from the slopes and joins the main stream

d. All the above.

34. For the stability of a structure against seepage pressure according to Khosla's creep theory, the critical gradient is

a. 0.25

b. 0.50

c. 0.75

d. 1.00

35. Finally formed berms in canals are provided for

a. protection of banks erosion by the waves

b. control of seepage losses

c. strengthening of banks

d. all the above.

36. In rigid module, the discharge

a. is independent of water levels in the distributary and water course

b. depends upon the water level in distributary

c. depends upon the water level in the water course

d. depends upon the water levels of both, i.e. distributary and water course

37. According to IRC : 52-1973, for a single lane National Highway in hilly region,

a. width of the carriageway must be 3.75 m

b. shoulders on either side must be 1.25 m

c. total width of the road-way must be 6.25 m

d. total of the above.

38. On the recommendations of Indian Road Congress, the ruling gradient in plains, is

a. 1 in 20

b. 1 in 30

c. 1 in 45

d. 1 in 100

39. Width of the shoulders of carriage way is generally kept

a. 125 cm

b. 150 cm

c. 200 cm

d. 250 cm

40. If L metres is the distance between extreme axles of a vehicle, its gross load should not exceed

a. 1526 (L + 5.3) - 14.7 L2

b. 1525 (L + 6.3) - 14.7 L2

c. 1525 (L + 7.3) - 14.7 L2

d. 1526 (L + 8.3) - 14.7 L2

41. The main factor to be considered while preparing a detailed estimate, is

a. Availability of materials

b. Transportation of materials

c. Location of site and local labour charges

d. All the above.

42. A portion of an embankment having a uniform up-gradient 1 in 500 is circular with radius 1000 m of the centre line. It subtends 180° at the centre. If the height of the bank is 1 m at the lower end, and side slopes 2:1, the earth work involved.

a. 26, 000 m3

b. 26, 500 m3

c. 27, 000 m3

d. 27, 500 m3

43. The rate of payment is made for 100 cu m (per % cu m) in case of

a. Rock cutting

b. Excavation in trenches for foundation

c. Earth work in filling the plinth

d. all the above.

44. The excavation exceeding 1.5 m in width and 10 sq.m in plan area with a depth not exceeding 30 cm, is termed as

a. Excavation

b. Surface dressing

c. Cutting

d. Surface excavation.

45. A dummy activity

a. is artificially introduced

b. is represented by a dotted line

c. does not consume time

d. all the above.

46. A goiden rule for the procurement of construction stones, suggests

a. 67% at the site and 33% under procurement

b. 50% at the site and 50% under procurement

c. 33% at the site and 67% under procurement

d. none of these.

47. In CPM analysis,

a. uncertainties are not allowed

b. activities are represented by arrows

c. beginning and end of an activity, are denoted by nodes

d. all the above.

48. While filling the tender for any work, the contractor considers

a. availability of construction materials

b. availability of labour

c. study of specifications

d. all the above.

49. The length of runway is increased per 300 m rise above M.S.L.

a. 4%

b. 5%

c. 6%

d. 7%

50. Total correction for elevation, temperature and gradient for a runway should not be more than

a. 20%

b. 25%

c. 30%

d. 35%