***Sub Engineer***

1. One of the Lehmann's rules of plane tabling, is

a. location of the instrument station is always distant from each of the three rays from the known points in proportion to their distances

b. when looking in the direction of each of the given points, the instrument station will be on the right side of one and left side of the other ray

c. when the instrument station is outside the circumscribing circle its location is always on the opposite side of the ray to the most distant point as the inter-section of the other two rays

d. none of these.

2. The ratio of the angles subtended at the eye, by the virtual image and the object, is known as telescope's

a. resolving power

b. brightness

c. field of view

d. magnification.

3. The difference in the lengths of an arc and its subtended chord on the earth surface for a distance of 18.2 km, is only

a. 1 cm

b. 5 cm

c. 10 cm

d. 100 cm.

4. It θ is the probable error of an observed bearing of a line of length l, the error over the whole length of the traverse of n lines of length l is

a. l n

b.  n

c. θ

d.  θ n

5. The rocks having alumina or clay as their major constituents, are known as

a. argillaceous rocks

b. calcareous rocks

c. sedimentary rocks

d. igneous rocks.

6. Invar contains

a. 12% of nickel

b. 24% of nickel

c. 30% to nickel

d. 36% of nickel.

7. Soundness test of cement determines

a. quality of free lime

b. ultimate strength

c. durability

d. initial setting

8. The plastics prepared from Vinyl resin are

a. non-toxic

b. transparent

c. colourless

d. all of these.

9. If a concrete column 200 x 200 mm in cross-section is reinforced with four steel bars of 1200 mm2 total cross-sectional area. Calculate the safe load for the column if permissible stress in concrete is 5 N/mm2 and Es is 15 Ec

a. 274 MN

b. 284 MN

c. 294 MN

d. None of these.

10. The horizontal thrust on the ends of a two hinged semicircular arch of radius R carrying

a. a uniformly distributed load ω per unit run over its right half span, is

b. a uniformly distributed load ω per unit run over its entire span is

c. a distributed load varying from zero at the left end to ω per unit horizontal run at the right end, is

d. all the above.

11. A beam is said to be of uniform strength, if

a. deflection is same throughout the beam

b. bending stress is same throughout the beam

c. shear stress is same throughout the beam

d. none of these.

12. In a three hinged arch, the shear force is usually

a. maximum at crown

b. maximum at springings

c. maximum at quarter points

d. varies with slope.

13. An orifice is called a large orifice if water head, is

a. twice the diameter of the orifice

b. thrice the diameter of the orifice

c. four times the diameter of the orifice

d. five times the diameter of the orifice.

14. Pick up the correct statement from the following :

a. Dimensional homogeneity means the dimensions of each term in an equation on both sides are equal

b. Dimensionally homogeneous equations are independent of the system of units

c. In dimensionally homogeneous equation, the powers of fundamental dimensions on either side of the equation are identical

d. All the above.

15. If the Mach number for a fluid flow is less than 1, the flow is

a. sonic

b. supersonic

c. sub-sonic

d. none of these.

16. For a most economical rectangular channel, the width of the channel must be

a. equal to depth of flow

b. twice the depth of flow

c. half the depth of flow

d. None of these.

17. The specific gravity of Calcite is

a. 2.65

b. 2.72

c. 2.85

d. 2.90

18. A saturated soil sample has water content of 40% and specific gravity of soil particles 2.7. The void ratio of the soil, is

a. 0.4

b. 0.52

c. 1.08

d. none of these.

19. The compression index of a soil

a. decreases with an increase in the liquid limit

b. increases with an increase in the liquid limit

c. decreases with an increase in the plastic limit

d. is not related with plastic limit.

20. The angle of internal friction, is least for

a. angular -grained dense sand

b. round-grained loose sand

c. round-grained loose sand

d. clays.

21. An intermediate T-beam reinforced with two layers of tensile steel with clear cover 13 cm encasted with the floor of a hall 12 metres by 7 metres, is spaced at 3 metres from adjoining beams and if the width of the beam is 20 cm, the breadth of the flange is

a. 233 cm

b. 176 cm

c. 236 cm

d. 255 cm

22. The width of the flange of a L-beam, should be less than

a. one-sixth of the effective span

b. breadth of the rib + four times thickness of the slab

c. breadth of the rib + half clear distance between ribs

d. least of the above.

23. The ratio of the breadth to effective depth of a beam is kept

a. 0.50

b. 0.70

c. 0.75

d. none of these.

24. Side face reinforcement shall be provided in the beam when depth of the web in a beam exceeds

a. 50 cm

b. 75 cm

c. 100 cm

d. 120 cm

25. The angle between skew back of a flat arch and the horizontal, is kept approximately equal to

a. 30°

b. 60°

c. 90°

d. 120°.

26. The minimum distance between the centres of bulb of diameter du, of a multi under reamed piles, is

a. 1.25 du

b. 1.5 du

c. 1.75 du

d. 2 du.

27. The final operation of finishing floors, is known as

a. floating

b. trowelling

c. finishing

d. all the above.

28. Pick up the correct statement from the following:

a. Insufficient quantity of water makes the concrete unworkable

b. Excess quantity of water makes the concrete segregated

c. Excess quantity of water causes bleeding in concrete

d. All the above.

29. Pick up the correct statement from the following :

a. The amount of rainfall in one year, is known as annual rainfall

b. The rain cycle period in India is taken as 35 years

c. The ratio of the actual rainfall in the year to the normal rainfall at a place, is called index of the wetness

d. All the above.

30. If pH value of water is

a. 7 water it is said to be neutral

b. less than 7 it is said to be acidic

c. more than 7 it is said to be alkaline

d. all the above.

31. The gas evolved in sewers is

a. Carbondioxide

b. Hydrogen sulphide

c. Methane

d. All of these.

32. Pick up the correct statement from the following :

a. The black colour indicates septic sewage

b. The fresh sewage is practically odourless

c. The sewage omits offensive odours after four hours

d. All the above.

33. Bed bars in a canal are provided

a. to watch the general behaviour of canal

b. to measure the discharge

c. to raise the supply level

d. to control the silting

34. The width of a dowla is generally kept between 30 to 60 cm and its height above the road level should invariably be more than

a. 10 cm

b. 20 cm

c. 30 cm

d. 40 cm

35. For a unique design of a channel by Kennedy's theory

a. its breadth must only be known

b. its depth must only be known

c. its breadth and depth ratio must only be known

d. all the above.

36. Pick up the correct statement from the following

a. Approach of the water line in a flumed channel section should not be steeper than

b. Departure of the water line from a flumed channel section should not be steeper than 30°

c. Approach of the water line should not be steeper than 30° and departure line not steeper than  in a flumed channel section

d. Approach and departure of the water line in a flumed channel section, should not be steeper than

37. While designing hair pin bends on higways, the minimum

a. gradient is 1 in 40

b. gradient is 1 in 200

c. superelevations is 1 in 10

d. All the above.

38. An exceptional grade may be provided upto 1 in 12 along hill roads, if the length does not exceed

a. 60 m per km

b. 75 m per km

c. 90 m per km

d. 100 m per km

39. Traffic engineering only includes

a. design and application of control devices

b. analysis of traffic characteristics

c. traffic opeation

d. all the above.

40. The minimum ratio of the radii of two circular curves of a compound curve, is kept

a. 1.25

b. 1.5

c. 1.75

d. 2.0

41. The weight of an item is measured correct to nearest

a. 0.50 kg

b. 0.75 kg

c. 1.00 kg

d. 5 kg

42. The inspection pit or chamber is a manhole provided in a base drainage system

a. at every change of gradient

b. at every 30 m intervals

c. at the point where vertical soil pipe joins the house drain

d. All the above.

43. Pick up the incorrect statement from the following:

a. Dimensions are measured to the nearest 0.01 m

b. Areas are measured to the nearest 0.01 sq.m

c. Cubic contents are measured to the nearest 0.1 cm m

d. Weights are measured to the nearest 0.001 tonnes

44. For 100 sq. m cement concrete (1 : 2: 4) 4 cm thick floor, the quantity of cement required, is

a. 0.90 m3

b. 0.94 m3

c. 0.98 m3

d. 1.00 m3

45. The time which results in the leasi, possible construction cost of an activity, is known

a. normal time

b. slow time

c. crash time

d. standard time.

46. For the supply of materials for concrete, form work reinforcing and placing of concrete, removal of form work and curing of concrete, number of bar(s) required on bar chart, is

a. 1

b. 2

c. 3

d. 4

47. Works costing less than Rs. 20,000 are treated as

a. projects

b. major projects

c. minor projects

d. all the above.

48. The object of technical planning, is

a. preparation of estimates

b. initiating the procurement action of resources

c. taking remedial action for likely bottleneck in the execution

d. all the above.

49. The height of the pilot's eye above the runway surface is assumed

a. 1 m

b. 3 m

c. 4 m

d. 5 m

50. The lift off distance is the distance along the centre of the runway between the starting point and

a. end of the runway

b. end of stopway

c. point where air craft becomes air borne

d. point where air craft attains a height of 10.7 m