***Sub Engineer***

1. ABCD is a rectangular plot of land. If the bearing of the side AB is 75°, the bearing of DC is

a. 75°

b. 255°

c. 105°

d. 285°

2. Pantagraph is used for

a. measuring distances

b. measuring areas

c. enlarging or reducing plans

d. setting out right angles

3. A back sight

a. is always taken on a point of known elevation or can be computed

b. is added to the known level to obtain the instrument height

c. taken on an inverted staff is treated as negative

d. all the above.

4. An angles of 45° with a chain line may be set out with

a. optical square

b. open cross staff

c. Fench cross staff

d. prismatic square.

5. For melting one tonne of cast iron

a. 700 m3 air is required

b. 20 kg limestone is required

c. one quintal coke is required

d. all the above.

6. Bitumen emulsion is

a. a liquid containing bitumen in suspension

b. a paint

c. used as anti-corrosive paint

d. all the above.

7. The cement becomes unsound by the presence of excess

a. sulphur

b. magnesia

c. lime

d. All of these.

8. Plaster of Paris is obtained by calcining

a. gypsum

b. kankar

c. lime stone

d. none of these.

9. A rolled steel joist is simply supported at its ends and carries a uniformly distributed load which causes a maximum deflection of 10 mm and slope at the ends of 0.002 radian. The length of the joist will be,

a. 12 m

b. 14 m

c. 16 m

d. 18 m

10. A load of 1960 N is raised at the end of a steel wire. The minimum diameter of the wire so that stress in the wire does not exceed 100 N/mm2 is :

a. 4.5 mm

b. 5.0 mm

c. 5.5 mm

d. 6.0 mm

11. The value of Poisson's ratio always remains

a. greater than one

b. less than one

c. equal to one

d. none of these.

12. The length of a column, having a uniform circular cross-section of 7.5 cm diameter and whose ends are hinged, is 5 m. If the value of E for the material is 2100 tonnes/cm2, the permissible maximum crippling load will be

a. 12.88

b. 128.8 tonnes

c. 288.0

d. none of these.

13. Gauge pressure is

a. absolute pressure - atmospheric pressure

b. absolute pressure + atmospheric pressure

c. atmospheric pressure - absolute pressure

d. none of these.

14. The hydrostatic force acts through

a. centre of pressure

b. centre of top edge

c. centre of bottom edge

d. metacentre.

15. Cavitation is caused by

a. Low pressure

b. High pressure

c. Low velocity

d. High velocity

16. Discharge over an ogee weir remains the same as that of

a. sharp crested weir

b. triangular weir

c. cippoletti weir

d. drowned weir.

17. A flow net may be utilised for the determination of

a. seepage

b. hydrostatic pressure

c. seepage pressure

d. all the above.

18. The minimum centre to centre distance of friction piles of 1 m diameter, is

a. 2 m

b. 2 m to 3 m

c. 3 m to 4 m

d. 5 m

19. The density of soil can be increased

a. by reducing the space occupied by air

b. by elastic compression of soil grains

c. by expelling water from pores

d. All the above.

20. Pick up the correct statement from the following:

a. Failure plane carries maximum shear stress

b. Failure plane does not carry maximum shear stress

c. Failure plane carries shear stress equal to maximum shear stress

d. None of these.

21. [A + (m - 1)ASC] known as equivalent concrete area of R.C.C. is given by

a. modular ratio method

b. load factor method

c. ultimate load method

d. none of these.

22. In testing a pile by load test, pile platform is loaded with one and half times the design load and a maximum settlement is noted. The load is gradually removed and the consequent rebound is measured. For a safe pile, the net settlement (i.e. total settlement minus rebound) per tonne of test load should not exceed

a. 15 mm

b. 20 mm

c. 25 mm

d. 30 mm

23. The thickness of the flange of a Tee beam of a ribbed slab is assumed as

a. width of the rib

b. depth of the rib

c. thickness of the concrete topping 0d) half the thickness of the rib

d. twice the width of the rib.

24. The Young's modulus of elasticity of steel, is

a. 150 KN/mm2

b. 200 KN/mm2

c. 250 KN/mm2

d. 275 KN/mm2

25. A solid core of rock is formed in side the cylinder in the case of

a. auger boring

b. percussion drilling

c. diamond drilling

d. wash boring.

26. The arrangement made to support an unsafe structure temporarily, is known as

a. shoring

b. scaffolding

c. underpinning

d. jacking

27. For preparing a test-specimen, it is necessary

a. to mix coarse aggregates

b. to mix water to the cement, fine aggregates and coarse aggregates

c. to oil inner surface of the mould and the base plate, before placing concrete

d. all the above.

28. Sand generally contains salt if it is obtained from:

a. river beds

b. sea beds

c. all the above

d. none of these.

29. For calculation of economical diameter D of a pipe in metres for a discharge Q to be pumped in cumecs, Lea suggested the empirical formula

a. D = 1.22 Q

b. D = 2.22 Q

c. D = 3.22 Q

d. D = 1.33 Q.

30. Rapid gravity filters

a. make use of coarser sand with effective size as 0.5 mm

b. yield as high as 30 times the yield of slow sand filters

c. are fed with coagulation treated water

d. all the above.

31. If 2% solution of a sewage sample is incubated for 5 days at 20°C and depletion of oxygen was found to be 5 ppm, B.O.D. of the sewage is

a. 200 ppm

b. 225 ppm

c. 250 ppm

d. None of these.

32. The pH value of fresh sewage is usually

a. equal to 7

b. more than 7

c. less than 7

d. equal to zero.

33. Canals taken off from ice-fed perennial rivers, are known

a. ridge canals

b. perennial canals

c. inundation canals

d. ice canals.

34. The field capacity of a soil is 25%, its permanent wilting point is 15% and specific dry unity weight is 1.5. If the depth of root zone of a crop, is 80 cm, the storage capacity of the soil, is

a. 10 cm

b. 12 cm

c. 14 cm

d. 16 cm

35. The measure to remove water logging of land, is

a. to reduce percolation from canals and water courses

b. to increase outflow from the ground water reservoir

c. both (a) and (b)

d. neither (a) nor (b)

36. A land is said to be water-logged if its soil pores within

a. a depth of 40 cm are saturated

b. a depth of 50 cm are saturated

c. root zone of the crops are saturated

d. all the above.

37. If brakes of vehicles are effective, the vehicle-running at 30 km/hour comes to a stop in

a. 10 metres

b. 12 metres

c. 15 metres

d. 18 metres

38. If the difference in elevation of an edge of the pavement 9 m wide and its crown is 15 cm, the camber of the pavement, is

a. 1 in 45

b. 1 in 30

c. 1 in 15

d. 1 in 7.5

39. Parapet walls along hill roads, are provided

a. to retain the back filling

b. to prevent the hill from sliding

c. to prevent the wheels of the vehicle from coming on the retaining wall

d. none of these.

40. Longitudinal pavement lines marked broken in white paint

a. are for the guidance of the drivers

b. are not to be crossed over

c. may be crossed over at the discretion of the driver

d. all the above.

41. Brick walls are measured in sq. m if the thickness of the wall is

a. 10 cm

b. 15 cm

c. 20 cm

d. None of these.

42. The expected out turn of 2.5 cm cement concrete floor per manson per day

a. 2.5 sqm

b. 5.0 sqm

c. 7.5 sqm

d. 10 sqm

43. In case of laying gullies, siphons, intercepting traps, the cost includes

a. Setting and laying

b. Bed concreting

c. Connection to drains

d. All of these.

44. Pick up the excavation where measurements are made in square metres for payment.

a. surface dressing up to 15 cm depths

b. Surface excavation up to 30 cm depths

c. Both (b) and (c)

d. Both (a) and (b)

45. Pick up the PERT event from the following:

a. Digging of foundation completed

b. Laying of concrete started

c. Laying of concrete completed

d. All the above.

46. The artificial activity which indicates that an activity following it, cannot be started unless the preceding activity is complete, is known as

a. event

b. free float

c. dummy

d. constant

47. Frederick W. Taylor introduced a system of working known as

a. line organisation

b. line and staff organisation

c. functional organisation

d. effective organisation.

48. PERT analysis is based on

a. optimistic time

b. pessimistic time

c. most likely time

d. all the above.

49. For night landing, the thresholds are lighted

a. green

b. red

c. white

d. yellow

50. Pick up the correct statement from the following:

a. Approximate geometric centre of the landing area, is called air port reference point

b. The boundaries of horizontal surface and conical surface are marked with reference to air port reference point

c. The location of the air port on a map is the position of air port reference point

d. All the above.