***Sub Engineer***

1. Pick up the wrong statement from the following :

a. the diaphragm is placed between the eyepiece and the objective but nearer to the later

b. the outer component of the objective is a double-convex lens of crown glass

c. the inner component of the objective is a flint glass, convexo-concave

d. all the above.

2. Pick up the correct statement from the following :

a. with both handles in his left hand, the chain man throws out the chain with his right hand and the second chain man assists him to free it from knots

b. the follower of the chaining operation should be more experienced than the leader

c. at the end of the tenth chain length, the two chain men meet and the ten arrows are handed over to the leader

d. all the above.

3. The zero of the graduated circle of a prismatic compass is located at

a. north end

b. east end

c. south end

d. west end.

4. Transition curves are introduced at either end of a circular curve, to obtain

a. gradually decrease of curvature from zero at the tangent point to the specified quantity at the junction of the transition curve with main curve

b. gradual increase of super-elevation from zero at the tangent point to the specified amount at the junction of the transition curve with main curve

c. gradual change of gradient from zero at the tangent point to the specified amount at the junction of the transition curve with main curve

d. none of these.

5. Pick up the correct statement from the following:

a. Blistering may be cured by applying water paint finished with oil paint dried with a little copal varnish

b. Cracked paints may be cured by removing paint and giving a fresh coat of paint

c. Crawling paints may be cured by sand preparing the surface and giving a fresh coat with plenty of turps

d. All the above.

6. A good quality stone absorbs water less than

a. 5%

b. 10%

c. 15%

d. 20%

7. Durability of building stone is affected by its

a. texture

b. resistance to atmosphere

c. location in structure

d. all the above.

8. The rock generally used for roofing, is

a. granite

b. basalt

c. slate

d. pumice.

9. For a strongest rectangular beam cut from a circular log, the ratio of the width and depth, is

a. 0.404

b. 0.505

c. 0.606

d. 0.707

10. Pick up the correct statement from the following:

a. Mcg = (M M2 + r2) where letters carry their usual meanings

b. Tcp = m2 + T2 where letters carry their usual meanings

c. The torque which when acting alone would produce maximum shear stress equal to the maximum shear stress caused by the combined bending and torsion, is called equivalent torque

d. All the above.

11. As the elastic limit reaches, tensile strain

a. increases more rapidly

b. decreases more rapidly

c. increases in proportion to the stress

d. decreases in proportion to the stress.

12. The maximum deflection of

a. a fixed ended beam carrying a distributed load over the span is

b. a fixed ended beam carrying a concentrated load at the mid span is

c. a cantilever beam subjected to a moment M at the free end is

d. All the above.

13. An independent mass of a fluid does not posses

a. elevation energy

b. kinetic energy

c. pressure energy

d. none of these.

14. In a venturimeter, the divergent cone is kept

a. shorter than convergent cone

b. equal to covergent cone

c. longer than convergent cone

d. none of these.

15. The upper surface of the weir over which water flows, is known as

a. vein

b. nappe

c. sill

d. none of these.

16. If the volume of a liquid weighing 3000 kg is 4 cubic metres, 0.75 is its

a. specific weight

b. specific mass

c. specific gravity

d. none of these.

17. The compression index of a soil

a. decreases with an increase in the liquid limit

b. increases with an increase in the liquid limit

c. decreases with an increase in the plastic limit

d. is not related with plastic limit.

18. Pick up the correct statement from the following :

a. To an agriculturist, soil is the substance existing on the earth's surface, which grows and develops plants

b. To a geologist, soil is the material in a relatively thin surface zone within which roots occur, and rest of the crust is termed as rock irrespective of hardness

c. To an engineer, soil is the unaggregated and uncemented deposits of minerals and organic particles covering the earth's crust

d. All the above.

19. The slip at critical angle, is generally known

a. δ 1-line

b. rupture plane

c. slip plane

d. all the above.

20. The fluid generally used for grouting is

a. clay suspension

b. sodium silicate

c. bitumen emulsion

d. all the above.

21. In the zone of R.C.C. beam where shear stress is less than 5 kg/cm2, nominal reinforcement is provided at a pitch of

a. one-half lever arm of the section

b. one-third lever arm of the section

c. lever arm of the section

d. one and half lever arm of the section.

22. If the effective length of a 32 cm diameter R.C.C. column is 4.40 m, its slenderness ratio, is

a. 45

b. 50

c. 55

d. 60

23. A reinforced concrete cantilever beam is 3.6 m long, 25 cm wide and has its lever arm 40 cm. It carries a load of 1200 kg at its free end and vertical stirrups can carry 1800 kg. Assuming concrete to carry one-third of the diagonal tension and ignoring the weight of the beam, the number of shear stirrups required, is

a. 35

b. 40

c. 45

d. 50

24. A very comfortable type of stairs is

a. straight

b. dog legged

c. geometrical

d. open newel.

25. The type of pointing in which upper side of mortar joints is kept about 12 mm inside the face of the masonry and bottom is kept flushed with face of wall, is

a. truck pointing

b. recessed pointing

c. struck pointing

d. grooved pointing.

26. In case of foundations on sandy soil, maximum permissible differential settlement, is usually limited to

a. 25 mm

b. 35 mm

c. 45 mm

d. 55 mm.

27. If the effective working time is 7 hours and per batch time of concrete is 3 minutes, the output of a concrete mixer of 150 litre capacity, is

a. 15, 900 litres

b. 16, 900 litres

c. 17, 900 litres

d. 18, 900 litres

28. The process of hardening the concrete by keeping its surface moist is known

a. wetting

b. curing

c. compacting

d. none of these.

29. An aquiclude is

a. a non artesian aquifer

b. an artesian acquifer

c. a confined bed of impervious material between acquifers

d. a large water body under ground.

30. Chlorination of water does not remove

a. ammonia content

b. B.O.D.

c. organic matter content

d. dissolved oxygen.

31. The sewer pipe which carries sewage from a building for immediate disposal is

a. house sewer

b. lateral sewer

c. intercepting sewer

d. main sewer.

32. The dimensions of a rectangular settling tank are : length 24 m, width 6 m and depth 3 m. If 2 hour detention period for tanks is recommended, the rate of flow of sewage per hour, is

a. 208 cu m

b. 212 cu m

c. 216 cu m

d. 220 cu m.

33. Irrigation canals are generally aligned along

a. ridge line

b. contour line

c. valley line

d. straight line.

34. For the design of major hydraulic structures on the canals, the method generally preferred to, is based on

a. Bligh's theory

b. Electrical analogy method

c. The relaxation method

d. Khosla's method of independent variables.

35. A minimum of 90 cm free board is provided if the discharge in the canal is between

a. 30 to 33 cumecs

b. 30 to 60 cumecs

c. Over 60 cumecs

d. Over 100 cumecs

36. For cereal crops the most commonly adopted method of irrigation, is

a. free flowing method

b. check method

c. furrow method

d. sprinkling method.

37. The top height of a route marker above crown level is

a. 1.50 m

b. 1.75 m

c. 2.00 m

d. 2.25 m

38. A district road with a bituminous pavement has a horizontal curve of 1000 m for a design speed of 75 km ph. The super-elevation is

a. 1 in 40

b. 1 in 50

c. 1 in 60

d. 1 in 70

39. If L is the length of the transition curves provided on either side of a circular curve of radius R, the maximum angle of deflection with tangent for the junctions of the transition curve and circular curve, is

a. L/2R

b. L/3R

c. L/4R

d. none of these.

40. To prevent compressive stresses in a rigid concrete pavement, the joint prvoided, is

a. expansion joint

b. contraction joint

c. hinged joint

d. all the above.

41. The slope of the oultet of 'P trap' below the horizontal is kept

a. 8°

b. 10°

c. 12°

d. 14°

42. The height of the sink of wash basin above floor level is kept

a. 60 cm

b. 70 cm

c. 75 cm to 80 cm

d. 80 cm

43. The area is measured correct to the nearest

a. 0.01 sqm

b. 0.02 sqm

c. 0.03 sqm

d. 0.04 sqm

44. Pick up the incorrect statement from the following:

a. Lead is the average horizontal straight distance between the borrow pit and the place of spreading soil

b. The lead is calculated for each block of the excavated area

c. The unit of lead is 50 m for a distance upto 500 m

d. The unit of lead is 1 km where the lead exceeds 2 km.

45. Railway projects are treated as

a. light construction

b. heavy construction

c. industrial construction

d. none of these.

46. A dummy activity

a. is artificially introduced

b. is represented by a dotted line

c. does not consume time

d. all the above.

47. Pick up the incorrect statement from the following:

a. The tail of the arrow indicates the start of the activity

b. The head of the arrow indicates the end of the activity

c. The arrows are drawn to scale from left to right

d. Each activity consumes a given time.

48. Military organisation is known as

a. line organisation

b. line and staff organisation

c. functional organisation

d. none of these.

49. The maximum value of the angle of turning of the nose gear large jet aircrafts, is limited to

a. 20°

b. 30°

c. 45°

d. 60°

50. The runway orientation is made so that landing and take off are

a. against the wind direction

b. along the wind direction

c. perpendicular to wind direction

d. none of these.