***Sub Engineer***

1. The difference of level between a point below the plane of sight and one above, is the sum of two staff readings and an error would be produced equal to

a. the distance between the zero of gradient and the foot of the staff

b. twice the distance between the zero of graduation and the foot of the staff

c. thrice the distance between the zero of graduation and the foot of the staff

d. none of the above.

2. The Random errors tend to accumulate proportionally to

a. numbers of operations involved

b. reciprocal of operations involved

c. square root of the number of operation involved

d. cube root of the number of operation involved.

3. For indirect ranging, number of ranging rods required, is

a. 2

b. 3

c. 4

d. 5

4. While viewing through a level telescope and moving the eye slightly, a relative movement occurs between the image of the levelling staff and the cross hairs. The instrument is

a. correctly focussed

b. not correctly focussed

c. said to have parallax

d. free from parallax.

5. Wrought iron is manufactured from pig iron by

a. puddling

b. shingling

c. rolling

d. all the above.

6. Veneering means

a. carving out designs on timber planks

b. chemically treating timber planks

c. thick   layer   of superior wood glued to inferior wood

d. thin   layer   of superior   wood glued to inferior wood.

7. Fibre boards can be

a. distempered

b. painted

c. painted and distempered

d. used for furniture.

8. Pick up the polymineralic rock from the following:

a. Pure gypsum

b. Magnesite

c. Granite

d. None of these.

9. A shaft rotating N.R.M. under a torque T, transmits a power

a.  Newton metres/sec

b.  Newton metres/min

c.  Newton metres/min

d.  Newton metres/sec

10. At yield point of a test piece, the material

a. obeys Hooke's law

b. behaves in an elastic manner

c. regains its original shape on removal of the load

d. undergoes plastic deformation.

11. The ratio of the moment of inertia of a circular plate and that of a square plate for equal depth, is

a. equal to one

b. more than one

c. equal to 3π/16

d. none of these.

12. The ratio of the effective length of a column and minimum radius of gyration of its cross-sectional area, is known

a. buckling factor

b. slenderness ratio

c. crippling factor

d. none of these.

13. Total energy line is

a. pressure head

b. datum head

c. kinetic head

d. All the above.

14. Discharge over an ogee weir remains the same as that of

a. sharp crested weir

b. triangular weir

c. cippoletti weir

d. drowned weir.

15. When no air is left below the nappe and water stream adheres to the down stream face of the weir, it is known as

a. free nappe

b. depressed nappe

c. clinging nappe

d. none of these.

16. Maximum efficiency of transmission of power through a pipe, is

a. 25%

b. 33.3%

c. 50%

d. 66.67%.

17. Terzaghi's analysis assumes :

a. soil is homogeneous and isotropic

b. elastic zone has straight boundaries inclined at ψ = φ to the horizontal and plastic zones fully developed

c. failure zones do not extend above the horizontal plane through the base of the footing

d. all the above.

18. The angle of internal friction is maximum for

a. angular-grained dense sand

b. round-grained dense sand

c. round-grained loose sand

d. clays.

19. Pick up the correct statement from the following :

a. Liquid limit minus plastic limit, is known as plasticity index of the soil

b. Plastic limit minus shrinkage limit, is known as shrinkage index of the soil

c. The ratio of the plasticity index to the flow limit, is known as toughness index of the soil

d. All the above.

20. The triaxial apparatus is usually used for

a. unconsolidated-undrained test

b. consolidated-undrained test

c. drained test

d. all the above tests.

21. Design of R.C.C. cantilever beams, is based on the resultant force at

a. fixed end

b. free end

c. mid span

d. mid span and fixed support.

22. An R.C.C. roof slab is designed as a two way slab if

a. it supports live loads in both directions

b. the ratio of spans in two directions is less than 2

c. the slab is continuous over two supports

d. the slab is discontinuous at edges.

23. In a combined footing if shear stress does not exceed 5 kg/cm2, the nominal stirrups provided are

a. 8 legged

b. 10 legged

c. 12 legged

d. none of these.

24. A foundation is called shallow if its depth, is

a. half of its width

b. three-fourth of its width

c. equal to its width

d. all the above.

25. If height of the first storey of a building is 3.2 m and riser is 13 cm, the number of treads required, is

a. 18

b. 24

c. 25

d. 30.

26. An arch may fail due to

a. uneven settlement of abutments

b. sliding of voussoirs

c. crushing of the material

d. all the above.

27. The shrinkage of concrete

a. is proportional to water content is the mix

b. is proportional to cement concrete

c. increases with age of concrete

d. all the above.

28. The aggregate containing moisture in pores and having its suface dry, is known as

a. very dry aggregates

b. dry aggregates

c. saturated surface dry aggregate

d. none of these.

29. m and n are monsoon duration factor and catchment factor respectively. If P is yearly rainfall in cm, runoff can be calculated by

a. velocity area method

b. weir or spillway method

c. use of venturi-meter

d. using power plant consumption

30. Total flow in stream is known

a. run off

b. stream flow

c. discharge

d. all the above.

31. A sewer running partially full and hurried with back filled, fails in compression due to

a. weight of the pipe

b. weight of the back fill

c. superimposed traffic loads

d. all the above.

32. In a sludge tank, the gas mainly produced, is

a. Nitrogen

b. Hydrogen

c. Carbon dioxide

d. Methane.

33. If H and d are the water depth and drop in the bed level at a Sarda fall, the width B of the trapezoidal crest, is given by

a. B = 0.22 H + d

b. B = 0.33 H + d

c. B = 0.44 H + d

d. B = 0.55 H + d

34. While deciding the alignment of a proposed canal taking off from a river at B, three alignments of approximately equal lengths are available. These cross a drainages at C1, C2 and C3 where drainage bed levels C1 > C2 > C3, you will provide at site C3

a. an aqueduct

b. a syphon aqueduct

c. a super passage

d. a syphon.

35. In a syphon aqueduct

a. drainage passes over the canal and F.S.L. of the canal is below the bottom of the drainage trough

b. drainage passes over the canal and F.S.L. of the canal is above the bottom of the drainage trough

c. canal passes over the drainage and H.F.L. of the drainage is above the bottom of the canal trough

d. canal passes over the drainage and H.F.L. of the drainage is below the bottom of the canal trough.

36. If the straight sides of a triangular section of a lined canal with circular bottom of radius D, make an angle θ with horizontal, the hydraulic mean depth is

a. D/2

b. D/3

c. D/4

d. D/5

37. The width of road pavements, depends upon

a. width of traffic lane

b. number of traffic

c. width of median strip

d. all the above.

38. Concrete pavement is provided if daily traffic per lane exceeds

a. 750 tonnes

b. 1000 tonnes

c. 1250 tonnes

d. 1300 tonnes

39. In retaining and breast walls, weep holes are provided at

a. 50 cm vertical height and 50 cm centre to centre horizontally

b. 100 cm vertical height and 100 cm centre to centre horizontally

c. 100 cm vertical height and 120 cm centre to centre horizontally

d. 120 cm vertical height and 100 cm centre to centre horizontally.

40. The absolute minimum sight distance required for stopping a vehicle moving with a speed of 80 km ph, is

a. 120 m

b. 200 m

c. 640 m

d. none of these.

41. The area is measured correct to the nearest

a. 0.01 sqm

b. 0.02 sqm

c. 0.03 sqm

d. 0.04 sqm

42. The assumption on which the trapezoidal formula for volumes is based, is

a. The end sections are parallel planes

b. The mid-area of a pyramid is half the average area of the ends

c. The volume of the prismoidal is over-es-timted and hence a prismoidal correction is applied

d. All the above.

43. The minimum width of a septic tank is taken

a. 70 cm

b. 75 cm

c. 80 cm

d. 90 cm

44. The floor area includes the area of the balcony up to

a. 100%

b. 75%

c. 50%

d. 25%

45. Various activities of a project, are shown on bar charts by

a. vertical lines

b. horizontal lines

c. dots

d. crosses.

46. Pick up the correct statement from the following:

a. The duration of time by which an activity can be delayed without affecting the succeeding activity, is called free float

b. The float which affects neither the processor nor the successor activities, is called independent float

c. The difference between total float and free float, is called interfering float

d. All the above.

47. CPM is

a. is built of activities oriented programme

b. is based on time estimate

c. is used for repetitive works

d. all the above.

48. Bar charts are suitable for

a. minor works

b. major works

c. large projects

d. all the Above.

49. According to the International Civil Aviation Organisation (I.C.A.O.), the runway lengths of aerodromes, have been coded by

a. Seven English alphabets

b. Last Seven English alphabets

c. First Seven English alphabets

d. First seven natural numbers.

50. For the proposed runway stated in if the aerodrome reference temperature is 17°.2, the net designed length of the runway is

a. 2716 m

b. 2816 m

c. 2916 m

d. 3016 m