***Sub Engineer***

1. The radius of a simple circular curve is 300 m and length of its specified chord is 30 m. The degree of the curve is

a. 5.73°

b. 5.37°

c. 3.57°

d. 3.75°.

2. One of the tacheometric constants is additive, the other constant, is

a. subtractive constant

b. multiplying constant

c. dividing constant

d. indicative constant.

3. Tilt of the staff in stadia tacheometry increases the intercept if it is

a. away from the telescope pointing down hill

b. towards the telescope pointing up-hill

c. away from the telescope pointing up-hill

d. none of these.

4. In a closed traverse, sum of south latitudes exceeds the sum of north latitudes and the sum of east departures exceeds the sum of west departures, then, the closing line will lie in

a. north-west quadrant

b. north east quadrant

c. south-east quadrant

d. south-west quadrant.

5. Seasoning of timber is done

a. to make it water proof

b. to paint its surface

c. to increase its temperature

d. to remove water.

6. Whitworth compressed steel is obtained when molten steel is subjected to a pressure of

a. 9 kg/mm2

b. 13 kg/mm2

c. 15 kg/mm2

d. 10 kg/mm2.

7. Pick up the correct statement from the following:

a. For mass concrete structures subjected to wetting and drying, the water ratio should be 0.55

b. For thin structures which remain continuously under water, the water-cement ratio by weight should be 0.55

c. For massive concrete structures which remain continuously under water, the water cement ratio by weight should be 0.65

d. All the above.

8. Stones used for rubble masonry must be

a. soft

b. hard

c. light

d. heavy.

9. The greatest load which a spring can carry without getting permanently distorted, is called

a. stiffness

b. prof resilience

c. proof stress

d. proof load.

10. If a solid shaft (diameter 20 cm, length 400 cm, N = 0.8 x 105 N/mm2) when subjected to a twisting moment, produces maximum shear stress of 50 N/mm2, the angle of twist in radians, is

a. 0.002

b. 0.0025

c. 0.003

d. 0.005

11. The ratio of strengths of solid to hollow shafts, both having outside diameter D and hollow having inside diameter D/2, in torsion, is

a. 1/2

b. 1/16

c. 15/16

d. 3/8

12. In a simply supported beam (l + 2a) with equal overhangs (a) and carrying a uniformly distributed load over its entire length, B.M. at the middle point of the beam will be zero if

a. l = 2a

b. l = 4a

c. l < 2a

d. l > a

13. For the most economical trapezoidal open channel,

a. half of the top width must be equal to one of the sloping sides

b. the hydraulic mean depth must be equal to half the depth of flow

c. the semicircle drawn with top width as diameter must touch the three sides of the channel

d. All of these.

14. The thickness of a sharp crested weir is kept less than

a. one-half of the height of water on the sill

b. one-fourth of the height of water on the sill

c. two-third of the height of water on the sill

d. none of these.

15. Bernoulli's equation assumes that

a. fluid is homogeneous

b. flow is steady

c. flow is along the stream line

d. all the above.

16. An error of 1% in measuring the head of water over the crest of a rectangular weir, produces an error in the discharge which is equal to

a. 1.25%

b. 1.5%

c. 1.75%

d. 2.25%

17. A grillage foundation

a. is provided for heavily loaded isolated columns

b. is treated as spread foundation

c. consists of two sets of perpendicularly placed steel beams

d. all the above.

18. A triaxial shear test is prefered to direct shear test, because

a. it can be performed under all three driange conditions with complete control

b. precise measurement of pore pressure and change in volume during test, is not posible

c. stress distribution on the failure plane, is non uniform

d. none of these.

19. The passive earth pressure of a soil, is proportional to (where φ is the angle of internal friction of the soil.)

a. tan (45° - φ)

b. tan (45° + φ)

c. tan2 (45° - φ

d. tan2 (45° + φ)

20. The bearing capacity of a soil depends upon

a. shape of the particles

b. cohesive properties of particles

c. internal frictional resistance of particles

d. all the above.

21. A ribbed slab is provided for

a. a plain ceiling

b. thermal insulation

c. acoustic insulation

d. all the above.

22. The minimum thickness of a flat slab is taken

a. L/32 for end panels without drops

b. L/36 for end panels without drops

c. L/36 for interior panels without drop

d. all the above.

23. The zone in which transverse bending is likely to occur may be obtained by drawing a line from the faces of the column making an angle θ° with horizontal where θ° is

a. 30°

b. 45°

c. 60°

d. none of these.

24. Design of R.C.C. cantilever beams, is based on the resultant force at

a. fixed end

b. free end

c. mid span

d. mid span and fixed support.

25. If height of the first storey of a building is 3.2 m and riser is 13 cm, the number of treads required, is

a. 18

b. 24

c. 25

d. 30.

26. Raft foundations are used for :

a. providing increased area of foundation over poor bearing capacity of soil

b. spanning over small soft or loose pockets

c. counter acting the hydrostatic effect

d. all the above.

27. The entrained air in concrete

a. increases workability

b. decreases workability

c. decreases resistance to weathering

d. increases strength

28. An ideal ware house, is provided

a. water proof roof

b. few windows which remain generally closed

c. 15 cm thick concrete floor laid on a dry course of soling

d. all the above.

29. The maximum permitted loss of head in a rapid sand filter, is

a. 2 m

b. 3 m

c. 4 m

d. 5 m.

30. Gravity conduits for carrying water from the source are

a. canals

b. flumes

c. acqueducts

d. all the above.

31. For drains up to 15 cumecs, the depth d and width B are related by

a. d = 0.2 B

b. d = 0.5 B

c. B = 0.2 d

d. B = 0.5 d.

32. Assertion (A) : Discharging the effluents from the oxidation ponds just up stream of lakes or reservoirs is undesirable. Reason (R) : The discharged algae get settled in the reservoirs and cause anaerobic decomposition and other water qualities.

a. Both A and R are true and R is the correct explanation of A

b. Both A and R are true but R is not a correct explanation of A

c. A is true but R is false

d. A is false but R is true.

33. In a barrage, the crest level is kept

a. low with large gates

b. high with large gates

c. high with no gates

d. low with no gates.

34. If d1 is the depth of cutting, d2 is the height of the bank from bed level r2 : 1 and r1 : 1 are the slopes in filling and cutting respectively, the horizontal distance n between the bed and bank, is

a. x = r1 d1

b. x = r2 d2

c. x = d1 / r1

d. x = d2 / r2

35. The state of the soil when plants fail to extract sufficient water for their requirements, is

a. maximum saturated point

b. permanent wilting point

c. ultimate utilisation point

d. none of these.

36. The most suitable section of a lined canal, is

a. trapezoidal section with rounded corners for large canals

b. rectangular section with rounded corners for large canals

c. both (a) and (b)

d. both (a) and (c)

37. The maximum safe speed on roads, depends on the

a. type of road surface

b. type of curves

c. sight distance

d. all the above.

38. Maximum number of vehicles that can pass a given point on a lane during one hour without creating unreasonable delay, is known as

a. traffic density of lane

b. basic capacity of lane

c. probable capacity of lane

d. practical capacity of lane.

39. To prevent movement of moisture from subgrade to road pavement at the same level as that of water-table, thickness of a cut off layer of coarse sand, is

a. 15 cm

b. 20 cm

c. 30 cm

d. 45 cm

40. Design of flexible pavement, involves

a. intensity of traffic

b. climate of the region

c. sub-grade conditions

d. all the above.

41. In the mid-section formula

a. The area of mid-sections is calculated by using mean depth

b. The volume of the earth work is calculated by multiplying the mid-section area by the distance between the two sections

c. The volume of the earth work is calculated by multiplying the mid-section area by the distance between the two original sections

d. (a), (b) and (c) of the above.

42. The expected out turn of 12 mm plastering with cement mortar is

a. 4.0 sq m

b. 6.0 sq m

c. 8.0 sq m

d. 10 sq m

43. The brick work is not measured in cu m in case of

a. One or more than one brick wall

b. Brick work in arches

c. Reinforced brick work

d. Half brick wall.

44. Pick up the incorrect statement regarding a master trap from the following :

a. It is provided a cleaning eye at the top of the trap

b. The height of fresh air inlet pipe fixed vertically with wall is 3 m

c. The mica flap valve which opens inwards only, is fitted at the top of the inlet pipe

d. The water seal is less than that of ordinary traps.

45. Frequency distribution curves

a. having a single lump, are called uninodal curves

b. if symmetrical, are called normal curves

c. if not symmetrical, are called skew curves

d. all the above.

46. Completion of an activity on CPM network diagram, is generally known

a. Event

b. Node

c. Connector

d. All the above.

47. PERT analysis is based on

a. optimistic time

b. pessimistic time

c. most likely time

d. all the above.

48. In CPM analysis,

a. uncertainties are not allowed

b. activities are represented by arrows

c. beginning and end of an activity, are denoted by nodes

d. all the above.

49. An aircraft is flying in an atmosphere of 30°C with a speed of 1260 km ph. Its speed is known as

a. subsonic

b. sonic

c. super-sonic

d. Mach.

50. For the taxiways, the following statement is true

a. The maximum longitudinal grade is 3%

b. The permissible rate of change of grade is 1%

c. The permissible transverse grade is 1.5%

d. All the above.