***Sub Engineer***

1. Number of subdivisions per metre length of a levelling staff is

a. 100

b. 200

c. 500

d. 1000

2. Pick up the correct statement from the following :

a. the angle between the true meridian and the magnetic meridian is called magnetic variation

b. one end of the magnetic needle supported at its centre of gravity tends to dip down towards the. nearer magnetic pole of the earth

c. the magnet properly pivoted is balanced by means of riding weight movable along the needle

d. all the above.

3. If whole circle bearing of a line is 120°, its reduced bearing is

a. S 20° E

b. S 60° E

c. N 120° E

d. N 60° E.

4. From any point on the surface with a given inclination

a. only one contour gradient is possible

b. two contour gradients are possible

c. indefinite contour gradients are possible

d. all the above.

5. In order of increasing percentage of silica, the correct sequence is

a. sandy clay, calcareous clay, pure clay

b. calcareous clay, pure clay, sandy clay

c. pure clay, sandy clay, calcareous clay

d. None of these.

6. Index number expressing the relative sizes of both coarse and fine aggregates, is called

a. proportioning of aggregates

b. fineness modulus

c. grading of aggregates

d. none of these.

7. Depending on the chemical composition and mechanical properties, iron may be classified as

a. cast iron

b. wrought iron

c. steel

d. all the above.

8. The curved swellings from the growth of layers or wounds left after branches are cut off in an irregular manner, are known as

a. knots

b. rindgalls

c. burls

d. none of these.

9. A three hinged arch is generally hinged at its supports and

a. at one quarter span

b. at the crown

c. any where in the rib

d. none of these.

10. If a solid shaft (diameter 20 cm, length 400 cm, N = 0.8 x 105 N/mm2) when subjected to a twisting moment, produces maximum shear stress of 50 N/mm2, the angle of twist in radians, is

a. 0.002

b. 0.0025

c. 0.003

d. 0.005

11. At either end of a plane frame, maximum number of possible bending moments, are

a. two

b. three

c. four

d. zero.

12. For keeping the stress wholly compressive the load may be applied on a circular column anywhere within a concentric circle of diameter

a. d/3

b. d/4

c. d/8

d. d/10

13. A fluid particle may possess the displacement of

a. translation

b. rotation

c. distortion

d. all the above.

14. An ideal flow of a liquid obeys

a. Continuity equation

b. Newton's law of viscosity

c. Newton's second law of motion

d. dynamic viscosity law,

15. An open container filled with water is moved vertically downward with a uniform linear acceleration. The pressure at its bottom will be

a. greater than static pressure

b. equal to static pressure

c. lesser than static pressure

d. none of these.

16. In an open tube, free surface of mercury remains

a. horizontal

b. curved upwards

c. curved downwards

d. none of these.

17. For slopes of limited extent the surface of slippage, is usually along

a. a parabolic arc

b. an elliptical arc

c. a straight line

d. a circular arc.

18. Cohesionless soil is

a. sand

b. silt

c. clay

d. clay and silt.

19. Pick up the correct statement applicable to plate load test

a. At the centre of the test pit, a hole is dug out whose size is kept equal to the size of the test plate

b. Bottom level of the hole dug at the centre of the test pit, is kept at the level of the actual formation

c. Ratio of the depth of the hole to the width of the test plate is the same as the ratio of the depth of the actual formation to the width of the formation

d. All the above.

20. A stratum of clay 2 m thick will get consolidated 80% in 10 years. For the 80% consolidation of 8 m thick stratum of the same clay, the time required is

a. 120 years

b. 140 years

c. 160 years

d. 180 years.

21. Top bars are extended to the projecting parts of the combined footing of two columns L distance apart for a distance of

a. 0.1 L from the outer edge of column

b. 0.1 L from the centre edge of column

c. half the distance of projection

d. one-fourth the distance of projection.

22. An R.C.C. roof slab is designed as a two way slab if

a. it supports live loads in both directions

b. the ratio of spans in two directions is less than 2

c. the slab is continuous over two supports

d. the slab is discontinuous at edges.

23. An R.C.C. beam not provided with shear reinforcement may develop cracks in its bottom inclined roughly to the horizontal at

a. 35°

b. 45°

c. 55°

d. 60°

24. If diameter of a reinforcement bar is d, the anchorge value of the hook is

a. 8d

b. 12d

c. 16d

d. none of these.

25. A temporary rigid structure having platforms to enable masons to work at different stages of a building, is known as

a. scaffolding

b. dead shore

c. raking shore

d. under pinning.

26. In soils possessing low bearing capacity, the type of foundation generally provided, is

a. grillage footing

b. raft footing

c. mat footing

d. all the above.

27. Pick up the incorrect statement from the following:

a. Tricalcium silicate (C3S) generates more heat of hydration

b. Tricalcium silicate (C3S) develops early strength

c. Tricalcium silicate (C3S) has more resistance to sulphate attack

d. None of these.

28. The aggregate containing moisture in pores and having its suface dry, is known as

a. very dry aggregates

b. dry aggregates

c. saturated surface dry aggregate

d. none of these.

29. According to IS : 1172-1963, a minimum of 135 litres of water capita per day, is required for

a. Boarding schools

b. Nurses home and medical quarters

c. hostels

d. all the above.

30. The head against which the motor works for lifting water, is

a. maximum depression head

b. velocity head

c. frictional losses in the delivery pipe

d. all the above.

31. Self-cleansing velocity is

a. velocity at dry weather flow

b. velocity of water at flushing

c. velocity at which no accumulation remains in the drains

d. velocity of water in a pressure filter.

32. For evaporation and measurement of settlable solids, the apparatus used, is

a. a jar

b. a breaker

c. a test tube

d. an Imhoff cone.

33. A hydraulic structure is designed to withstand

a. seepage forces

b. hydraulic jump

c. hydraulic pressure

d. all the above.

34. If average particle size of the silt in millimetres is m, the Lacey's silt factor f is proportional to

a. m3

b. m1/2

c. m1/3

d. m2/3

35. F.S.L. of a canal at its head with respect to parent channel is kept

a. at the same level

b. 15 cm lower

c. 15 cm higher

d. none of these.

36. In a canal syphon, the flow is

a. under atmospheric pressure

b. pipe flow

c. with critical velocity

d. under negative pressure.

37. Setting out of Lemniscate transition curves, is done with

a. perpendicular offsets

b. radial offsets

c. deflection angles

d. polar deflection angles.

38. To indicate proper control of consistency of a freshly mixed concrete for pavement construction, the slump should be between

a. 4 to 6 cm

b. 5 to 7 cm

c. 7 to 10 cm

d. 10 to 12 cm

39. The maximum safe speed on roads, depends on the

a. type of road surface

b. type of curves

c. sight distance

d. all the above.

40. Concrete pavement is provided if daily traffic per lane exceeds

a. 750 tonnes

b. 1000 tonnes

c. 1250 tonnes

d. 1300 tonnes

41. Pick up the item of work not included in the plinth area estimate

a. Room area

b. Verandah area

c. W.C. area

d. Courtyard area.

42. Pick up the correct statement from the following:

a. In order to check up the average depth of excavation, 'Dead mans' are left at the mid-widths of borrow pits

b. The earthwork calculation in excavation is made from the difference in levels obtained with a level

c. The earth work in excavation to form the road embankment includes the formation of correct profile and depositing the soil in layers

d. All the above.

43. The rate of an item of work depends on

a. Specifications of materials

b. Proportion of mortar

c. Method of construction

d. All the above.

44. The rate of payment is made for 100 cu m (per % cu m) in case of

a. Rock cutting

b. Excavation in trenches for foundation

c. Earth work in filling the plinth

d. all the above.

45. The main disadvantage of line organisation, is

a. rigid structure

b. extraordinary delay in communications

c. top level executions over work

d. all the above.

46. Pick up the correct statement from the following:

a. Latest occurrence time is denoted by TL

b. Contractual obligation time is denoted by Ts

c. Latest occurrence time is taken as contractual obligation time

d. All the above.

47. The first stage of a construction, is

a. survey of the site

b. initiation of proposal

c. preparation of tender

d. allotment of funds.

48. Pick up the incorrect statement from the following:

a. The tail of the arrow indicates the start of the activity

b. The head of the arrow indicates the end of the activity

c. The arrows are drawn to scale from left to right

d. Each activity consumes a given time.

49. The meterological condition which influences the size and location of an air port is

a. air density

b. reduced level

c. wind direction

d. all the above.

50. Pick up the correct statement from the following:

a. The wind data of an air port is depicted in the form of a chart known as wind rose

b. The landing and take off of the air craft is made against the wind direction

c. The product of wind loading and power loading is called air craft rating

d. All the above.