***Sub Engineer***

1. Probable accidental error in precise levelling as recommended by International Geodetic Association, should not exceed (where k is in kilometers.)

a. ± 0.5 k mm

b. ± 1 k mm

c. ± 2 k mm

d. ± 5 k mm.

2. The sensitiveness of a level tube decreases if

a. radius of curvature of its inner surface is increased

b. diameter of the tube is increased

c. length of the vapour bubble is increased

d. both viscosity and surface tension are increased.

3. The slope correction may be ignored if

a. the slope of the ground is less than 3°

b. to slope of the ground is say 1 in 19

c. both (a) and (b)

d. neither (a) nor (b)

4. The bearings of the lines AB and BC are 146° 30' and 68° 30'. The included angle ABC is

a. 102°

b. 78°

c. 45°

d. none of these.

5. Commonly used thinner in

a. cellulose paints is ethyle acetate

b. oil paints, is naptha

c. distemper, is water

d. all the above.

6. Pick up the correct statement from the following:

a. Alexander Parkes, a Scottish chemist prepared a hard material by mixing camphor and alcohol with nitro cellulose and called it, as Parkesite

b. Dr. L. Bakeland, a Belgian scientist prepared a product known as Bakelite

c. Pollark, an Austrian scientist prepared a substance from urea and formaldehyde and called it Plastic

d. All the above.

7. Acrylic sheets

a. are generally unaffected by most household detergents

b. possess the light transmission rate of 93%

c. are available in various shapes

d. all the above.

8. Pick up the correct statement from the following:

a. The lime stone resists fire upto about 800°C and at higher temperature it splits into CaO and CO2

b. The sand stone with silicates resist a fire in a better way

c. The argillaceous stone though poor in strength can resist fire quite weak

d. All the above.

9. Shear strain energy theory for the failure of a material at elastic limit, is due to

a. Guest or Trecas

b. St. Venant

c. Haig

d. Von Mises.

10. A three hinged arch is generally hinged at its supports and

a. at one quarter span

b. at the crown

c. any where in the rib

d. none of these.

11. In a three hinged arch, the bending moment will be zero

a. at right hinge only

b. at left hinge only

c. at both right and left hinges

d. at all the three hinges.

12. A solid cube is subjected to equal normal forces on all its faces. The volumetric strain will be x-times the linear strain in any of the three axes when

a. x = 1

b. x = 2

c. x = 3

d. x = 4

13. The discharge through an internal mouth piece is more if its length is

a. <  diameter

b. < diameter

c. ≥ diameter

d. none of these.

14. For the two dimensional flow, the stream function is given by ψ = 2xy. The velocity at a point (3, 4) is

a. 8 m/sec

b. 10 m/sec

c. 12 m/sec

d. 15 m/sec.

15. To ensure that water does not rise more than 100 cm above the crest, for a discharge of 5.00 m3/sec, the length of water will be

a. 2.48 m

b. 2.49 m

c. 2.50 m

d. 2.51 m.

16. Total pressure on the top of a closed cylindrical vessesl completely filled with liquid, is directly proportional to

a. (radius)2

b. (radius)3

c. (radius)4

d. none of these.

17. A compacted soil sample using 10% moisture content has a weight of 200 g and mass unit weight of 2.0 g/cm3. If the specific gravity of soil particles and water are 2.7 and 1.0, the degree of saturation of the soil is

a. 11.1%

b. 55.6%

c. 69.6%

d. none of these.

18. The inventor of the term soil mechanics, was

a. Kray

b. Dr. Karl Terzaghi

c. Leygue

d. Fellenius.

19. For determining the ultimate bearing capacity of soil, the recommended size of a square bearing plate to be used in load plate test should be 30 to 75 cm square with a minimum thickness of

a. 10 mm

b. 15 mm

c. 20 mm

d. 25 mm

20. Factor of safety against sliding of a slope, is the ratio of

a. actual cohesion to that required to maintain stability of slope

b. shear strength to shear stress along the surface

c. neither (a) nor (b)

d. both (a) and (b).

21. If the permissible compressive stress for a concrete in bending is C kg/m2, the modular ratio is

a. 2800/C

b. 2300/2C

c. 2800/3C

d. 2800/C2

22. In a combined footing for two columns carrying unequal loads, the maximum hogging bending moment occurs at

a. more loaded column

b. a point equidistant from either column

c. a point of the maximum shear force

d. a point of zero shear force.

23. If T and R are tread and rise respectively of a stair, then

a. 2R + T = 60

b. R + 2T = 60

c. 2R + T = 30

d. R + 2T= 30

24. The design of heel slab of a retaining wall, is based on the maximum bending moment due to:

a. weight of the soil above it,

b. load of the surcharge, if any

c. upward soil reaction

d. all the above.

25. Safe bearing capacity of black cotton soil varies from

a. 2 to 3 t/m2

b. 5 to 7.5 t/m2

c. 8 to 10 t/m2

d. 10 to 12 t/m2.

26. In places where the soil is soft and has small resistance to the flow of concrete, which one of the following types of piles, is used

a. vibro pile

b. pressure pile

c. Franki pile

d. pedestal pile.

27. The process of proper and accurate measurement of concrete ingredients for uniformity of proportion, is known

a. curing

b. mixing

c. batching

d. none of these.

28. The aggregate impact value of the aggregate used in

a. building concrete is less than 45

b. road pavement concrete is less than 30

c. runway concrete is less than 30

d. all the above.

29. Gravity conduits for carrying water from the source are

a. canals

b. flumes

c. acqueducts

d. all the above.

30. For centrifugal pumps

a. initial cost is low

b. limited space is required

c. the discharge obtained is steady and non-pulsating

d. all the above.

31. If a paper moistened with lead acetate for five minutes when placed in manhole turns black. The sewer certainly contains

a. Hydrogen sulphide

b. Carbon dioxide

c. Methane

d. Oxygen

32. The factor responsible for purification of sewage in river is

a. Dissolved oxygen in water

b. Temperature

c. Turbulence

d. All the above.

33. While deciding the alignment of a proposed canal taking off from a river at B, three alignments of approximately equal lengths are available. These cross a drainages at C1, C2 and C3 where drainage bed levels C1 > C2 > C3, you will provide at site C3

a. an aqueduct

b. a syphon aqueduct

c. a super passage

d. a syphon.

34. In case of a trapezoidal notch fall

a. top length of the piers should not be less than their thickness

b. splay up stream from the notch section is 45°

c. splay down stream from the notch section is

d. all the above.

35. Pick up the incorrect statement from the following:

a. In free flooding irrigation, water is admitted at one corner of a field and is allowed to spread over the entire area

b. In check method of irrigation, the field is divided into smaller compartments and water is admitted to each in turn

c. In furrow irrigation water is admitted between the rows of plants in the field

d. None of these.

36. Which one of the following statements is correct ?

a. The canal system and the drainage system are complementary

b. The canal system runs on ridges, gets divided into smaller channels and spreads its water on the land

c. The drainage system runs along the lowest line, collects drainage water from the slopes and joins the main stream

d. All the above.

37. Design of horizontal curves on highways, is based on

a. permissible friction on the road surface

b. permissible centrifugal ratio

c. permissible maximum super-elevation

d. all the above.

38. When load is applied on concrete pavement

a. away from edges, the maximum bending moment is negative

b. away from edges, the maximum bending moment cause compression

c. on the edges, the maximum stress is parallel to the edge of the slab

d. none of these.

39. For a vehicle moving with a speed of 80 km per hour, the brake reaction time, in ordinary cases, is

a. 1.5 sec

b. 2.0 sec

c. 2.5 sec

d. 3.0 sec

40. If a Lemniscate curve having an angle of deflection Δ, is transitional throughout, the maximum polar angle is

a. Δ/3

b. Δ/4

c. Δ/5

d. Δ/6

41. The expected out turn of 2.5 cm cement concrete floor per manson per day

a. 2.5 sqm

b. 5.0 sqm

c. 7.5 sqm

d. 10 sqm

42. A cement concrete road is 1000 m long, 8 m wide and 15 cm thick over the sub-base of 10 cm thick gravel. The box cutting in road crust is

a. 500 m3

b. 1000 m3

c. 1500 m3

d. 2000 m3

43. The area of a sloping surface of a protective embankment of mean height d, side slopes S : 1 and length L is

a. d x d x s

b. d2 x (ds)2

c. L.D 1 + s2

d. 2 LD 1 + s2

44. The weight of an item is measured correct to nearest

a. 0.50 kg

b. 0.75 kg

c. 1.00 kg

d. 5 kg

45. If TL is the latest allowable event occurrence time, total activity slack(s), is equal to

a. LST-EST

b. LFT-EFT

c. TL-EFT

d. all the above.

46. Various activities of a project, are shown on bar charts by

a. vertical lines

b. horizontal lines

c. dots

d. crosses.

47. Critical Path Net Work helps an engineer

a. to concentrate his attention on critical activities

b. to divert the resources from non-critical advanced activities to critical activities

c. to be cautious for avoiding any delay in the critical activities to avoid delay of the whole project

d. all the above.

48. Pick up the incorrect statement from the following:

a. The various functions under each activity, are shown by one bar on Bar Charts

b. Bar chart establishes the interdependency of one event on another

c. Only approximate percentage of the completed work is reported

d. None of these,

49. Pick up the incorrect statement from the following:

a. In single engine aeroplanes, the engine is provided in the nose of the aircraft

b. In double engine aeroplanes, one engine on either wing is placed symmetrically

c. In three engine aeroplanes, two engines are placed on both wings and one engine is placed in the tail

d. None of these.

50. The landing and take off of the air craft is made against the direction of wind. In no case the centre line of the runway should make an angle with the wind direction exceeding

a. 10°

b. 20°

c. 30°

d. 40°