***Sub Engineer***

1. Under ordinary conditions, the precision of a theodolite traverse is affected by

a. systematic angular errors

b. accidental linear errors

c. systematic linear errors

d. accidental angular errors.

2. The desired sensitivity of a bubble tube with 2 mm divisions is 30". The radius of the bubble tube should be

a. 13.75 m

b. 3.44 m

c. 1375 m

d. none of these.

3. Resolving power of a telescope depends on

a. the diameter of the aperture

b. the pupil aperture of the eye

c. the diameter of the object glass

d. all the above.

4. Removal of parallax, may be achieved by focussing

a. the objective

b. the eye-piece

c. the objective and the eye-piece

d. none of these.

5. The most commonly used base for timber painting, is

a. red lead

b. zinc white

c. white lead

d. titanium white.

6. Pick up the correct statement from the following:

a. Quick lime contains calcium oxide

b. Slaked lime may be obtained from quick lime

c. Slaked lime is obtained by adding water to quick lime

d. All the above.

7. Cement is said to be of good quality if

a. one feels cool by thrusting one's hand in the cement bag

b. it is not smooth when rubbed in between fingers

c. a handful of cement thrown into a bucket of water does not float

d. none of these.

8. Seasoning of timber is done

a. to make it water proof

b. to paint its surface

c. to increase its temperature

d. to remove water.

9. Flat spiral springs

a. are supported at outer end

b. are wound by applying a torque

c. are used in clock-work mechanism

d. all the above.

10. A close coil helical spring when subjected to a moment M having its axis along the axis of the helix

a. it is subjected to pure bending

b. its mean diameter will decrease

c. its number of coils will increase

d. all the above.

11. The type of butt joints in common use, is :

a. single inverted V-butt joint

b. double V-butt joint

c. double U-butt joint

d. single V-butt joint.

12. If a constant section beam is subjected to a uniform bending moment throughout, its length bends to

a. a circular arc

b. a parabolic arc

c. a catenary

d. none of these.

13. The acceleration f required to accelerate a rectangular tank containing water horizontally so that the slope of its free surface is 45°, is

a. g

b. 2 g

c. 2.5 g

d. 3 g.

14. The discharge through a V-notch weir varies as

a. 1/H

b. H3/2

c. H7/2

d. H5/2.

15. Total energy line is

a. pressure head

b. datum head

c. kinetic head

d. All the above.

16. Pick up the correct statement regarding convergent divergent mouth piece from the following :

a. It converges upto Venacontracta and then diverges

b. In this mouth piece there is no loss of energy due to sudden enlargement

c. The coefficient of discharge is unity

d. All the above.

17. Darcy's law is applicable to seepage if a soil is

a. homogeneous

b. isotropic

c. incompressible

d. all the above.

18. The coefficient of compressibility of soil, is the ratio of

a. stress to strain

b. strain to stress

c. stress to settlement

d. rate of loading to that of settlement.

19. Pick up the correct definition from the following:

a. The lateral pressure exerted by the soil when the retaining wall moves away from the back fill, is generally known as active earth pressure of the soil

b. The lateral pressure exerted by the soil when the retaining wail moves towards the soil, is generally known as 'Passive earth pressure of the soil'

c. The lateral pressure exerted by the soil when the retaining wall has no movement relative to the back fill, is known as 'earth pressure at rest of the soil'

d. All the above.

20. Stoke's law sates that the velocity at which a grain settles out of suspension, the other factors remaining constant, is dependent upon

a. weight of grain

b. size of grain

c. shape and size of grain

d. shape, size and weight of grain.

21. As per IS : 456, the reinforcement in a column should not be less than

a. 0.6% and not more than 6% of cross-see-tional area

b. 0.7% and not more than 7% of cross-sec-tional area

c. 0.8% and not more than 8% of cross-sectional area

d. none of these.

22. For normal cases, stiffness of a simply supported beam is satisfied if the ratio of its span to its overall depth does not exceed

a. 15

b. 20

c. 25

d. 30

23. The diameter of longitudinal bars of a column should never be less than

a. 8 mm

b. 10 mm

c. 12 mm

d. none of these.

24. If H is the overall height of a retaining wall retaining a surcharge, the width of the base slab usually provided, is

a. 0.4 H

b. 0.5 H

c. 0.6 H

d. 0.7 H

25. Which one of the following rocks is used for monumental buildings :

a. granite

b. marble

c. sand stone

d. slate.

26. A wall constructed with stones to protect slopes of cuttings in natural ground from the action of weathering agents, is called

a. retaining wall

b. breast wall

c. buttress

d. parapet wall.

27. The type of aggregates of same nominal size, which contain less voids when compacted, are

a. rounded spherical

b. irregular

c. flaky

d. none of these.

28. The shrinkage of concrete

a. is proportional to water content is the mix

b. is proportional to cement concrete

c. increases with age of concrete

d. all the above.

29. The equation  in which Q is yearly run off in cm, P is yearly total rainfall in cm, H is difference of R.L.s. of lowest and highest points and A is area of catchment in square metres, is known as

a. English formula

b. Khosla's formula

c. Justin's formula

d. Vermule's formula.

30. Normal values of overflow rate for plain sedimentation tank using coagulants, is

a. 750 to 1000 litres/hr/m2

b. 1000 to 1250 litres/hr/m2

c. 1250 to 1500 litres/hr/m2

d. 1500 litres/hr/m2.

31. Pick up the correct statement from the following :

a. In acidic sewage, the pH value is less than 7

b. In alkaline sewage, the pH value is more than 7

c. Fresh sewage is generally alkaline

d. All the above.

32. The arrangement made for passing the sewer line below an obstruction below the hydraulic gradient lines called

a. inverted syphon

b. depressed sewer

c. sag pipe

d. all of these.

33. To hold hydraulic jumps, baffle walls are provided in

a. Sarda type falls

b. English type falls

c. Montague type falls

d. Vertical type falls.

34. Pick up the incorrect statement from the following:

a. In free flooding irrigation, water is admitted at one corner of a field and is allowed to spread over the entire area

b. In check method of irrigation, the field is divided into smaller compartments and water is admitted to each in turn

c. In furrow irrigation water is admitted between the rows of plants in the field

d. None of these.

35. If water table is comparatively high, the irrigation canal becomes useless, due to

a. water logging of the cultivated areas

b. uncertain water demand

c. spread of malaria

d. all the above.

36. The structure constructed to allow drainage water to flow under pressure through an inverted syphon below a canal, is called

a. super passage

b. aqueduct

c. super-aqueduct

d. syphon aqueduct.

37. Thickness of broken line markings on multi-lane road for lanes is generally kept

a. 10 cm

b. 12 cm

c. 15 cm

d. 18 cm

38. If the width of a pavement slab is 7.5 m, thickness 20 cm and working stress 1400 kg/cm2, spacing of 10 mm tie bars for the longitudinal joint, is

a. 20 cm

b. 30 cm

c. 40 cm

d. 50 cm

39. The top height of a route marker above crown level is

a. 1.50 m

b. 1.75 m

c. 2.00 m

d. 2.25 m

40. An ideal vertical curve is

a. cubic spiral

b. cubic parabala

c. lamniscate

d. none of these.

41. In case of laying gullies, siphons, intercepting traps, the cost includes

a. Setting and laying

b. Bed concreting

c. Connection to drains

d. All of these.

42. Pick up the correct statement from the following:

a. Cement is paid per 50 kg bag

b. Lime is paid per quintal

c. Brick aggregates is paid per cum

d. All the above.

43. The most reliable estiamte is

a. Detailed estimate

b. Preliminary estimate

c. Plinth area estimate

d. Cube rate estimate

44. Pick up the incorrect statement from the following:

a. Lead is the average horizontal straight distance between the borrow pit and the place of spreading soil

b. The lead is calculated for each block of the excavated area

c. The unit of lead is 50 m for a distance upto 500 m

d. The unit of lead is 1 km where the lead exceeds 2 km.

45. Final technical authority of a project lies with

a. Assistant Engineer

b. Executive Engineer

c. Superintending Engineer

d. Chief Engineer.

46. An event is indicated on the network by a number enclosed in

a. a square

b. a triangle

c. an ellipse

d. all the above.

47. The first stage of a construction, is

a. survey of the site

b. initiation of proposal

c. preparation of tender

d. allotment of funds.

48. Pick up the incorrect statement from the following:

a. The beginning and end of a job, are called events

b. The activity which consumes maximum time, is called a node

c. Logically and sequentially connected activities and events form a network

d. None of these.

49. According to I.C.A.O. the recommended length of air ports is decided on

a. sea level elevation

b. standard sea level temperature (15°C)

c. effective gradient percentage

d. all the above.

50. Conical surface of the approach area rises outwards

a. 1 in 10

b. 1 in 15

c. 1 in 20

d. 1 in 25