***Sub Engineer***

1. Accurate measurement of deflection angles with a transit not properly adjusted may be made by

a. setting the vernier A at zero at back station and then plunging the telescope

b. setting the vernier A at zero at back station and turning the instrument to the forward station

c. taking two back sights one with the telescope normal and the other with telescope inverted

d. none of these.

2. An imaginary line lying throughout on the surface of the earth and preserving a constant inclination to the horizontal, is called

a. contour line

b. contour gradient

c. level line

d. line of gentle scope.

3. Removal of parallax, may be achieved by focussing

a. the objective

b. the eye-piece

c. the objective and the eye-piece

d. none of these.

4. Cross-staff is used for

a. setting out right angles

b. measuring contour gradient

c. taking levels

d. measuring distances

5. The steel used for the manufacture of rails, is

a. Bessemer steel

b. mild steel

c. cast steel

d. stainless steel.

6. Laterite is a/an

a. volcanic rock

b. argillaceous rock

c. calcareous rock

d. silicious rock.

7. Bitumen felt is used for

a. water proofing

b. damp proofing

c. both (a) and (b)

d. neither (a) nor (b).

8. The usual percentages of clay and metal in cermet are :

a. 50%, 50%

b. 60%, 40%

c. 70%, 30%

d. 80%, 20%.

9. A simply supported beam which carries a uniformly distributed load has two equal overhangs. To have maximum B.M. produced in the beam least possible, the ratio of the length of the overhang to the total length of the beam, is

a. 0.207

b. 0.307

c. 0.407

d. 0.508

10. Pick up the correct statement from the following:

a. For I sections, the shear centre coincides with the centroid of the cross section of the beam

b. For channels, the shear centre does not concide its centroid

c. Bending loads should pass through the shear centre to avoid twisting

d. All the above.

11. The bending moment is maximum on a section where shearing force

a. is maximum

b. is minimum

c. is equal

d. changes sign.

12. In a shaft shear stress intensity at a point is not

a. directly proportional to the distance from the axis

b. inversely proportional to the distance from the axis

c. inversely proportional to the polar moment of inertia

d. directly proportional to the applied torque.

13. The main assumption of Bernoulli's equation is :

a. The velocity of energy of liquid particle, across any cross-section of a pipe is uniform

b. No external force except the gravity acts on the liquid

c. There is no loss of energy of the liquid while flowing

d. All the above.

14. Discharge with velocity of approach, over a rectangular weir is

a.  Cd x L x 2g [(H + ha)3/2 + ha3/2]

b.  Cd x L x 2g [(H + ha)3/2 - ha3/2]

c.  Cd x L x 2g [(H - ha)3/2 + ha3/2]

d.  Cd x L x 2g [(H - ha)3/2 - ha3/2]

15. The ratio of the percentagge error in the discharge and percentage error in the measurement of head over a triangular notch, is

a. 2/3

b. 3/2

c. 2/5

d. 5/2

16. The flow in a channel is said to be non-uniform, if

a. free water surface of an open channel is not parallel to the bed of channel

b. head needed to overcome frictional reistance is less than the drop in elevation of channel bed

c. head needed to overcome frictional resistance is more than the drop in elevation of channel bed

d. all the above.

17. Back fill with a sloping surface exerts a total active pressure Pa on the wall of height H and acts at

a. H/4 above the base parallel to base

b. H/2 above the base parallel to base

c. H/3 above the base parallel to base

d. H/5 above the base parallel to base.

18. The ratio of the undrained strength in the undrained state to the undrained strength, at the same water content, in the remoulded state, is called the sensivity of the clay. Its value for quick clays is

a. 8

b. 12

c. 16

d. 20

19. A moist soil sample weighing 108 g has a volume of 60 cc. If water content is 25% and value of G = 2.52, the void ratio is

a. 0.55

b. 0.65

c. 0.75

d. 0.80

20. The specific gravity of quartz, is

a. 2.65

b. 2.72

c. 2.85

d. 2.90

21. If the loading on a prestressed rectangular beam, is uniformly distributed, the tendon to be provided should be .

a. parabolic with convexity downward

b. parabolic with convexity upward

c. straight above centroidal axis

d. none of these.

22. The diameter of longitudinal bars of a column should never be less than

a. 8 mm

b. 10 mm

c. 12 mm

d. none of these.

23. The breadth of a ribbed slab containing two bars must be between

a. 8 cm to 10 cm

b. 10 cm to 12 cm

c. 12 cm to 15 cm

d. none of these.

24. Distribution reinforcement in a simply supported slab, is provided to distribute

a. load

b. temperature stress

c. shrinkage stress

d. all the above.

25. The method of moving each brick through a small horizontal distance before it is finally laid in any brick wall and pressing it by means of brick hammer, is known as

a. trowelling

b. laying

c. grouting

d. placing.

26. Dado is usually provided in

a. bath rooms

b. living rooms

c. verandah

d. roofs.

27. The aggregate containing moisture in pores and having its suface dry, is known as

a. very dry aggregates

b. dry aggregates

c. saturated surface dry aggregate

d. none of these.

28. If aggregates completely pass through a sieve of size 75 mm and are retained on a sieve of size 60 mm, the particular aggregate will be flaky if its minimum dimension is less than

a. 30.5 mm

b. 40.5 mm

c. 50.5 mm

d. none of these.

29. The prescribed hardness limit of potable water ranges between

a. 75 to 115 P.P.M.

b. 100 to 150 P.P.M.

c. 150 to 200 P.P.M.

d. none of these.

30. The bed slope in slow sand filters, is generally kept

a. 1 in 50

b. 1 in 75

c. 1 in 100

d. 1 : 200

31. House connections to the laterals is generally made by

a. R.C.C.

b. P.C.C.

c. Cast iron

d. Glazed stonewares.

32. For the COD test of sewage, organic matter is oxidised by K2Cr207 in the presence of

a. H2SO4

b. HNO3

c. HCl

d. none of these.

33. Pick up the correct statement from the following:

a. Duty of canal water is usually calculated at the head discharge of a channel

b. Duty of canal water at the head of a channel and at the outlet of a distributary, differ considerably due to progressive losses of water

c. Duty of canal water at the outlet of a distributary, is generally known as outlet factor or outlet discharge factor

d. All the above.

34. The crest level of a canal diversion head work, depends upon

a. discharge perimeters

b. pond level

c. all the above

d. none these.

35. The main cause of silting up a channel,

a. inadequate slope

b. defective head regulator

c. defective outlets

d. all the above.

36. The state of the soil when plants fail to extract sufficient water for their requirements, is

a. maximum saturated point

b. permanent wilting point

c. ultimate utilisation point

d. none of these.

37. If the designed speed on a circular curve of radius 1400 m is 80 km/hour, no superelevation is provided, if the camber, is

a. 4%

b. 3%

c. 2%

d. 1.7%

38. Side drains on both sides of a hill road, are essential when the road is

a. along the spur curves

b. along the re-entrant curves

c. in cutting

d. none of these.

39. The statement, regarding the size of stones used at the wearing surface. "The size of a stone used on a road must be in due proportion to the space occupied by a wheel of ordinary dimensions on a smooth level surface. The point of contact will be found to be longitudinal about 2.5 cm and every piece of stone put into the road which exceeds 2.5 cm in any of its dimension is mischievious" was made by

a. Telford

b. Macadam

c. Tresguet

d. Sully.

40. If the radius of curvature of a hill road is 50 m, the percentage grade compensation should be

a. 70/R

b. 75/R

c. 80/R

d. 100/R

41. The rate of payment is made for 100 cu m (per % cu m) in case of

a. Rock cutting

b. Excavation in trenches for foundation

c. Earth work in filling the plinth

d. all the above.

42. The order of booking dimensions is

a. Length, breadth, height

b. Breadth, length, height

c. Height, breadth, length

d. None of these.

43. The expected out turn of 2.5 cm cement concrete floor per manson per day

a. 2.5 sqm

b. 5.0 sqm

c. 7.5 sqm

d. 10 sqm

44. For the construction of buildings, the subheads of the estimate are

a. Brickwork, Stone work, Roofing

b. Brickwork Flooring, Wood work, Steel work

c. Plastering or pointing, finishing, water supply and sanitary work

d. All the above.

45. The estimated time required to perform an activity, is known as

a. event

b. dummy

c. duration

d. float.

46. Time and progress chart of a construction, is also known as

a. Gantt chart

b. Modified Mile stone chart

c. Critical path method chart

d. All the above.

47. An Executive Engineer may have powers upto

a. Rs. 25,000

b. Rs. 50,000

c. Rs. 100,000

d. Rs. 200,000

48. Site order book is used for recording

a. instructions by the executive engineers

b. construction measurements

c. issue of store equipments

d. names of the casual labour.

49. Pick up the component not applicable to aeroplanes

a. engines

b. air screw

c. fuse lage

d. none of these.

50. International Civil Aviation Organisation (I.C.A.O.) was set up at Montreal (Canada), in

a. 1929

b. 1939

c. 1947

d. 1950