***Sub Engineer***

1. In levelling operation,

a. The first sight on any change point is a back sight

b. The second sight on any change point is a fore sight

c. The line commences with a fore sight and closes with a back sight

d. The line commences with a back sight and closes with a foresight.

2. In tacheometrical observations, vertical staff holding is generally preferred to normal staffing, due to

a. ease of reduction of observations

b. facility of holding

c. minimum effect of careless holding on the result

d. none of these.

3. In chain surveying tie lines are primarily provided

a. to check the accuracy of the survey

b. to take offsets for detail survey

c. to avoid long offsets from chain lines

d. to increase the number of chain lines.

4. The difference of level between a point below the plane of sight and one above, is the sum of two staff readings and an error would be produced equal to

a. the distance between the zero of gradient and the foot of the staff

b. twice the distance between the zero of graduation and the foot of the staff

c. thrice the distance between the zero of graduation and the foot of the staff

d. none of the above.

5. Minimum of 40% of iron, is available in

a. Red haemetite

b. Limonite

c. Siderite

d. Black band.

6. Pick up the correct statement from the following:

a. In stone arches, the stones are placed with their natural beds radial

b. In cornices, the stones are placed with their natural beds as vertical

c. In stone walls, the stones are placed with their natural beds as horizontal

d. All the above.

7. Pick up the correct statement from the following:

a. The silica in excess makes the cement stronger but its setting time also increases

b. The excess amount of alumina weakens the cement

c. The addition of gypsum increases the initial setting time of cement

d. All the above.

8. Veneering means

a. carving out designs on timber planks

b. chemically treating timber planks

c. thick   layer   of superior wood glued to inferior wood

d. thin   layer   of superior   wood glued to inferior wood.

9.  is the equation of Euler's crippling load, if

a. both the ends are fixed

b. both the ends are hinged

c. one end is fixed and other end is free

d. one end is fixed and other end is hinged.

10. In plastic analysis, the shape factor for a circular section, is

a. 1.5

b. 1.6

c. 1.7

d. 1.75

11. When equal and opposite forces applied to a body, tend to elongate it, the stress so produced, is called

a. shear stress

b. compressive stress

c. tensile stress

d. transverse stress.

12. Stress in members of statically determinate simple frames, can be determined by

a. method of joints

b. method of sections

c. graphical solution

d. all the above.

13. For the two dimensional flow, the stream function is given by ψ = 2xy. The velocity at a point (3, 4) is

a. 8 m/sec

b. 10 m/sec

c. 12 m/sec

d. 15 m/sec.

14. Cavitation is caused by

a. Low pressure

b. High pressure

c. Low velocity

d. High velocity

15. Orifice-meter is used to measure

a. pressure at the point

b. discharge

c. average speed

d. velocity.

16. An error of 1% in measuring the head of water over the crest of a triangular notch, produces an error in the discharge which is equal to

a. 1.25%

b. 1.5%

c. 2.0%.

d. 2.5%

17. The minimum water content at which the soil retains its liquid state and also possesses a small shearing strength against flowing, is known

a. liquid limit

b. plastic limit

c. shrinkage limit

d. permeability limit.

18. The water content of soil is defined as the ratio of

a. volume of water to volume of given soil

b. volume of water to volume of voids in soil

c. weight of water to weight of air in voids

d. weight of water to weight of solids of given mass of soil.

19. The Terzaghi's general bearing capacity equation for a continuous footing is given by (where Nc, 2V& and Ny are bearing capacity factors.)

a. qf = cNc + γDNq + 0.5γBNγ

b. qf = cNc - γDNq + 0.5γBNγ

c. qf = cNc + γDNq - 0.5γBNγ

d. qf = cNc - γDNq - 0.5γBNγ

20. If the failure of a finite slope occurs through the toe, it is known as

a. slope failure

b. face failure

c. base failure

d. toe failure.

21. The live load to be considered for an accessible roof, is

a. Nil

b. 75 kg/m3

c. 150 kg/m2

d. 200 kg/cm2

22. Total pressure on the vertical face of a retaining wall of height h acts parallel to free surface and from the base at a distance of

a. h/4

b. h/3

c. h/2

d. 2h/3

23. An intermediate T-beam reinforced with two layers of tensile steel with clear cover 13 cm encasted with the floor of a hall 12 metres by 7 metres, is spaced at 3 metres from adjoining beams and if the width of the beam is 20 cm, the breadth of the flange is

a. 233 cm

b. 176 cm

c. 236 cm

d. 255 cm

24. In favourable circumstances a 15 cm concrete cube after 28 days, attains a maximum crushing strength

a. 200 kg/cm2

b. 300 kg/cm2

c. 400 kg/cm2

d. 500 kg/cm2

25. The wedge shaped bricks forming an arch ring, are called

a. Soffits

b. voussoirs

c. haunchs

d. spandrils.

26. The method of moving each brick through a small horizontal distance before it is finally laid in any brick wall and pressing it by means of brick hammer, is known as

a. trowelling

b. laying

c. grouting

d. placing.

27. While designing an air entrained concrete

a. proportion of aggregates is reduced

b. an allowance for the entrained air is made

c. strength of the concrete, is reduced

d. all the above.

28. Pick up the correct statement from the following:

a. Continuous grading is not necessary for obtaining a minimum of air voids

b. The omission of a certain size of aggregate is shown by a straight horizontal line on the grading curve

c. The omission of a certain size of aggregate in concrete increases the workability but also increases the liability to segregation

d. All the above.

29. Estimates of a water supply project depends upon the rate of water supply per capita consumption and probable population estimated at the end of the design period of

a. 10 to 15 years

b. 15 to 20 years

c. 20 to 30 years

d. 25 years.

30. The maximum non-verticality of the bore of a well 200 m deep, may be permitted up to

a. 50 cm

b. 75 cm

c. 100 cm

d. 150 cm.

31. Maximum permissible velocity 1.5 m/sec, is adopted in drains

a. with beds of rocks and gravels

b. lined with stones

c. both (a) and (b)

d. neither (a) nor (b).

32. Hydraulic mean radius is

a. mean radius of sewer

b. difference in heads between two points in circular pipes

c. mean of radii in a pipe line of varying cross-sections

d. cross-sectional area/wetted perimeter.

33. To control the silt entry into a distributary at head regulator, King's vanes are provided which are the walls

a. of height l/3rd to l/4th the depth of water in the main canal

b. spaced at interval of  times their heights

c. extending about one metre beyond a line drawn at an inclination of 2 : 1 to the axis of main canal

d. all the above.

34. Canals constructed for draining off water from water logged areas, are known

a. drains

b. inundation canals

c. valley canals

d. contour canals

35. Finally formed berms in canals are provided for

a. protection of banks erosion by the waves

b. control of seepage losses

c. strengthening of banks

d. all the above.

36. In a barrage, the crest level is kept

a. low with large gates

b. high with large gates

c. high with no gates

d. low with no gates.

37. The statement, regarding the size of stones used at the wearing surface. "The size of a stone used on a road must be in due proportion to the space occupied by a wheel of ordinary dimensions on a smooth level surface. The point of contact will be found to be longitudinal about 2.5 cm and every piece of stone put into the road which exceeds 2.5 cm in any of its dimension is mischievious" was made by

a. Telford

b. Macadam

c. Tresguet

d. Sully.

38. The desirable camber for straight roads with thin bituminous surfacing, is

a. 1 in 40 to 1 in 33

b. 1 in 150 to 1 in 140

c. 1 in 160 to 1 in 140

d. none of these.

39. In case of a multi-lane road, overtaking is generally permitted

a. from right

b. from left

c. from both sides right and left

d. not at all.

40. The width formation of a road means the width of

a. carriageway

b. pavement and shoulders

c. embankment at ground level

d. embankment at the top level.

41. Pick up the correct statement from the following:

a. Area of canal in cutting = BD + Sd2 where B = bed width, d = depth of cutting and S is the side slope

b. Area of the bank of canal = B1d1 + Sd2 where B1, d1 and S are the width of bank, height of the bank above N.S.L. and side slope respectively

c. If F.S.L. is above N.S.L the canal is called partly in cutting and partly in filling and berms are designed as 2d where d is full supply depth

d. All the above.

42. Berms are provided in canals if these are

a. fully in excavation

b. partly in excavtion and partly in embankment

c. fully in embankment

d. All the above.

43. If B is the width of formation, d is the height of the embankment, side slope S : 1, for a highway with no transverse slope, the area of cross-section is

a. B + d + Sd

b. Bd + Sd2

c. B x d - Sd1/2

d. (Bd + Sd2)

44. Pick up the correct statement from the following:

a. Plastering is measured in sq.m

b. Glazing is measured in sq.m

c. Striking is measured in sq.m

d. All the above.

45. The first method invented for planning projects, was

a. Bar chart method

b. Milestone chart

c. Critical path method (CPM)

d. Programme Evaluation and Review Technique (PERT)

46. The difference between the time avail-to do a job and the time required to do the job, is known as

a. event

b. float

c. duration

d. constraint.

47. For completion of a project, the critical path of the network represents

a. minimum time

b. maximum time

c. maximum cost

d. minimum cost.

48. Optimistic time, most likely time and pessimistic times for the activities of a network in the given figure are written above their arrows. If the contractual obligation time for the project is 75, the latest occurrence time for the event 2, is

a. 20

b. 25

c. 35

d. 15

49. A gradient of + 0.08% is followed by a gradient of - 0.07%. If the permissible rate of change of grade is 0.003 per 30 metres, the length of the transition curve, is

a. 150 m

b. 140 m

c. 160 m

d. 175 m

50. Pick up the correct statement from the following:

a. Localizer indicates to the pilot his position with respect to the proposed alignment

b. The glide slope indicates the correct angle of descent

c. L.O.M. and L.M.M. help the pilot to judge his position for the runway

d. All the above.