***Sub Engineer***

1. The staff reading at a distance of 80 m from a level with the bubble at its centre is 1.31 m. When the bubble is moved by 5 divisions out of the centre, the reading is 1.39 m. The angular value of the one division of the bubble, is

a. 41.25 secs

b. 14.52 secs

c. 25.05

d. none of these

2. Rankine's deflection angle in minutes is obtained by multiplying the length of the chord by

a. degree of the curve

b. square of the degree of the curve

c. inverse of the degree of the curve

d. none of these.

3. One of the Lehmann's rules of plane tabling, is

a. location of the instrument station is always distant from each of the three rays from the known points in proportion to their distances

b. when looking in the direction of each of the given points, the instrument station will be on the right side of one and left side of the other ray

c. when the instrument station is outside the circumscribing circle its location is always on the opposite side of the ray to the most distant point as the inter-section of the other two rays

d. none of these.

4. ABCD is a regular parallelogram plot of land whose angle BAD is 60°. If the bearing of the line AB is 30°, the bearing of CD, is

a. 90°

b. 120°

c. 210°

d. 270°

5. Water paint is a

a. colour wash

b. whiting

c. distemper

d. all the above.

6. Percentage content of silica in window glass, is

a. 40 to 45

b. 50 to 55

c. 60 to 65

d. 70 to 75.

7. For the manufacture of stainless steel, steel is mixed with

a. chromium

b. nickel

c. tungsten

d. none of these.

8. If the iron ore contains clay as an impurity, the flux added during calcination, is

a. clay

b. lime stone

c. argillaceous iron ore

d. all the above.

9. In plastic analysis, the shape factor for a circular section, is

a. 1.5

b. 1.6

c. 1.7

d. 1.75

10. At any point of a beam, the section modulus may be obtained by dividing the moment of inertia of the section by

a. depth of the neutral axis

b. maximum tensile stress at the section

c. maximum compressive stress at the section

d. none of these.

11. The tensile force required to cause an elongation of 0.045 mm in a steel rod of 1000 mm length and 12 mm diameter, is (where E = 2 x 106 kg/cm2)

a. 166 kg

b. 102 kg

c. 204 kg

d. 74 kg

12. A cantilever beam rectangular in cross-section is subjected to an isolated load at its free end. If the width of the beam is doubled, the deflection of the free end will be changed in the ratio of

a. 1/8

b. 1/2

c. 2

d. 3

13. When two layers of a fluid separated by dy move over the other with a difference of velocity dv, causes a shearing stress , where μ is known as

a. absolute viscosity

b. dynamic viscosity

c. viscosity

d. all the above.

14. A rectangular channel 6 m wide and 3 m deep and having a bed slope as 1 in 2000 is running full. If Chezy's constant C = 54.8, pick up the correct specification of the channel from the following :

a. hydraulic mean depth = 1.5 m

b. Velocity of flow = 1.5 m/sec

c. Rate of flow = 27 m3/sec

d. All the above.

15. The discharge through a V-notch weir varies as

a. 1/H

b. H3/2

c. H7/2

d. H5/2.

16. For most economical rectangular section of a channel, the depth is kept

a. three times the hydraulic radius

b. half the width

c. hydraulic mean depth

d. none of these.

17. In active state of plastic equilibrium in a non cohesive soil with horizontal ground surface

a. major principal stress is horizontal

b. minor principal stress is vertical

c. major principal stress is vertical

d. minor and major principal stresses are equally inclined to horizontal.

18. Pick up the correct statement from the following:

a. A soil classification system is meant to provide an accepted and systematic method of describing the various types of soils eliminating personal factors

b. The first category of soil classification is based on grain size of the soil

c. The second category of soil classification is based on fine as well as coarse grains

d. All the above.

19. Tergazhi's theory of one dimensional consolidation assumes

a. water and soil particles are incompressible

b. deformation of the soil, is entirely due to change in volume

c. Darcey's law for the velocity of flow of water through soil, is perfectly valid

d. all the above.

20. A decrease in water content results in a reduction of the volume of a soil in

a. liquid state

b. plastic state

c. semi solid state

d. all of these.

21. If the average bending stress is 6 kg/cm2 for M 150 grade concrete, the length of embedment of a bar of diameter d according to I.S. 456 specifications, is

a. 38 d

b. 48 d

c. 58 d

d. 95 d

22. An intermediate T-beam reinforced with two layers of tensile steel with clear cover 13 cm encasted with the floor of a hall 12 metres by 7 metres, is spaced at 3 metres from adjoining beams and if the width of the beam is 20 cm, the breadth of the flange is

a. 233 cm

b. 176 cm

c. 236 cm

d. 255 cm

23. The load stress of a section can be reduced by

a. increasing the total perimeter of bars

b. replacing larger bars by greater number of small bars

c. replacing smaller bars by greater number of greater bars

d. none of these.

24. If the length of an intermediate span of a continuous slab is 5m, the length of the end span is kept

a. 4.5 m

b. 4.0 m

c. 3.5 m

d. 3.0 m

25. The formWork including the props can be removed from beams, only after

a. 3 day

b. 7 days

c. 14 days

d. 21 days.

26. The process of keeping concrete moist for a certain period after its finishing, is known as

a. curing of concrete

b. placing of concrete

c. compaction of concrete

d. none of these.

27. The following proportion of the ingredients of concrete mix, is not in conformation to arbitrary method of proportioning

a. 1 : 2 : 4

b. 1 : 3 : 6

c. 1 : 2 : 8

d. 1 : 4 : 10

28. Proper batching ensures

a. durability

b. workability

c. strength

d. all the above.

29. The pH value of water fit for drinking, is

a. 11

b. 9

c. 7

d. 5

30. The best process of disinfection of public water supply, is by

a. boiling

b. chlorination

c. adding lime

d. adding zone.

31. In R.C. sewer pipes, the percentage longitudinal reinforcement to the cross-sectional area of concrete is kept

a. 10.0

b. 5.0

c. 2.0

d. 0.25

32. The underground sewers are more subjected to

a. tensile force

b. compressive force

c. bending force

d. shearing force.

33. The structure constructed to allow drainage water to flow under pressure through an inverted syphon below a canal, is called

a. super passage

b. aqueduct

c. super-aqueduct

d. syphon aqueduct.

34. Retrogression of the bed level of a river downstream a weir, occurs due to

a. heavy impact of water

b. increase of the bed level

c. less percentage of silt

d. soft soil strata.

35. For a standing crop, the consumptive use of water is equal to the depth of water

a. transpired by the crop

b. evaporated by the crop

c. transpired and evaporated by the crop

d. used by the crop in transpiration, evaporation and also the quantity of water evaporated from adjacent soil.

36. The down stream expansion head of a guide bank is extended to subtend an angle at the centre, equal to

a. 30°

b. 45°

c. 60°

d. 90°

37. Pick up the correct statement from the following:

a. Seasonal cycle of traffic volume during April and November, is usually near the annual average

b. Mid-winter seasonal cycle of traffic is least

c. Mid-summer seasonal cycle of traffic is highest

d. All the above.

38. To indicate proper control of consistency of a freshly mixed concrete for pavement construction, the slump should be between

a. 4 to 6 cm

b. 5 to 7 cm

c. 7 to 10 cm

d. 10 to 12 cm

39. The minimum vertical clearance of overhanging cliffs or any other structure above the surface of a hill road, should be

a. 4 m

b. 5 m

c. 6 m

d. 4.5 m

40. Formation width of a hill road is the distance between

a. outer end of side drain to outer end of parapet wall

b. outer end of side drain to inner end of parapet wall

c. inner end of side drain to outer end of parapet wall

d. inner end of side drain to inner end of parapet wall

41. The detention period in a septic tank is assumed

a. 20 minutes

b. 25 minutes

c. 30 minutes

d. 40 minutes

42. The ground surface slopes 1 in 50 along a proposed railway embankment 150 m in length. The height of the embankment at zero chainage is 0.5 m, the width is 11 m and side slopes 2:1. If the falling gradient of the embankment is 1 in 150, the quantity of the earthwork calculated by prismoidal formula, is

a. 3250 m3

b. 3225 m3

c. 3275 m3

d. 3300 m3

43. If the formation level of a highway has a uniform gradient for a particular length, and the ground is also having a longitudinal slope, the earthwork may be calculated by

a. Mid-section formula

b. Trapezoidal formula

c. Prismoidal formula

d. All the above.

44. Size, capacity and materials need be specified for

a. Stop-cocks

b. Ferrules

c. Ball valves

d. All the above.

45. Frederick W. Taylor introduced a system of working known as

a. line organisation

b. line and staff organisation

c. functional organisation

d. effective organisation.

46. Construction team means

a. an architect

b. an owner

c. a contractor

d. all the above.

47. The time which results in the leasi, possible construction cost of an activity, is known

a. normal time

b. slow time

c. crash time

d. standard time.

48. Frequency distribution curves

a. having a single lump, are called uninodal curves

b. if symmetrical, are called normal curves

c. if not symmetrical, are called skew curves

d. all the above.

49. The maximum value of the angle of turning of the nose gear large jet aircrafts, is limited to

a. 20°

b. 30°

c. 45°

d. 60°

50. Pick up the correct statement from the following:

a. The distance between the points of intersection of the extreme tangents to the transition curve is kept greater than 7500 m x sum of grade changes at the point of intersection

b. The rate of change of grade is limited to 0.3% per 30 m length of the curve

c. According to I.C.A.O. the maximum longitudinal gradient along a runway is limited to 1.5%

d. All the above.