***Sub Engineer***

1. If whole circle bearing of a line is 120°, its reduced bearing is

a. S 20° E

b. S 60° E

c. N 120° E

d. N 60° E.

2. Cross-staff is used for

a. setting out right angles

b. measuring contour gradient

c. taking levels

d. measuring distances

3. Contours of different elevations may cross each other only in the case of

a. an over hanging cliff

b. a vertical cliff

c. a saddle

d. an inclined plane.

4. Tilt of the staff in stadia tacheometry increases the intercept if it is

a. away from the telescope pointing down hill

b. towards the telescope pointing up-hill

c. away from the telescope pointing up-hill

d. none of these.

5. Stuco paints are suitable for

a. stone masonry

b. brick walls

c. both (a) and (b)

d. neither (a) nor (b).

6. Strength of cement concrete primarily depends upon

a. quality of water

b. quantity of aggregate

c. quantity of cement

d. water-cement ratio.

7. The steel used for rails under heavy traffic and on sharp curves, is

a. Nickel steel

b. Chrome steel

c. Magnese steel

d. Vanadium steel.

8. Pick up the compound responsible for early strength of cement from the following:

a. Tetra-calcium alumino-ferrite

b. Tricalcium silicate

c. Tricalcium aluminate

d. Dicalcium silicate.

9. A body is said to be in equilibrium if

a. it moves horizontally

b. it moves vertically

c. it rotates about its C.G.

d. none of these.

10. Pick up the correct statement from the following:

a. Mcg = (M M2 + r2) where letters carry their usual meanings

b. Tcp = m2 + T2 where letters carry their usual meanings

c. The torque which when acting alone would produce maximum shear stress equal to the maximum shear stress caused by the combined bending and torsion, is called equivalent torque

d. All the above.

11. The ratio of the flexural strengths of two square beams one placed with its two sides horizontal and the other placed with one diagonal vertical, diagonal, is

a. 2

b. 2

c. 5

d. 7

12. If the depth of a simply supported beam carrying an isolated load at its centre, is doubled, the deflection of the beam at the centre will be changed by a factor of

a. 1/2

b. 8

c. 1/8

d. 4

13. A steady uniform flow is through

a. a long pipe at constant rate

b. an expanding tube at constant rate

c. an expanding tube at increasing rate

d. a long pipe at increasing rate.

14. Practical fluids possess

a. viscosity

b. surface tension

c. compressibility

d. all the above.

15. Liquids

a. cannot be compressed

b. do not occupy definite shape

c. are not affected by change in pressure and temperature

d. none of these.

16. Discharge over an ogee weir remains the same as that of

a. sharp crested weir

b. triangular weir

c. cippoletti weir

d. drowned weir.

17. The fundamental equation of specific gravity (G), dry density (γd), unit weight of water (γω) and void ratio (e), is

a. e =

b. G =

c. γd =

d. γω =

18. Pick up the correct statement from the following:

a. When stress decreases, void, ratio decreases

b. When stress decreases, coefficient of permeability decreases

c. When stress decreases, coefficient of volume change decreases

d. When stress decreases void ratio, co-effi-cients of per-meability and volume change decrease.

19. The minimum water content at which the soil just begins to crumble when rolled into threads 3 mm in diameter, is known

a. liquid limit

b. plastic limit

c. shrinkage limit

d. permeability limit.

20. An unsaturated 100 cm3 sample of soil weighs 190 g. If its dried weight is 160 g, water content of the soil, is

a. 0.188

b. 0.288

c. 0.388

d. 0.488

21. The maximum ratio of span to depth of a slab simply supported and spanning in two directions, is

a. 30

b. 35

c. 40

d. 15

22. Pick up the correct statement from the following:

a. A pile is a slender member which transfers the load through its lower end on a strong strata

b. A pile is a slender member which transfers its load to the surrounding soil

c. A pile is a slender member which transfers its load by friction

d. A pile is a cylindrical body of concrete which transfers the load at a depth greater than its width.

23. The maximum diameter of a bar used in a ribbed slab, is

a. 6 mm

b. 20 mm

c. 22 mm

d. 24 mm

24. High strength concrete is used in prestressed member

a. to ovecome bursting stresses at the ends

b. to provide high bond stresses

c. to overcome cracks due to shrinkage

d. all the above.

25. The stone masonry of finely dressed stones laid in cement or lime, is

a. random rubble masonry

b. coursed rubble masonry

c. dry rubble masonry

d. ashlar masonry.

26. Pick up the incorrect statement from the following :

a. No  timbering is  required  for  shallow trenches

b. Shallow foundations can be constructed on made-up soil

c. Grillage   foundation   is   classified as a shallow foundation

d. Black cotton soil is very good for foundation bed.

27. Higher workability of concrete is required if the structure is

a. made with cement concrete

b. thick and reinfored

c. thin and heavily reinforced

d. thick and heavily reinfored.

28. According to Water-Cement Ratio Law, the strength of workable plastic concrete

a. depends upon the amount of water used in the mix

b. does not depend upon the quality of cement mixed with aggregates

c. does not depend upon the quanity of cement mixed with aggregates

d. all the above.

29. Pick up the incorrect statement from the following :

a. Water disinfected with ozone is free from any odour

b. Ozone removes bacterias as well as colour and odour

c. ozonised water becomes tasteless

d. None of these.

30. Sunlight

a. helps growth of bacterias

b. impedes growth of algae

c. increases dissolved oxygen content

d. reduces turbidity.

31. Dried sewage after treatment is used as

a. fertilizer

b. building material

c. chemical for lowering B.O.D.

d. base material for paints.

32. During putrifaction process of sewage the gas given off, is

a. hydrogen

b. ammonia

c. methane

d. all the above.

33. The Lacey's regime velocity is proportional to

a. R1/2 S3/4

b. Q3/4 S1/3

c. R3/4 S1/3

d. R2/3 S1/2

34. The velocity of drainage water in the barrels of a syphon-aqueduct, is normally limited to

a. 1 to 2 m per second

b. 2 to 3 m per second

c. 3 to 4 m per second

d. 4 to 5 m per second.

35. While deciding the alignment of a proposed canal taking off from a river at B, three alignments of approximately equal lengths are available. These cross a drainages at C1, C2 and C3 where drainage bed levels C1 > C2 > C3, you will provide at site C3

a. an aqueduct

b. a syphon aqueduct

c. a super passage

d. a syphon.

36. The depth of the crest of a scouring sluice below the crest of a head regulator, is generally kept

a. 1.20 m

b. 2.20 m

c. 3.20 m

d. 4.90 m

37. The minimum cross fall of shoulders is kept

a. 1.0%

b. 1.5%

c. 2.5%

d. 3%

38. An exceptional grade may be provided upto 1 in 12 along hill roads, if the length does not exceed

a. 60 m per km

b. 75 m per km

c. 90 m per km

d. 100 m per km

39. One-way streets are generally provided in crowded cities as, these

a. reduce delays to vehicles

b. permit higher speed

c. reduce the number of accidents

d. all the above.

40. Pick up the correct statement from the following:

a. Curve length should be at least 150 metres for a deflection angle of 5 degress

b. For every degree decrease in the deflection angle, 30 metre length of curve to be increased

c. If the deflection angle is less than 1°, no curve is designed

d. All the above.

41. The assumption on which the trapezoidal formula for volumes is based, is

a. The end sections are parallel planes

b. The mid-area of a pyramid is half the average area of the ends

c. The volume of the prismoidal is over-es-timted and hence a prismoidal correction is applied

d. All the above.

42. The brick work is not measured in cu m in case of

a. One or more than one brick wall

b. Brick work in arches

c. Reinforced brick work

d. Half brick wall.

43. The total length of a cranked bar through a distance (d) at 45° in case of a beam of effective length L, is

a. L + 0.42 d

b. L + 2 x 0.42 d

c. L - 0.42 d

d. L - 2 x 0.4 d

44. The following item of earth work is not measured separately.

a. Site clearence

b. dead men

c. Steps in deep exavaction

d. All the above.

45. The final selection of a construction site, is done by

a. local civil authority representative

b. representative of engineer authority

c. representative of administration

d. all the above.

46. The main disadvantage of line organisation, is

a. rigid structure

b. extraordinary delay in communications

c. top level executions over work

d. all the above.

47. The difference between the time avail-to do a job and the time required to do the job, is known as

a. event

b. float

c. duration

d. constraint.

48. Critical path method

a. provides a realistic approach to daily problems

b. avoids delays which are very common in bar charts

c. was invented by Morgan R. Walker of Dupot and James E. Kalley or Remington U.S.A. in 1957

d. All the above.

49. According to I.C.A.O. the slope of transitional surface at right angles to the centre line of runway, is kept

a. 1 in 5

b. 1 in 6

c. 1 in 7

d. 1 in 10

50. Pick up the correct statement from the following:

a. The propellers which are driven by turbine engines, are technically called turboprops

b. The aircrafts which obtain the thrust directly from turbine engine, are called turbo-jets

c. The aircraft which is filled with a fan infront of turbo-jet engine, is called turbofan propelled aeroplane

d. All the above.