***Sub Engineer***

1. Diaphragm of a surveying telescope is held inside

a. objective

b. telescope tube at its mid point

c. telescope at the end nearer the eye-piece

d. telescope at its end nearer the objective.

2. In a perfect prismatic compass

a. ends of the needle and pivot are in same vertical and horizontal planes

b. pivot is vertically over the centre of the graduated circle

c. needle is always kept sensitive

d. all the above.

3. The difference in the lengths of an arc and its subtended chord on the earth surface for a distance of 18.2 km, is only

a. 1 cm

b. 5 cm

c. 10 cm

d. 100 cm.

4. Pick up the correct statement from the following :

a. A level line lies in level surface

b. A horizontal surface is normal to the direction of gravity at only one point

c. A horizontal line is tangential to the level surface

d. All the above.

5. Pick up the correct statement from the following:

a. The edges of a finished wrought iron, are rough due to red shortage

b. Pig iron (charcoal) is manufactured from magnetic ore (Fe3O4)

c. For the manufacture of wrought iron, non-sulphurous fuel is not necessary

d. All the above.

6. A prime coat is given to steel work with

a. an oxide of iron paint

b. a mixture of white lead and lead paint

c. a special paint

d. cement paint.

7. The plastics made from cellulose resin

a. are as clear as glass

b. are tough and strong

c. possess excellent electrical properties

d. All the above.

8. A 1st class brick immersed in water for 24 hours, should not absorb water (by weight) more than

a. 15%

b. 20%

c. 25%

d. 5%.

9. The ratio of lateral strain to axial strain of a homogeneous material, is known

a. Yield ratio

b. Hooke's ratio

c. Poisson's ratio

d. Plastic ratio.

10. The locus of the moment of inertia about inclined axes to the principal axis, is

a. straight line

b. parabola

c. circle

d. ellipse.

11. Pick up the correct statement from the following :

a. A ductile material has large plastic zone

b. A brittle material has no plastic zone

c. A rigid material has no plastic zone

d. All the above.

12. The section modulus of a rectangular section is proportional to

a. area of the section

b. square of the area of the section

c. product of the area and depth

d. product of the area and width

13. Piezometers are used to measure

a. pressure in water channels, pipes etc.

b. difference in pressure at two points

c. atmospheric pressure

d. very low pressure.

14. To avoid the tendency of separation of liquid flow, the most suitable ratio of the diameters of the throat and the pipe, is

a. 1/4 to 1/8

b. 1/3 to 1/2

c. 1/2 to 3/4.

d. none of these.

15. The depth of the centre of pressure on a vertical rectangular gate (4 m wide, 3 m high) with water upto top surface, is

a. 1.0 m

b. 1.5 m

c. 2.0 m

d. 2.5 m.

16. The Empirical formula  for discharge over large rectangular weirs, is known as

a. Francis formula

b. Bazin formula

c. Rehbook formula

d. Kutter's formula.

17. The minimum depth of building foundations on

a. sandy soils is 80 cm to 100 cm

b. clay soils is 90 cm to 160 cm

c. rocky soils is 5 cm to 50 cm

d. all the above.

18. The ratio of the volume of voids to the volume of soil solids in a given soil mass, is known

a. porosity

b. specific gravity

c. void ratio

d. water content.

19. Fundamental relationship between dry density (γd), specific gravity (G), water content (ω) and percentage of air voids (na) is :

a. γd =

b. γd =

c. γd =

d. γd =

20. Soil classification of composite soils, exclusively based on the particle size distribution, is known

a. particle classification

b. textural classification

c. High Way Research Board classification

d. unified soil classification.

21. An R.C.C. beam of 25 cm width and 50 cm effective depth has a clear span of 6 metres and carries a U.D.L. of 3000 kg/m inclusive of its self weight. If the lever arm constant for the section is 0.865, the maximum intensity of shear stress, is

a. 8.3 kg/cm2

b. 7.6 kg/cm2

c. 21.5 kg/cm2

d. 11.4 kg/cm2

22. If the diameter of the main reinforcement in a slab is 16 mm, the concrete cover to main bars is

a. 13 mm

b. 14 mm

c. 15 mm

d. 16 mm

23. Thickened part of a flat slab over its supporting column, is technically known as

a. drop panel

b. capital

c. column head

d. none of these.

24. The advantage of reinforced concrete, is due to

a. fire-resisting and durability

b. economy because of less maintenance cost

c. moulding in any desired shape

d. All the above.

25. In soft clay of low bearing capacity, the type of steel pile generally used, is

a. screw pile

b. disc pile

c. pipe pile

d. raking pile.

26. The angular steps used for changing direction of the stairs, are called

a. angular steps

b. winders

c. radial steps

d. circular steps.

27. Pick up the correct statement from the following:

a. According to the petrological characteristics, concrete aggregates are classified as heavy weight, normal weight and light weight

b. According to the shape of the particles, concrete aggregates are classified as rounded irregular, angular and flaky

c. According to the surface texture of the particles, the concrete aggregates are classified as glassy, smooth, granular, rough, crystalline, honey combed and porous

d. All the above.

28. The internal dimensions of a ware house are 15 m x 5.6 m, and the maximum height of piles is 2.70 m, the maximum number of bags to be stored in two piles, are

a. 2000 bags

b. 2500 bags

c. 3000 bags

d. 4000 bags

29. The head against which the motor works for lifting water, is

a. maximum depression head

b. velocity head

c. frictional losses in the delivery pipe

d. all the above.

30. By boiling water, hardness can be removed if it is due to

a. magnesium sulphate

b. calcium nitrate

c. calcium bicarbonate

d. none of these.

31. The rainfall at any place may be determined by

a. its intensity

b. its direction

c. its frequency

d. all the above.

32. For detecting the nitrites in the sewage, the matching colour may be developed by adding

a. Pottassium permanganate

b. Sulphuric acid and napthamine

c. Phenol-di-sulphuric acid pottassium hydroxide

d. None of these.

33. The intensity of irrigation means

a. percentage of culturable commanded area to be irrigated annually

b. percentage of gross commanded area to be irrigated annually

c. percentage of the mean of culturable commanded area and the gross commanded area to be irrigated annually

d. total depth of water supplied by the number of waterings.

34. For the design of major hydraulic structures on the canals, the method generally preferred to, is based on

a. Bligh's theory

b. Electrical analogy method

c. The relaxation method

d. Khosla's method of independent variables.

35. Pick up the incorrect statement from the following

a. A bank in the cross-section of a canal is provided on the right

b. A bank in the cross-section of a canal is provided on the left

c. A 'dowla' in the cross-section of a canal is provided on the left

d. None of these.

36. Irrigation canals are generally aligned along

a. ridge line

b. contour line

c. valley line

d. straight line.

37. Maximum number of vehicles that can pass a given point on a lane during one hour without creating unreasonable delay, is known as

a. traffic density of lane

b. basic capacity of lane

c. probable capacity of lane

d. practical capacity of lane.

38. For a 6.6 m wide two lane pavement, berms are provided on either side having a width of

a. 1.25 m

b. 1.50 m

c. 1.75 m

d. 2.00 m

39. The ability of a driver to stop the vehicle moving with the designed speed, depends upon:

a. brake reaction time

b. efficiency of the brakes

c. frictional resistance between road surface and vehicle

d. all the above.

40. One-way streets are generally provided in crowded cities as, these

a. reduce delays to vehicles

b. permit higher speed

c. reduce the number of accidents

d. all the above.

41. Pick up the correct statement from the following:

a. In order to check up the average depth of excavation, 'Dead mans' are left at the mid-widths of borrow pits

b. The earthwork calculation in excavation is made from the difference in levels obtained with a level

c. The earth work in excavation to form the road embankment includes the formation of correct profile and depositing the soil in layers

d. All the above.

42. Anti-siphonage pipe is connected to

a. Main soil pipe

b. Bottom of P trap W.C.

c. Top of P trap W.C.

d. Side of water closet.

43. The trap which is provided to disconnect the house drain from the street sewer is called

a. Intercepting trap

b. Interception manhole

c. Intercepterchamber

d. All the above.

44. Pick up the incorrect statement from the following:

a. The built up covered area at the floor level of any storey of a building is called plinth area

b. The usable covered area of the rooms of any storey of a building is called carpet area

c. The carpet area of a building along with area of its kitchen, pantry, store, lavatory, bath room and glazed verandah, is called floor area

d. None of these.

45. Frequency distribution curves

a. having a single lump, are called uninodal curves

b. if symmetrical, are called normal curves

c. if not symmetrical, are called skew curves

d. all the above.

46. Various activities of a project, are shown on bar charts by

a. vertical lines

b. horizontal lines

c. dots

d. crosses.

47. PERT analysis is based on

a. optimistic time

b. pessimistic time

c. most likely time

d. all the above.

48. The main disadvantage of line organisation, is

a. rigid structure

b. extraordinary delay in communications

c. top level executions over work

d. all the above.

49. The air is blowing at a speed of 75 knots in the direction of the aircraft flying at a ground speed of 775 knots. The air speed of the aircraft, is

a. 775 knots

b. 75 knots

c. 850 knots

d. 675 knots

50. For the proposed air port, the survey project provides

a. topograhic plan

b. grading plan

c. drainage plan

d. all the above.