***Sub Engineer***

1. The longitudinal section of the surface of bubble tube is

a. straight

b. circular

c. parabolic

d. elliptic.

2. Transition curves are introduced at either end of a circular curve, to obtain

a. gradually decrease of curvature from zero at the tangent point to the specified quantity at the junction of the transition curve with main curve

b. gradual increase of super-elevation from zero at the tangent point to the specified amount at the junction of the transition curve with main curve

c. gradual change of gradient from zero at the tangent point to the specified amount at the junction of the transition curve with main curve

d. none of these.

3. The limiting length of an offset does not depend upon

a. accuracy of the work

b. method of setting out perpendiculars

c. scale of plotting

d. indefinite features to be surveyed.

4. The combined effect of curvature and refraction over a distance L kilometres is

a. 67.2 L2 mm

b. 76.3 L2 mm

c. 64.5 L2 mm

d. none of these.

5. Lime mortar is generally made with

a. fat lime

b. hydraulic lime

c. plain lime

d. none of these.

6. Stainless steel contains

a. 18% of chromuim and 8% nickel

b. 8% of chromium and 18% of nickel

c. 12% of chromium and 36% of nickel

d. 36% of chromium and 12% of nickel.

7. The minimum compressive strength of Ilnd class bricks should be

a. 75 kg/cm2

b. 90 kg/cm2

c. 100 kg/cm2

d. 120 kg/cm2

8. Pick up the correct statement from the following:

a. Air bubbles in casting produce a dull sound by tapping their surfaces lightly with a hammer

b. Cupola furnace is used for the manufacture of cast iron

c. Red short iron is of no value for welding purpose

d. All the above.

9. Gradually applied static loads do not change with time their

a. magnitude

b. direction

c. point of application

d. all the above.

10. Beams composed of more than one material, rigidly connected together so as to behave as one piece, are known as

a. Compound beams

b. Indeterminate beams

c. Determinate beams

d. Composite beams.

11. A diagram which shows the variations of the axial load for all sections of the span of a beam, is called

a. shear force diagram

b. thrust diagram

c. stress diagram

d. none of these.

12. A bending moment may be defined as :

a. Arithmetic sum of the moments of all the forces on either side of the section

b. Arithmetic sum of the forces on either side of the section

c. Algebraic sum of the moments of all the forces on either side of the section

d. None of these.

13. The flow in open channel is said to be critical if the Froude number is :

a. less than 1.0

b. equal to 1.0

c. greater than 1.0

d. None of these.

14. The ratio of the inertia and gravitational force acting in any flow, ignoring other forces, is called

a. Euler number

b. Frode number

c. Reynold number

d. Weber number.

15. The ratio of the percentage error in the discharge and percentage error in the measurement of head, over rectangular notch, is

a. 1/2

b. 2/3

c. 3/2

d. 3/4

16. Back water curve is caused if

a. friction head loss is more than the bed slope

b. pressure is due to weir in the channel

c. there is an increase in width of the channel

d. none of these.

17. The triaxial apparatus is usually used for

a. unconsolidated-undrained test

b. consolidated-undrained test

c. drained test

d. all the above tests.

18. Pick up the correct statement from the following:

a. Kaolinite shows a very little sign of swelling on wetting

b. Kaolinite when wet, becomes moderately plastic because negative electro magnetic charges on platelets attrack water

c. Kaolinite is also called China clay

d. All the above.

19. The consolidation time for soils

a. decreases with increasing permeability

b. increases rapidly with increasing size of soil mass

c. is independent of the magnitude of the stress change.

d. All the above.

20. Terzaghi's analysis assumes :

a. soil is homogeneous and isotropic

b. elastic zone has straight boundaries inclined at ψ = φ to the horizontal and plastic zones fully developed

c. failure zones do not extend above the horizontal plane through the base of the footing

d. all the above.

21. The amount of reinformcement for main bars in a slab, is based upon

a. minimum bending moment

b. maximum bending moment

c. maximum shear force

d. minimum shear force.

22. The stem of a cantilever retaining wall which retains earth level with top is 6 m. If the angle of repose and weight of the soil per cubic metre are 30° and 2000 kg respectively, the effective width of the stem at the bottom, is

a. 52.5

b. 53.5

c. 54.5

d. 55.5

23. An R.C.C. beam of 25 cm width and 50 cm effective depth has a clear span of 6 metres and carries a U.D.L. of 3000 kg/m inclusive of its self weight. If the lever arm constant for the section is 0.865, the maximum intensity of shear stress, is

a. 8.3 kg/cm2

b. 7.6 kg/cm2

c. 21.5 kg/cm2

d. 11.4 kg/cm2

24. The steel generally used in R.C.C. work, is

a. stainless

b. mildsteel

c. high carbon steel

d. high tension steel.

25. The arrangement made to support an unsafe structure temporarily, is known as

a. shoring

b. scaffolding

c. underpinning

d. jacking

26. The type of bond in which every course contains both headers and stretchers, is called

a. English bond

b. Flemish bond

c. Russian band

d. Mixed bond.

27. A concrete using an air entrained cement

a. has more resistance to weathering

b. is more plastic and workable

c. is free from segregation and bleeding

d. all the above.

28. The rock which is not calcareous, is :

a. macl

b. chalk

c. laterite

d. none of these.

29. P.V.C. pipes can withstand pressure bead of water upto

a. 50 m

b. 75 m

c. 100 m

d. 125 m.

30. Per capita demand of water is calculated in litres

a. per person per day

b. per person per month

c. per person per year

d. none of these.

31. In circular sewers if depth of flow is 0.2 times the full depth, the nominal gradient,

a. is only provided

b. is doubled

c. is trebled

d. is not enough.

32. The pH value of sewage is determined with the help of

a. Imhoff Cone

b. turbidimeter

c. potentiometer

d. none of these.

33. If d1 is the depth of cutting, d2 is the height of the bank from bed level r2 : 1 and r1 : 1 are the slopes in filling and cutting respectively, the horizontal distance n between the bed and bank, is

a. x = r1 d1

b. x = r2 d2

c. x = d1 / r1

d. x = d2 / r2

34. Bed bars in a canal are provided

a. to watch the general behaviour of canal

b. to measure the discharge

c. to raise the supply level

d. to control the silting

35. The down stream expansion head of a guide bank is extended to subtend an angle at the centre, equal to

a. 30°

b. 45°

c. 60°

d. 90°

36. For cereal crops the most commonly adopted method of irrigation, is

a. free flowing method

b. check method

c. furrow method

d. sprinkling method.

37. The width of the right of way in urban area, is kept between 24 m to 60 m for

a. National Highways

b. State Highways

c. Both (a) and (b)

d. None of these.

38. The number of vehicles moving in a specified direction on a roadway that pass a given point during specified unit of time, is called

a. traffic volume

b. traffic density

c. basic capacity

d. traffic capacity.

39. Camber in pavements is provided by

a. straight line method

b. parabola method

c. straight at the edges and parabolic at the crown

d. all the above.

40. If degree of a road curve is defined by assuming the standard length of an arc as 30 metres, the radius of 1° curve is equal

a. 1719 m

b. 1146 m

c. 1046 m

d. 1619 m

41. Pick up the incorrect statement from the following:

a. Lead is the average horizontal straight distance between the borrow pit and the place of spreading soil

b. The lead is calculated for each block of the excavated area

c. The unit of lead is 50 m for a distance upto 500 m

d. The unit of lead is 1 km where the lead exceeds 2 km.

42. The inspection pit or chamber is a manhole provided in a base drainage system

a. at every change of gradient

b. at every 30 m intervals

c. at the point where vertical soil pipe joins the house drain

d. All the above.

43. The cross-sections for a highway is taken at

a. 30 metres apart

b. intermediate points having abrupt change in gradient

c. the starting end points of the curves

d. All the above.

44. The assumption on which the trapezoidal formula for volumes is based, is

a. The end sections are parallel planes

b. The mid-area of a pyramid is half the average area of the ends

c. The volume of the prismoidal is over-es-timted and hence a prismoidal correction is applied

d. All the above.

45. Pre-tender stage requires

a. selection of site

b. formalisation of alignment of work

c. formalisation of designs and preparation of estimate

d. all the above.

46. Various activities of a project, are shown on bar charts by

a. vertical lines

b. horizontal lines

c. dots

d. crosses.

47. Critical path lies along the activities having total float

a. positive

b. negative

c. zero

d. same.

48. The time which results in the leasi, possible construction cost of an activity, is known

a. normal time

b. slow time

c. crash time

d. standard time.

49. Pick up the component not applicable to aeroplanes

a. engines

b. air screw

c. fuse lage

d. none of these.

50. The strength of winds is measured with the help of

a. Benfort scale

b. Wind indicator

c. Barometres

d. None of these.