***Sub Engineer***

1. Simpson's rule for calculating areas states that the area enclosed by a curvilinear figure divided into an even number of strips of equal width, is equal to

a. one third the width of a strip, multiplied by the sum of two extreme offsets, twice the sum of remaining odd offsets and four times the sum of the even offsets

b. one third the width of a strip, multiplied by the sum of two extreme offsets, four times the sum of the remaining odd offsets, and twice the sum of the even offsets

c. one sixth the width of a strip, multiplied by the sum of the two extreme offsets, twice the sum of remaining odd offsets and four times the sum of the even offsets

d. none of these

2. The theodolites used for making tacheometric observations by optical wedge system, are

a. provided with stadia hairs in front of eye piece

b. not provided with stadia hairs at all

c. fitted with a glass wedge inside the telescope

d. fitted with a glass wedge in front of telescope.

3. The method of reversal

a. is usually directed to examine whether a certain part is truly parallel or perpendicular to another

b. makes the erroneous relationship between parts evident

c. both (a) and (b)

d. neither (a) nor (b).

4. For indirect ranging, number of ranging rods required, is

a. 2

b. 3

c. 4

d. 5

5. The most commonly used synthetic abrasive is

a. aluminium carbide

b. boric acid

c. silicon

d. All of these.

6. Stones used for rubble masonry must be

a. soft

b. hard

c. light

d. heavy.

7. Putty is

a. made with finely powdered chalk and linseed oil

b. used for fixing glass panes

c. softened by a solution of pearl ash and quick-lime soaked in water

d. all the above.

8. Bitumen felt

a. is used as water proofing material

b. is used as damp proofing material

c. is made from bitumen and hessian fibres

d. all the above.

9. The ratio of crippling loads of a column having both the ends fixed to the column having both the ends hinged, is

a. 1

b. 2

c. 3

d. 4

10. The normal and tangential components of stress on an inclined plane through θ° to the direction of the force, will be equal if θ is

a. 45°

b. 30°

c. 60°

d. 90°

11. If a constant section beam is subjected to a uniform bending moment throughout, its length bends to

a. a circular arc

b. a parabolic arc

c. a catenary

d. none of these.

12. The ratio of the tensile stress developed in the wall of a boiler in the circumferential direction to the tensile stress in the axial direction, is

a. 4

b. 3

c. 2

d. 1

13. The total pressure force on a plane area is equal to the area multiplied by the intensity of pressure at its centroid, if

a. area is horizontal

b. area is vertical

c. area is inclined

d. all the above.

14. Power transmitted through a pipe is maximum when friction head loss, is

a. one-half of the total head supplied

b. one-third of the total head supplied

c. one-fouth of the total head supplied

d. equal to the total head supplied.

15. From a nozzle exposed to atmosphere, the liquid jet traverses

a. a straight line

b. a circular path

c. an elliptical path

d. a parabolic path.

16. Pick up the correct statement from the following :

a. When the length of the tube is equal to diameter of the internal mouth piece, the jet of liquid comes out without touching the sides of the tube

b. When the length of the tube is three times the diameter of the internal mouth piece, the jet diameter is equal to diameter of the tube.

c. both (a) and (b)

d. Neither (a) nor (b).

17. The liquid limit and plastic limit exist in

a. sandy soils

b. silty soils

c. gravel soils

d. clay soils.

18. The shearing strength of a cohesion-less soil depends upon

a. dry density

b. rate of loading

c. confining pressure

d. nature of loading.

19. A pycnometer is used to determine

a. voids ratio

b. dry density

c. water content

d. density index.

20. Determination of water content of a soil sample suspected to contain gypsum is made by drying the sample for longer period at a temperature not more than

a. 60°C

b. 80°C

c. 100°C

d. 110°C

21. In a slab, the pitch of the main reinforcement should not exceed its effective depth

a. three times

b. four times

c. five times

d. two times.

22. If the loading on a prestressed rectangular beam, is uniformly distributed, the tendon to be provided should be .

a. parabolic with convexity downward

b. parabolic with convexity upward

c. straight above centroidal axis

d. none of these.

23. A flat slab is supported

a. on columns

b. on beams and columns

c. on columns monolithicaily built with slab

d. all the above

24. An intermediate T-beam reinforced with two layers of tensile steel with clear cover 13 cm encasted with the floor of a hall 12 metres by 7 metres, is spaced at 3 metres from adjoining beams and if the width of the beam is 20 cm, the breadth of the flange is

a. 233 cm

b. 176 cm

c. 236 cm

d. 255 cm

25. Pick up the incorrect statement from the following :

a. No  timbering is  required  for  shallow trenches

b. Shallow foundations can be constructed on made-up soil

c. Grillage   foundation   is   classified as a shallow foundation

d. Black cotton soil is very good for foundation bed.

26. The portion of a brick cut across the width, is called

a. closer

b. half brick

c. bed

d. bat.

27. Efflorescence in cement is caused due to an excess of

a. iron oxide

b. silica

c. alkalis

d. magnesium oxide.

28. ISI has specified full strength of concrete after

a. 14 days

b. 21 days

c. 28 days

d. none of these.

29. If Q is discharge is cubicmetres per sec and D is the economical diameter of the pipe. According to Lea

a. D = 0.67 to 0.87 Q

b. D = 0.77 to 0.97 Q

c. D = 0.97 to 1.22 Q

d. D = 1.22 to 1.33 Q.

30. Pick up the incorrect statement from the following. In take of water supply should

a. be nearer to the treatment plant

b. receive water from purer zone of the source

c. be located downstream of waste water disposal point

d. remain easily accessible during floods.

31. The ratio of design discharge to the surface area of a sedimentation tank is called

a. surface loading

b. overflow rate

c. overflow velocity

d. all of these.

32. In a grit chamber of a sewage treatment plant,

a. flow velocity 0.15 m to 0.3 m/sec is kept

b. depth of 0.9 m to 1.20 m is kept

c. one minute of detention period is kept

d. all the above.

33. Pick up the incorrect statement from the following

a. A bank in the cross-section of a canal is provided on the right

b. A bank in the cross-section of a canal is provided on the left

c. A 'dowla' in the cross-section of a canal is provided on the left

d. None of these.

34. Lacey's equation for calculating flood discharge in rivers, is

a. V = 10.8 R1/2 S1/2

b. V = 10.8 R2/3 S1/2

c. V = 10.8 R2/3 S1/3

d. V = 10.8 R1/3 S2/3

35. For diversion of flood water of rivers, the type of canal constructed, is

a. ridge canal

b. perennial canal

c. inundation canal

d. canal.

36. For a unique design of a channel by Kennedy's theory

a. its breadth must only be known

b. its depth must only be known

c. its breadth and depth ratio must only be known

d. all the above.

37. Normal formation width of a hill road for one-way traffic, is

a. 4.8 m

b. 6.6 m

c. 7.2 m

d. 8 m

38. Width of a rotary round should be equal to

a. twice the width of narrowest radial road

b. width of the widest road

c. width of the widest road plus the width of one lane

d. width of the widest road plus 2 metres.

39. Reinforcement in cement concrete slab of road pavements, is placed

a. transversely

b. longitudinally and transversely

c. in the form of welded wire mesh

d. none of these.

40. Roadway width for a National highways and State highways (two-lanes) is

a. 12 m

b. 9 m

c. 9.5 m

d. 15 m

41. Cost of fittings and their fixing is specified for the following sanitary fittings

a. Flushing pipes

b. Lavatary basins

c. Sinks

d. All the above.

42. If the formation level of a highway has a uniform gradient for a particular length, and the ground is also having a longitudinal slope, the earthwork may be calculated by

a. Mid-section formula

b. Trapezoidal formula

c. Prismoidal formula

d. All the above.

43. The 'centre line method' is specially adopted for estimating

a. Hexagonal buildings

b. Octagonal buildings

c. Other geometrical shaped buildings

d. All the above.

44. Pick up the correct statement in case of water supply.

a. Cutting through walls and floors are included with the item

b. Pipes are classified according to their sizes and quality

c. In laying pipes, the method of jointing and fixing is specifically specified

d. All the above.

45. The main principle of an organisation, is

a. cohegency

b. effective control at all levels

c. delegation of authority

d. all the above.

46. The object of technical planning, is

a. preparation of estimates

b. initiating the procurement action of resources

c. taking remedial action for likely bottleneck in the execution

d. all the above.

47. A construction schedule is prepared after collecting

a. output of labour

b. output of machinery

c. quantity of various items

d. all the above.

48. The final selection of a construction site, is done by

a. local civil authority representative

b. representative of engineer authority

c. representative of administration

d. all the above.

49. According to the International Civil Aviation Organisation (I.C.A.O.) the strength of runway pavements, have been coded by

a. Last Seven English alphabets

b. First Seven English alphabets

c. First seven numbers

d. None of these.

50. The strength of winds is measured with the help of

a. Benfort scale

b. Wind indicator

c. Barometres

d. None of these.