***Sub Engineer***

1. True meridians are generally preferred to magnetic meridians because

a. these converge to a point

b. these change due to change in time

c. these remain constant.

d. None of these.

2. While rotating the theodolite in the horizontal plane, the bubble of the bubble tube takes up the same position in its tube, it indicates

a. the rotation axis is vertical

b. the trummion axis is horizontal

c. the line of collimation is perpendicular to vertical axis

d. none of the above

3. Designation of a curve is made by :

a. angle subtended by a chord of any length

b. angle subtended by an arc of specified length

c. radius of the curve

d. curvature of the curve.

4. Bergchrund is a topograhical feature in

a. plains

b. water bodies

c. hills

d. glaciated region

5. For high grade instruments the steel preferred to, is

a. cast steel

b. bessemer steel

c. mild steel

d. whitworth compressed steel.

6. Vanadium steel is generally used for

a. railway switches and crossing

b. bearing balls

c. magnets

d. axles and springs.

7. Bitumen may be dissolved in

a. water

b. sodium chloride

c. carbon disulphide

d. none of these.

8. The steel used for the manufacture of rails, is

a. Bessemer steel

b. mild steel

c. cast steel

d. stainless steel.

9. A material is said to be perfectly elastic if

a. it regains its original shape on removal of the load

b. It regains its original shape partially on removal of the load

c. it does not regain its original shape at all

d. none of these.

10. In a shaft, the shear stress is not directly proportional to

a. radius of the shaft

b. angle of twist

c. length of the shaft

d. modulus of rigidity.

11. If the width of a simply supported beam carrying an isolated load at its centre is doubled, the deflection of the beam at the centre is changed by

a. 4 times

b. 8 times

c. 1/2 times

d. 3 times.

12. If two forces acting at a joint are not along the straight line, then for the equilibrium of the joint

a. one of the forces must be zero

b. each force must be zero

c. forces must be equal and of the same sign

d. forces must be equal in magnitude but opposite in sign.

13. The discharge through a V-notch weir varies as

a. 1/H

b. H3/2

c. H7/2

d. H5/2.

14. For the two dimensional flow, the stream function is given by ψ = 2xy. The velocity at a point (3, 4) is

a. 8 m/sec

b. 10 m/sec

c. 12 m/sec

d. 15 m/sec.

15. Poise is the unit of

a. viscosity

b. velocity gradient

c. mass density

d. kinematic viscous.

16. Atmospheric pressure varies with

a. altitude

b. temperature

c. weather conditions

d. all of the above.

17. The active earth pressure of a soil is proportional to (where φ is the angle of friction of the soil)

a. tan (45° - φ)

b. tan2 (45° + φ/2)

c. tan2 (45° - φ/2)

d. tan (45° + φ)

18. Fundamental relationship between dry density (γd), bulk density (γ) and water content (ω), is :

a. γ =

b. γd =

c. ω =

d. ω =

19. If the cohesive force, (c), is 1.5 t/m2, the density (γ) of the soil is 2.0 t/m3, factor of safety (F) is 1.5 and stability factor (Sn) is 0.05, the safe height of the §lope, is

a. 5 metres

b. 8 metres

c. 10 metres

d. 12 metres

20. If the unit weight of sand particles is 2.696 g/cc. and porosity in loose state is 44%, the critical hydraulic gradient for quick sand condition, is

a. 0.92

b. 0.93

c. 0.94

d. 0.95

21. A column is regarded as long column if the ratio of its effective length and lateral dimension, exceeds

a. 15

b. 20

c. 25

d. 30

22. The diameter of longitudinal bars of a column should never be less than

a. 8 mm

b. 10 mm

c. 12 mm

d. none of these.

23. The maximum area of tension reinforcement in beams shall not exceed

a. 0.15%

b. 1.5%

c. 4%

d. 1%

24. In case the factor of safety against sliding is less than 1.5, a portion of slab is constructed downwards at the end of the heel slab, which is known as

a. a key

b. a cut-off wall

c. a rib

d. all the above.

25. The pile which is provided with a bulb filled with concrete at its lower end, is known as

a. Mac-Arthur pile

b. Raymond pile

c. Franki pile

d. none of these.

26. The maximum permissible differential settlement, in case of foundations in clayey soil, is usually limited to

a. 20 mm

b. 30 mm

c. 40 mm

d. 50 mm.

27. Slump test is done for

a. clay

b. sand

c. lime

d. concrete.

28. In the method of voids for determination of the quantity of cement paste, it is assumed that

a. Voids in fine aggregates are filled by the cement paste

b. Volume of fine aggregates is equal to total voids in coarse aggregates plus 10% extra

c. Volume of cement paste required is equal to total volume of voids in fine aggregates plus 15% extra

d. All the above.

29. The permissible amount of nitrites present in potable water, is

a. 15 ppm

b. 20 ppm

c. 45 ppm

d. Nil.

30. Total flow in stream is known

a. run off

b. stream flow

c. discharge

d. all the above.

31. The settlement of a particle in sedimentation tank, is affected by

a. viscosity of water

b. size and shape of solid

c. specific gravity of solid

d. all the above.

32. If the diameter of sewer is 225 mm, the gradient required for generating self cleansing velocity, is

a. 1 in 60

b. 1 in 100

c. 1 in 120

d. none of these.

33. For standing crops in undulating sandy fields, the best method of irrigation, is

a. sprinkler irrigation

b. free flooding

c. check method

d. furrow method

34. The ratio of the rate of change of discharge of an outlet to the rate of change in level of water surface in a distributary at its normal depth, is known as

a. efficiency

b. sensitivity

c. flexibility

d. modular limit.

35. To control the silt entry into a distributary at head regulator, King's vanes are provided which are the walls

a. of height l/3rd to l/4th the depth of water in the main canal

b. spaced at interval of  times their heights

c. extending about one metre beyond a line drawn at an inclination of 2 : 1 to the axis of main canal

d. all the above.

36. The most suitable location of a canal head work, is

a. boulders stage of the river

b. delta stage of the river

c. rock stage of the river

d. trough stage of the river.

37. Minimum permissible speed on high speed roads, is decided on the basis of

a. 15 percentile cumulative frequency

b. 20 percentile cumulative frequency

c. 30 percentile cumulative frequency

d. 40 percentile cumulative frequency.

38. For a 6.6 m wide two lane pavement, berms are provided on either side having a width of

a. 1.25 m

b. 1.50 m

c. 1.75 m

d. 2.00 m

39. Pick up the correct statement from the following:

a. Various geometric design features and generally guided by ruling design speed

b. The design speed for a given highway should preferably be uniform

c. Abrupt change in the design speed should not be permitted

d. all the above.

40. Stability of hill slopes depends upon

a. angle of the slope

b. geological conditions

c. ground water conditions

d. all the above.

41. Pick up the excavation where measurements are made in square metres for payment.

a. surface dressing up to 15 cm depths

b. Surface excavation up to 30 cm depths

c. Both (b) and (c)

d. Both (a) and (b)

42. The measurement is not made in square metres in case of

a. D.P.C. (Damp proof course)

b. Form works

c. Concrete Jaffries

d. R.C. Chhajja.

43. While estimating the qualities for the construction of a building, the correct metric unit is

a. Cubic metre for area

b. Square metres for volume

c. Kilogram for weight

d. Litre for capacity

44. The area is measured correct to the nearest

a. 0.01 sqm

b. 0.02 sqm

c. 0.03 sqm

d. 0.04 sqm

45. Time and progress chart of a construction, is also known as

a. Gantt chart

b. Modified Mile stone chart

c. Critical path method chart

d. All the above.

46. The time which results in the leasi, possible construction cost of an activity, is known

a. normal time

b. slow time

c. crash time

d. standard time.

47. Construction team means

a. an architect

b. an owner

c. a contractor

d. all the above.

48. Residential buildings are treated as

a. light construction

b. heavy construction

c. industrial construction

d. private construction.

49. The height of the pilot's eye above the runway surface is assumed

a. 1 m

b. 3 m

c. 4 m

d. 5 m

50. For the taxiways, the following statement is true

a. The maximum longitudinal grade is 3%

b. The permissible rate of change of grade is 1%

c. The permissible transverse grade is 1.5%

d. All the above.