***Sub Engineer***

1. A theodolite is said to be in perfect adjustment if

a. rotation axis is vertical to the transit axis

b. transit axis is perpendicular to line of collimation

c. line of collimation sweeps out a vertical plane while the telescope is elevated or depressed

d. all the above.

2. While measuring the distance between two points along upgrade with the help of a 20 m chain, the forward end of the chain is shifted forward through a distance

a. 20 (sin θ - 1)

b. 20 (cos θ - 1)

c. 20 (sec θ - 1)

d. 20 (cosec θ - 1).

3. The best method of interpolation of contours, is by

a. estimation

b. graphical means

c. computation

d. all of these.

4. When the bubble of the level tube of a level, remains central

a. line of sight is horizontal

b. axis of the telescope is horizontal

c. line of collimation is horizontal

d. geometrical axis of the telescope is horizontal.

5. Strength of cement concrete primarily depends upon

a. quality of water

b. quantity of aggregate

c. quantity of cement

d. water-cement ratio.

6. In order of increasing percentage of silica, the correct sequence is

a. sandy clay, calcareous clay, pure clay

b. calcareous clay, pure clay, sandy clay

c. pure clay, sandy clay, calcareous clay

d. None of these.

7. Soundness test of cement determines

a. quality of free lime

b. ultimate strength

c. durability

d. initial setting

8. A good brick earth should contain :

a. about 50% to 60% of silica

b. not more than 5% of lime

c. about 5 to 6% of oxide of lime

d. All the above.

9. Pick up the correct statement from the following:

a. The pitch of a close coil spring, is very small

b. The angle made by the coil with horizontal, is called the angle of helix

c. In the open coil helical spring, the angle of helix is comparatively large

d. All the above.

10. In case of principal axes of a section

a. sum of moment of inertia is zero

b. difference of moment inertia is zero

c. product of moment of inertia is zero

d. none of these.

11. A long vertical member, subjected to an axial compressive load, is called

a. a column

b. a strut

c. a tie

d. a stanchion

12. Reactions at the supports of a structure can be determined by equating the algebraic sum of

a. horizontal forces to zero

b. vertical forces to zero

c. moment about any point to zero

d. all the above.

13. When water flows over a rectangular suppressed weir, the negative pressure created beneath the nappe

a. increases the discharge

b. decreases the discharge

c. does not effect the discharge

d. none of these.

14. Discharge over an ogee weir remains the same as that of

a. sharp crested weir

b. triangular weir

c. cippoletti weir

d. drowned weir.

15. Pick up the correct statement regarding Borda's mouth piece running full from the following :

a. Theoretical velocity at the outlet is 2gH

b. Coefficient of velocity is 1/2

c. Coefficient of contraction is 1

d. All the above.

16. A body of dimensions 1.5 m x 1.0 m x 2 m weighs 3000 kg in water. Its specific gravity is

a. 0.8

b. 0.9

c. 1.0

d. 1.1

17. For determining the moisture content of a soil sample, the following data is available Weight of container = 260 g, Weight of soil sample and = 320 g container, Weight of soil sample (dried) and = 310 g container. The moisture content of the soil sample, is

a. 15%

b. 18%

c. 20%

d. 25%

18. For determining the ultimate bearing capacity of soil, the recommended size of a square bearing plate to be used in load plate test should be 30 to 75 cm square with a minimum thickness of

a. 10 mm

b. 15 mm

c. 20 mm

d. 25 mm

19. The relationship between void ratio (e) and porosity ratio (n) is :

a. n =

b. e =

c. n =

d. e = n/(1 - n)

20. The plasticity index is the numerical difference between

a. liquid limit and plastic limit

b. plastic limit and shrinkage limit

c. liquid limit and shrinkage limit

d. none of these.

21. For a continuous floor slab supported on beams, the ratio of end span length and intermediate span length, is

a. 0.6

b. 0.7

c. 0.8

d. 0.9

22. Enlarged head of a supporting column of a flat slabs is technically .known as

a. top of the column

b. capital

c. drop panel

d. none of these.

23. Pick up the incorrect statement from the following. The intensity of horizontal shear stress at the elemental part of a beam section, is directly proportional to

a. area of the section

b. distance of the C.G. of the area from its neutral axis

c. moment of the beam section about its neutral axis

d. width of the beam.

24. The width of the flange of a L-beam, should be less than

a. one-sixth of the effective span

b. breadth of the rib + four times thickness of the slab

c. breadth of the rib + half clear distance between ribs

d. least of the above.

25. The angular steps used for changing direction of the stairs, are called

a. angular steps

b. winders

c. radial steps

d. circular steps.

26. Pick up the incorrect statement from the following :

a. Horizontal D.P.C. is provided at plinth level in internal walls

b. D.P.C. is provided under door and verandah openings

c. Vertical D.P.C. is not provided in internal walls

d. Cement concrete is a rigid damp-proofing material.

27. Pick up the correct proportions of chemical ingredients of cement

a. Lime : Silica : Alumina : Iron oxide : 63 : 22 : 6 : 3

b. Silica : Lime : Alumina : Iron oxide : 63 : 22 : 6 : 3

c. Alumina : Silica : Lime : Iron oxide : 63 : 22 : 6 : 3

d. Iron oxide : Alumina : Silica : Lime : 63 : 22 : 6 : 3

28. Log Angles machine is used to test the aggregate for

a. impact value

b. abrasion resistance

c. water absorption

d. none of these.

29. Run off is the quantity of water which flows

a. in sewer pipes

b. due to leakage in pipes

c. in rivers

d. none of these.

30. The four major water supply distribution systems, are

a. dead end, tree, grid iron and reticulation

b. dead end, tree, grid iron and circular

c. tree, grid iron, ring and radial

d. tree, reticulation, circular and ring.

31. In a residential colony, sewers of diameters 100 mm, 150 mm and 225 mm were laid with a gradient 1 in 120. Which portion of the sewage system does not choke in due course of time

a. 100 mm dia.

b. 150 mm dia.

c. 225 mm dia.

d. all of these.

32. Discrete or granular particles change their

a. size

b. shape

c. weight

d. none of these.

33. For the design of major hydraulic structures on the canals, the method generally preferred to, is based on

a. Bligh's theory

b. Electrical analogy method

c. The relaxation method

d. Khosla's method of independent variables.

34. Which one of the following statements is correct ?

a. The canal system and the drainage system are complementary

b. The canal system runs on ridges, gets divided into smaller channels and spreads its water on the land

c. The drainage system runs along the lowest line, collects drainage water from the slopes and joins the main stream

d. All the above.

35. For a standing crop, the consumptive use of water is equal to the depth of water

a. transpired by the crop

b. evaporated by the crop

c. transpired and evaporated by the crop

d. used by the crop in transpiration, evaporation and also the quantity of water evaporated from adjacent soil.

36. Pick up the incorrect statement from the following. Culturable commanded area is the gross area of an irrigation canal system less

a. populated area

b. alkaline area

c. forest area

d. fallow land.

37. If a Lemniscate curve having an angle of deflection Δ, is transitional throughout, the maximum polar angle is

a. Δ/3

b. Δ/4

c. Δ/5

d. Δ/6

38. Pick up the correct statement from the following:

a. The corss fall of the shoulder should be at least 0.5% steeper than camber

b. On superelevated sections, the shoulders should be provided a cross fall equal to camber

c. Earthern roads in general are provided steepest crossfall

d. All the above.

39. While designing hair pin bends on higways, the minimum

a. gradient is 1 in 40

b. gradient is 1 in 200

c. superelevations is 1 in 10

d. All the above.

40. The number of vehicles moving in a specified direction on a roadway that pass a given point during specified unit of time, is called

a. traffic volume

b. traffic density

c. basic capacity

d. traffic capacity.

41. The expected out turn of half brick partition wall per mason per day is

a. 1.5 m3

b. 2.0 m3

c. 4.0 m2

d. 5.0 m2

42. The slope of the oultet of 'P trap' below the horizontal is kept

a. 8°

b. 10°

c. 12°

d. 14°

43. The area is measured correct to the nearest

a. 0.01 sqm

b. 0.02 sqm

c. 0.03 sqm

d. 0.04 sqm

44. Pick up the incorrect statement from the following:

a. Dimensions are measured to the nearest 0.01 m

b. Areas are measured to the nearest 0.01 sq.m

c. Cubic contents are measured to the nearest 0.1 cm m

d. Weights are measured to the nearest 0.001 tonnes

45. The time by which activity completion time can be delayed without affecting the start of succeeding activities, is known as

a. duration

b. total flat

c. free float

d. interfering float.

46. CPM is

a. is built of activities oriented programme

b. is based on time estimate

c. is used for repetitive works

d. all the above.

47. For the supply of materials for concrete, form work reinforcing and placing of concrete, removal of form work and curing of concrete, number of bar(s) required on bar chart, is

a. 1

b. 2

c. 3

d. 4

48. The most popular type of organisation used for Civil Engineering Constructions, is

a. line organisation

b. line and staff organisation

c. functional organisation

d. effective organisation.

49. According to the International Civil Aviation Organisation (I.C.A.O.) the strength of runway pavements, have been coded by

a. Last Seven English alphabets

b. First Seven English alphabets

c. First seven numbers

d. None of these.

50. International Civil Aviation Organisation (I.C.A.O.) was set up at Montreal (Canada), in

a. 1929

b. 1939

c. 1947

d. 1950