***Sub Engineer***

1. In case of a direct vernier scale

a. graduations increase in opposite direction in which graduations of the main scale increase

b. smallest division is longer than smallest division of the main scale

c. graduations increase in the same direction in which graduations of the main scale increase

d. none of these.

2. Centering error of a theodolite produces an error

a. in all angles equally

b. which does not vary with the direction or pointing

c. which varies with the direction of pointing and inversely with the length of sight

d. none of these.

3. An imaginary line joining the points of equal elevation on the surface of the earth, represents

a. contour gradient

b. contour line

c. level line

d. none of these.

4. Contours of different elevations may cross each other only in the case of

a. an over hanging cliff

b. a vertical cliff

c. a saddle

d. an inclined plane.

5. Stones used for the construction of retaining walls must be

a. soft

b. hard

c. light

d. heavy.

6. A pug mill is used for

a. moulding brick earth

b. tempering brick earth

c. providing brick earth

d. all the above.

7. Bitumen felt

a. is used as water proofing material

b. is used as damp proofing material

c. is made from bitumen and hessian fibres

d. all the above.

8. If water required for 1 bag of cement is 30 litres, the water cement ratio is :

a. 0.40

b. 0.50

c. 0.60

d. None of these.

9. A steel bar 20 mm in diameter simply-supported at its ends over a total span of 40 cm carries a load at its centre. If the maximum stress induced in the bar is limited to N/mm2, the bending strain energy stored in the bar, is

a. 411 N mm

b. 511 N mm

c. 611 Nmm

d. 711 N mm

10. In a shaft, the shear stress is not directly proportional to

a. radius of the shaft

b. angle of twist

c. length of the shaft

d. modulus of rigidity.

11. If the stress produced by a prismatic bar is equal to the working stress, the area of the cross-section of the prismatic bar, becomes

a. zero

b. infinite

c. maximum

d. minimum.

12. The phenomenon of slow extension of materials having constant load, i.e. increasing with the time is called

a. creeping

b. yielding

c. breaking

d. none of these.

13. To measure very low pressure, we use

a. barometers

b. piezometers

c. manometers

d. differential manometers.

14. The width of a weir with end contraction, is

a. equal to the width of the channel

b. less than the width of the channel

c. half the width of the channel

d. none of these.

15. Water flows through a convergent mouthpiece of diameter 4 cm at convergence under a head of 3 metres. If the maximum vacuum pressure is 9 metres of water, the maximum diameter of divergence, to aviod separation of flow, is

a. 4 cm

b. 6 cm

c. 2 cm

d. 23 cm.

16. In a short cylindercial external mouthpiece, the venacontracta occurs at a distance from the outlet of orifice equal to

a. diameter of the orifice

b. one-fourth the diameter of the orifice

c. one-third the diameter of the orifice

d. two-third the diameter of the orifice

17. For slopes of limited extent the surface of slippage, is usually along

a. a parabolic arc

b. an elliptical arc

c. a straight line

d. a circular arc.

18. A moist soil sample weighing 108 g has a volume of 60 cc. If water content is 25% and value of G = 2.52, the void ratio is

a. 0.55

b. 0.65

c. 0.75

d. 0.80

19. The shearing strength of a cohesion-less soil depends upon

a. dry density

b. rate of loading

c. confining pressure

d. nature of loading.

20. The capillary rise of water

a. depends upon the force responsible

b. increases as the size of the soil particles increases

c. decreases as the size of the soil particles decreases

d. is less in wet soil than in dry soil.

21. The steel generally used in R.C.C. work, is

a. stainless

b. mildsteel

c. high carbon steel

d. high tension steel.

22. The toe projection of foundation slabs is taken

a. as one third of the base

b. as one sixth of overall height of the wall

c. equal to heel slab

d. below ground surface.

23. For a circular slab carrying a uniformly distributed load, the ratio of the maximum negative to maximum positive radial moment, is

a. 2

b. 3

c. 4

d. 5

24. A pre-cast pile generally used, is

a. circular

b. square

c. octagonal

d. square with corners chamfered.

25. For constructing a terrazo floor. Pick up the incorrect statement from the following :

a. a 32 mm thick layer of cement concrete (1 : 2 : 4) is laid on the base course and the surface is made smooth by trowelling

b. glass strips are driven into the layer according to the pattern required

c. after final grinding is over, oxalic acid mixed with water is spread over and rubbed hard with soft material

d. none of these.

26. The member which is placed horizontally to support common rafter of a sloping roof, is

a. purlin

b. cleat

c. batten

d. strut.

27. To obtain a very high strength concrete, use very fine grained

a. Granite

b. Magnetite

c. Barite

d. Volcanic scoria.

28. Pick up the correct statement from the following:

a. Insufficient quantity of water makes the concrete unworkable

b. Excess quantity of water makes the concrete segregated

c. Excess quantity of water causes bleeding in concrete

d. All the above.

29. Pickup the incorrect statement from the following :

a. The invert of pressure conduit is independent of the grade of the hydraulic gradient line

b. The pressure conduits may be taken uphill upto a maximum height of 8.3 m

c. Aqueducts and tunnels sections are generally kept circular

d. None of these.

30. If pH value of water is

a. 7 water it is said to be neutral

b. less than 7 it is said to be acidic

c. more than 7 it is said to be alkaline

d. all the above.

31. Sewer ventilating columns are generally placed at

a. distances 150 m to 300 m

b. upper ends of branch sewers

c. every change in the size of sewers

d. all the above.

32. Pick up the correct statement from the following :

a. Thermophilic organisms digest the sludge if the temperature ranges from 40° to 60°C

b. Mesophilic organisms digest the sludge if the temperature is between 25° and 40°C

c. 90% of digestion takes place in 30 days at 29°C, the optimum mesophilic temperature

d. All the above.

33. Lacy's regime condition is obtained if

a. silt grade in the channel is variable

b. discharge in the channel is variable

c. silt charge in the channel is variable

d. channel flows in unlimited, incoherent alluvium of the same character as that transported material.

34. Groynes are generally built

a. perpendicular to the bank

b. inclined up stream up to 30°

c. inclined down stream upto 30°

d. all the above.

35. The field capacity of a soil is 25%, its permanent wilting point is 15% and specific dry unity weight is 1.5. If the depth of root zone of a crop, is 80 cm, the storage capacity of the soil, is

a. 10 cm

b. 12 cm

c. 14 cm

d. 16 cm

36. In rigid module, the discharge

a. is independent of water levels in the distributary and water course

b. depends upon the water level in distributary

c. depends upon the water level in the water course

d. depends upon the water levels of both, i.e. distributary and water course

37. Before providing super-elevation on roads, the portion of the carriageway between the crown and the outer edge is made

a. to have a reduced fall

b. horizontal

c. to have slope of the camber on the other half of the carriageway

d. none of these.

38. Bottom-most component of a flexible pavement, is

a. subgrade

b. sub-base

c. base

d. base course.

39. 1.R.T.D.A. (Indian Roads and Transport Development Association) was set up at Bombay in

a. 1917

b. 1927

c. 1937

d. 1942

40. The convexity provided to the carriageway between the crown and edge of the pavement, is known as

a. super-elevation

b. camber

c. height of the pavement

d. None of these.

41. Due to change in price level, a revised estimate is prepared if the sanctioned estimate exceeds

a. 2.0%

b. 2.5%

c. 4.0%

d. 5.0%

42. Pick up the correct statement from the following:

a. The earth work of cutting in trenches or borrow pits in fairly uniform ground is measured with the help of average depths of the dead men

b. The earth work in trenches or borrow pits in irregular ground is measured by taking the difference in levels before and after completion of work

c. The earth work in trenches or borrow pits, where neither a nor b is feasible, are measured from the fillings after deduction of voids

d. All the above.

43. Pick up the correct statement from the following:

a. The bent up bars at a support resist the negative bending moment

b. The bent up bars at a support resist the sharing force

c. The bending of bars near supports is generally at 45°

d. All the above.

44. The following item of earth work is not measured separately.

a. Site clearence

b. dead men

c. Steps in deep exavaction

d. All the above.

45. Pick up the correct statement from the following:

a. Latest occurrence time is denoted by TL

b. Contractual obligation time is denoted by Ts

c. Latest occurrence time is taken as contractual obligation time

d. All the above.

46. Military organisation is known as

a. line organisation

b. line and staff organisation

c. functional organisation

d. none of these.

47. The difference between the time avail-to do a job and the time required to do the job, is known as

a. event

b. float

c. duration

d. constraint.

48. For completion of a project, the critical path of the network represents

a. minimum time

b. maximum time

c. maximum cost

d. minimum cost.

49. Pick up the correct statement from the following:

a. Landing speed is directly proportional to the wing loading

b. Wing loading remaining constant, the take off distance is directly proportional to the powder loading

c. Neither (a) nor (b)

d. Both (a) and (b)

50. An aircraft is flying in an atmosphere of 30°C with a speed of 1260 km ph. Its speed is known as

a. subsonic

b. sonic

c. super-sonic

d. Mach.