***Sub Engineer***

1. Pick up the correct statement from the following :

a. the lines of sight while observing back sight and fore  sight lie in the same horizontal plane

b. the   staff readings are measurements made vertically downwards from a horizontal plane

c. the horizontal plane with reference to which staff readings are taken, coincides with the level surface through the telescope axis

d. all the above.

2. Measuring with a 30 m chain, 0.01 m too short, introduces

a. positive compensating error

b. negative compensating error

c. positive cumulative error

d. negative cumulative error.

3. A clinometer is used for

a. measuring angle of slope

b. correcting line of collimation

c. setting out right angles

d. defining natural features.

4. Magnetic bearing of a survey line at any place

a. remains constant

b. changes systematically

c. varies differently in different months of the year

d. is always greater than true bearing.

5. Acrylic sheets

a. are generally unaffected by most household detergents

b. possess the light transmission rate of 93%

c. are available in various shapes

d. all the above.

6. Asbestos

a. is a natural fibrous mineral substance

b. is composed of hydrous silicates of calcium and magnesium (CaSiO3, 3MgSiO3)

c. contains iron oxide and alumina

d. all the above.

7. Pick up the correct statement from the following:

a. Quick lime is obtained by burning pure lime stone:

b. Hydraulic lime is obtained by burning lime stone containing clay 5% to 30%

c. Poor lime is obtained by burning lime stone containing impurities more than 5%

d. All the above.

8. Pick up the correct statement from the following:

a. Formaldehyde is prepared synthetically from methane

b. The melamine when reacted with formaldehyde forms the melamine-formaldehyde resin

c. The plastics made from melamine formaldehyde resin, are used for electrical insulators

d. All the above.

9. The forces acting normally on the cross section of a bar shown in the given figure introduce

a. compressive stress

b. tensile stress

c. shear stress

d. none of these.

10. The eccentricity (e) of a hollow circular column, external diameter 25 cm, internal diameter 15 cm for an eccentric load 100 t for non-development of tension, is

a. 3.00 cm

b. 3.50 cm

c. 4.25 cm

d. 5.0 cm

11. A rectangular log of wood is floating in water with a load of 100 N at its centre. The maximum shear force in the wooden log is

a. 50 N at each end

b. 50 N at the centre

c. 100 N at the centre

d. none of these.

12. A shaft 9 m long is subjected to a torque 30 t-m at a point 3 m distant from either end. The reactive torque at the nearer end will be

a. 10 tonnes metre

b. 15 tonnes metre

c. 20 tonnes metre

d. none of these.

13. Pick up the correct statement from the following :

a. In incompressible flow the density of a fluid remains constant

b. In compressible flow, the density of a fluid changes from point to point

c. In uniform flow, the velocity of a fluid does not change with respect to length of flow direction

d. All the above.

14. The instrument used for measuring the velocity of flow, is known as

a. venturimeter

b. orifice meter

c. pitot tube

d. none of these.

15. An orifice is taken as large if

a. (H2 - H1) > H

b. (H2 - H1) > H/4

c. (H2 - H1) > H/2

d. (H2 - H1) > H/3

16. The horizontal component of the force on a curved surface is equal to

a. weight of liquid vertically below the curved surface

b. force on a vetical projection of the curved surface

c. product of pressure at its centroid and the area

d. weight of liquid retained by the curved area.

17. Pick up the in-correct statement from the following: The soils which contain montmorillonite minerals

a. shrink more when dry

b. possess high plasticity

c. possess high coefficient of internal coefficient

d. None of these.

18. For determing the specific gravity of soil solids, using a pycnometer of 500 cc., the following data is available : Weight of dry empty pycnometer = 125 g Weight of dried soil and pycnometer = 500 g Weight of dried soil and distilled = 850 g water filled in pycnometer up to top The specific gravity of soil soilds, is

a. 2.0

b. 2.25

c. 2.50

d. 2.75

19. The phreatic line in an earth dam may be

a. elliptical

b. parabolic

c. a straight line

d. all the above.

20. The angle of internal friction, is least for

a. angular -grained dense sand

b. round-grained loose sand

c. round-grained loose sand

d. clays.

21. For a ribbed slab

a. clear spacing between ribs shall not be greater than 4.5 cm

b. width of the rib shall not be less than 7.5 cm

c. overall depth of the slab shall not exceed four times the breadth of the rib

d. all the above.

22. According to the steel beam theory of doubly reinforced beams

a. compression is resisted by compression steel

b. stress in tension steel equals the stress in compression steel

c. no stress is developed in compression concrete as well as in tension concrete

d. all the above.

23. If the tendon is placed at an eccentricity e below the centroidal axis of the lon-gitudial axis of a rectangular beam (sectional modulus Z and stressed load P in tendon) the stress at the extreme top edge

a. is increased by

b. is increased by

c. is decreased by

d. remains unchanged.

24. A pre-stressed concrete member is preferred because

a. its dimensions are not decided from the diagonal tensile stress

b. large size of long beams carrying large shear force need not be adopted

c. removal of cracks in the members due to shrinkage

d. all the above.

25. The inner section of a cavity wall, is generally known as

a. butress

b. leaf wall

c. pillaster

d. pillar.

26. Pick up the incorrect statement from the following :

a. No  timbering is  required  for  shallow trenches

b. Shallow foundations can be constructed on made-up soil

c. Grillage   foundation   is   classified as a shallow foundation

d. Black cotton soil is very good for foundation bed.

27. Sand requiring a high water cement ratio, belongs to

a. Zone I

b. Zone II

c. Zone III

d. Zone IV.

28. If P, Y and Z are the weights of cement, fine aggregates and coarse aggregates respetively and W/C is the water cement ratio, the minimum quantity of water to be added to first batch, is obtained by the equation

a. 0.3P + 0.1Y + 0.01Z = W/C x P

b. 0.4P + 0.2Y + 0.01Z = W/C x P

c. 0.5P + 0.3Y + 0.01Z = W/C x P

d. 0.2P + 0.5Y + 0.1Z = W/C x P

29. Steel pipe are

a. connected by riveted or welded joints

b. generally laid under ground and no expansion joint is required

c. likely to last 100 years under ordinary conditions

d. None of these.

30. The coefficient of permeability of soils, is generally expressed in

a. cm/sec

b. cm/minute

c. cm2/sec

d. cm2/minute.

31. If the peak discharge of a storm water drain (S.W. Drain) is e.xpected to exceed 150 cumecs, the free board to be provided, is

a. 100 cm

b. 90 cm

c. 80 cm

d. 50 cm.

32. A drop manhole is provided if

a. a sewer drops from a height

b. a branch sewer joins the main sewer at higher level

c. a lamp is inserted to check obstruction

d. none of these.

33. In gravity canals, F.S.L. is

a. always at the ground level

b. always below the ground level

c. generally 4 to 5 metres above the ground level

d. only a few cm above the ground level.

34. A water-logged land is found suitable for cultivation due to

a. ease of tillage for preparation of the field for optimum condition of germination

b. absence of aeration of soil from root zone of the plant

c. regular supply of water to plants from the water table by capillary action

d. none of these.

35. The length and width of a meander and also the width of the river, vary roughly as

a. square root of the discharge

b. discharge

c. square of the discharge

d. cube of the discharge.

36. Pick up the correct sequence of the part of a canal system from the following

a. Head work-distributary-branch canal-minor

b. Head works-main canal-branch canal-dis-tributary-minor

c. Head works-main canal-branch canal-minor-distributary

d. Heads works-branch canal-main canal distributary, minor.

37. Width of a rotary round should be equal to

a. twice the width of narrowest radial road

b. width of the widest road

c. width of the widest road plus the width of one lane

d. width of the widest road plus 2 metres.

38. Design of flexible pavements is based on

a. mathematical analysis

b. empirical formulae

c. a compromise of pure theory and pure empirical formula

d. none of these.

39. Customers prefer parking of their vehicles at

a. 85° to aisles

b. 80° to aisles

c. 75° to aisles

d. 60° to aisles

40. Pick up the incorrect statement from the following:

a. Highways are always constructed in straight line

b. Highways may be provided horizontal curves

c. Highways may be provided vertical curves

d. Highways may be provided both horizontal and vertical curves.

41. The item of the brick structure measured in sq.m, is

a. Reinforced brick work

b. Broken glass coping

c. Brick edging

d. Brick work in arches.

42. The diameter of a domestic sewer pipe laid at gradient 1 in 100 is recommended

a. 100 mm

b. 150 mm

c. 200 mm

d. 175 mm

43. The cross-sections for a highway is taken at

a. 30 metres apart

b. intermediate points having abrupt change in gradient

c. the starting end points of the curves

d. All the above.

44. Due to change in price level, a revised estimate is prepared if the sanctioned estimate exceeds

a. 2.0%

b. 2.5%

c. 4.0%

d. 5.0%

45. While scheduling a project by C.P.M.

a. required time for each activity is established

b. sequence of various activities is made according to their importance

c. net work is drawn by connecting the activities and the events

d. All the above.

46. The most popular type of organisation used for Civil Engineering Constructions, is

a. line organisation

b. line and staff organisation

c. functional organisation

d. effective organisation.

47. A construction schedule is prepared after collecting

a. output of labour

b. output of machinery

c. quantity of various items

d. all the above.

48. The final selection of a construction site, is done by

a. local civil authority representative

b. representative of engineer authority

c. representative of administration

d. all the above.

49. Pick up the incorrect statement from the following:

a. Channelization of pavement is caused by constant use of tri-cycle gears of aircraft

b. Blast pads are used over cohesionless soils to resist erosion due to tremendous speed of the jet blast

c. Over-run areas of at least 300 m length on either side of the runway are provided

d. None of these.

50. Speed of

a. sound varies 2.4 km p.h. per degree centrigrade rise in temperature

b. sound at 0°C is called one Mach

c. an aircraft flying at 2975 km p.h., is called 2.5 Mach

d. All the above.