***Sub Engineer***

1. Flint glass

a. has slightly the greater refracting power than crown glass

b. has roughly double refracting power than that of crown

c. and crown glass proportions yield the required focal length and neutralise the dispersion produced by the convex lens at the emergence from the concave

d. all the above.

2. Pick up the method of surveying in which field observations and plotting proceed simultaneously from the following

a. chain surveying

b. compass surveying

c. plan table surveying

d. tacheometric surveying.

3. Two contour lines, having the same elevation

a. cannot cross each other

b. can cross each other

c. cannot unite together

d. can unite together.

4. Accurate measurement of deflection angles with a transit not properly adjusted may be made by

a. setting the vernier A at zero at back station and then plunging the telescope

b. setting the vernier A at zero at back station and turning the instrument to the forward station

c. taking two back sights one with the telescope normal and the other with telescope inverted

d. none of these.

5. In order of increasing percentage of silica, the correct sequence is

a. sandy clay, calcareous clay, pure clay

b. calcareous clay, pure clay, sandy clay

c. pure clay, sandy clay, calcareous clay

d. None of these.

6. Mastic asphalt is normally used for

a. sound insulation

b. water proofing

c. fire proofing

d. none to these.

7. Due to attack of dry rot, the timber

a. cracks

b. shrinks

c. reduces to powder

d. none of these.

8. Pick up the correct statement from the following:

a. The percentage of silica in silica bricks is to the extent of about 95 to 97 percent

b. Roughly 1 to 2 percent of lime in silica bricks is added to act as binding material

c. The compressive strength of silica bricks is about 150 kg/cm2

d. All the above.

9. In plastic analysis, the shape factor for a circular section, is

a. 1.5

b. 1.6

c. 1.7

d. 1.75

10. The ratio of the area of cross-section of a circular section to the area of its core, is

a. 4

b. 8

c. 12

d. 16

11. If the stress produced by a prismatic bar is equal to the working stress, the area of the cross-section of the prismatic bar, becomes

a. zero

b. infinite

c. maximum

d. minimum.

12. At either end of a plane frame, maximum number of possible transverse shear forces, are

a. one

b. two

c. three

d. four

13. Atmospheric pressure is equal to water column head of

a. 9.81 m

b. 5.0 m

c. 10.30 m

d. 7.5 m.

14. Equation of continuity of flow is based on the principle of conservation of

a. mass

b. momentum

c. force

d. none of these.

15. An error of 1% in measuring the head of water over the crest of a rectangular weir, produces an error in the discharge which is equal to

a. 1.25%

b. 1.5%

c. 1.75%

d. 2.25%

16. The discharge through an internal mouth piece is more if its length is

a. <  diameter

b. < diameter

c. ≥ diameter

d. none of these.

17. Pick up the correct statement from the following:

a. The porosity of a soil is defined as the ratio of the volume of voids to the gross volume of the soil

b. The bulk density of a soil is defined as the unit weight of the soil

c. The dry density of a soil is defined as weight of solids to the total volume of the soil

d. All the above.

18. Hydrometer readings are corrected for:

a. meniscus correction

b. dispersing agent correction

c. meniscus and dispersing agent corrections

d. temperature, meniscus and dispersing agent corrections.

19. A soil sample has passing 0.075 mm sieve = 60% liquid limit = 65% and plastic limit = 40%. The group index of the soil, is

a. 5

b. 20

c. 40

d. none of these.

20. Which one of the following statements is true for Mohr-Coulomb envelope ?

a. The generalised Mohr theory suggests that, though the shear stress depends on the normal stress, the relation is not linear

b. Coulomb and Mohr suggest that a definite relationship exists among the principal stress and the angle of internal friction

c. For an ideal pure friction material, the straight line passes through the origin.

d. All the above.

21. The transverse reinforcements provided at right angles to the main reinforcement

a. distribute the load

b. resist the temperature stresses

c. resist the shrinkage stress

d. all the above.

22. The maximum ratio of span to depth of a slab simply supported and spanning in two directions, is

a. 30

b. 35

c. 40

d. 15

23. If the diameter of longitudinal bars of a square column is 16 mm, the diameter of lateral ties should not be less than

a. 5 mm

b. 6 mm

c. 8 mm

d. 10 mm

24. A reinforced concrete cantilever beam is 3.6 m long, 25 cm wide and has its lever arm 40 cm. It carries a load of 1200 kg at its free end and vertical stirrups can carry 1800 kg. Assuming concrete to carry one-third of the diagonal tension and ignoring the weight of the beam, the number of shear stirrups required, is

a. 35

b. 40

c. 45

d. 50

25. The type of pointing in which upper side of mortar joints is kept about 12 mm inside the face of the masonry and bottom is kept flushed with face of wall, is

a. truck pointing

b. recessed pointing

c. struck pointing

d. grooved pointing.

26. The columns of multi-storeyed buildings are designed to withstand the forces due to

a. live loads

b. wind loads

c. earthquakes

d. all of these.

27. Proper proportioning of concrete, ensures

a. desired durability

b. water tightness of the structure

c. resistance to water

d. all the above.

28. An aggregate is said to be flaky, if its least dimension is less than

a. 3/4 mean dimension

b. 3/5 mean dimension

c. 5/8 mean dimension

d. none of these.

29. As per IS : 1172-1963, water required per head per day for average domestic purposes, is

a. 65 litres

b. 85 litres

c. 105 litres

d. 135 litres.

30. The water level in an open well was depressed by pumping 2.5 m and recuperated 2.87 m in 3 hours and 50 minutes. The yield of the well per minute is

a. 0.0033

b. 0.0044

c. 0.0055

d. 0.0066

31. Antisyphonage pipe is fitted

a. at the end of septic tanks

b. on manholes

c. with a W.C. trap

d. at the beginning of sewer line.

32. In case of sewer lines

a. water test is carried out to check water tightness of the joints

b. test for straightness is carried out with the help of a lamp and mirror

c. obstruction test is carried out with the. help of smooth ball of diameter 13 mm

d. all the above.

33. Pick up the correct sequence of the part of a canal system from the following

a. Head work-distributary-branch canal-minor

b. Head works-main canal-branch canal-dis-tributary-minor

c. Head works-main canal-branch canal-minor-distributary

d. Heads works-branch canal-main canal distributary, minor.

34. A hydraulic jump is generally formed when a stream moving with

a. a hyper-critical velocity meets a stream moving with a critical velocity

b. a hyper-critical velocity meets a stream moving with a hyper-critical velocity

c. A hyper-critical velocity meets a stream moving with a sub-critical velocity

d. a sub-critical velocity meets a stream moving with a hyper-critical velocity.

35. Solution of Laplacian equation in three dimensions  of water in a syphon, is done by

a. analytical method

b. Khosla's method

c. method of relaxation

d. Unwin's method.

36. To hold hydraulic jumps, baffle walls are provided in

a. Sarda type falls

b. English type falls

c. Montague type falls

d. Vertical type falls.

37. Before providing super-elevation on roads, the portion of the carriageway between the crown and the outer edge is made

a. to have a reduced fall

b. horizontal

c. to have slope of the camber on the other half of the carriageway

d. none of these.

38. At a road junction, 16 cross conflict points are severe, if

a. both are one-way roads

b. both are two-way roads

c. one is two-way road and other is one-way road

d. none of these.

39. Minimum stopping distance for moving vehicles on road with a design speed of 80 km/hour, is

a. 100 m

b. 120 m

c. 150 m

d. 200 m

40. Widening of the roads on curves in hilly region, is done

a. on the outer side

b. on the inner side

c. on the outer and inner sides equally

d. less on outer side and more on inner side.

41. For 12 mm thick cement plastering 1 : 6 on 100 sq.m new brick work, the quantity of cement required, is

a. 0.200 m3

b. 0.247 m3

c. 0.274 m3

d. 0.295 m3

42. The measurement is made for stone work in square metre in case of

a. Columns, lintels, copings

b. Building work

c. Dressed stones in Chajjas

d. (a) and (d) of the above.

43. The ground surface slopes 1 in 50 along a proposed railway embankment 150 m in length. The height of the embankment at zero chainage is 0.5 m, the width is 11 m and side slopes 2:1. If the falling gradient of the embankment is 1 in 150, the quantity of the earthwork calculated by prismoidal formula, is

a. 3250 m3

b. 3225 m3

c. 3275 m3

d. 3300 m3

44. For the construction of buildings, the subheads of the estimate are

a. Brickwork, Stone work, Roofing

b. Brickwork Flooring, Wood work, Steel work

c. Plastering or pointing, finishing, water supply and sanitary work

d. All the above.

45. The critical activity has

a. maximum float

b. minimum float

c. zero float

d. none of these.

46. Pick up the correct statement from the following with regards to C.P.M. network analysis of projects

a. Earliest occurrence time of the event from which the activity arrow originates plus the duration of the activity, is called earliest finish time of the activity

b. The latest occurrence time of the node of which the activity arrow terminates minus the duration of the activity, is called latest start time

c. The latest occurrence time for the node at which the activity arrow terminates, is called latest finish time

d. All the above.

47. If the total float and duration of an activity are 5 and 10 days respectively, the particular activity can be

a. started 5 days later

b. completed 5 days later

c. performed at slower rate in 15 days

d. all the above.

48. For the execution of a project, a contractor is

a. a person

b. a firm

c. an agency

d. all the above.

49. Effective length of a runway is the distance between

a. ends of the runway

b. point of intersection of the obstruction clearance line and the extended plane of the runway surface, and the other end of the runway

c. point of intersection of the glide path and the extended plane of the runway surface and the other end of the runway

d. ends of the clear way on either side.

50. The runway orientation is made so that landing and take off are

a. against the wind direction

b. along the wind direction

c. perpendicular to wind direction

d. none of these.