***Sub Engineer***

1. Greater accuracy in linear measurements, is obtained by

a. tacheometry

b. direct chaining

c. direct taping

d. all the above.

2. The method generally preferred to for contouring an undulating area, is

a. chain surveying

b. plane table surveying

c. tacheometrical surveying

d. compass surveying.

3. The latitude of a traverse leg is obtained by multiplying its length by

a. tangent of its reduced bearing

b. sign of its reduced bearing

c. cosine of its reduced bearing

d. cosecant of its reduced bearing.

4. Pick up the correct statement from the following :

a. The vertical distance between a maximum ordinate and the next forward maximum ordinate represents the whole volume of the embankment

b. The vertical distance between a minimum ordinate and the next forward maximum ordinate represents the whole volume of a cutting

c. The area enclosed by a loop of the curve and balancing line, measures the haul in that direction.

d. all the above.

5. Plywood is normally available

a. 1 mm thick

b. 2 mm thick

c. 2 to 3 mm thick

d. 3 mm to 4 mm thick.

6. For lime concrete,

a. flexural strength at 90 days is 0.2 N/mm2

b. compressive strength at 90 days is 1.5 N/mm2

c. compressive strength at 26 days is 1.2 N/mm2

d. all the above.

7. Pick up the correct statement from the following:

a. The percentage of silica in silica bricks is to the extent of about 95 to 97 percent

b. Roughly 1 to 2 percent of lime in silica bricks is added to act as binding material

c. The compressive strength of silica bricks is about 150 kg/cm2

d. All the above.

8. In the cement the compound quickest to react with water, is

a. Tricalcium aluminate

b. Tetra-calcium alumino-ferrite

c. Tricalcium silicate

d. Dicalcium silicate.

9. Shear strain energy theory for the failure of a material at elastic limit, is due to

a. Guest or Trecas

b. St. Venant

c. Haig

d. Von Mises.

10. In a shaft, the shear stress is not directly proportional to

a. radius of the shaft

b. angle of twist

c. length of the shaft

d. modulus of rigidity.

11. For the beam shown in below figure, the maximum positive bending moment is nearly equal to negative bending moment when L1 is equal to

a. 1.0 L

b. 0.7 L

c. 0.5 L

d. 0.35 L.

12. A three-hinged arch is said to be :

a. statically determinate structure

b. statically indeterminate structure

c. a bent beam

d. none of these.

13. The metacentric height of a body equals the distance between

a. the centre of gravity and centre of buoyancy

b. the metacentre and centre of gravity

c. the centre of buoyancy and metacentre

d. none of these.

14. Water displaced by a floating wooden block of density 0.75, 5 m long, 2 m wide and 3 m high, is

a. 17.5 m3

b. 20.0 m3

c. 22.5 m3

d. 25 km3.

15. For solving network problems of pipes, necessary condition is

a. continuity equation

b. energy equation

c. Darcy-Weisbach equation

d. all the above.

16. In an open tube, free surface of mercury remains

a. horizontal

b. curved upwards

c. curved downwards

d. none of these.

17. The ratio of the difference between the void ratio of the soil in its loosest state and its natural void ratio (e) to the difference between the void ratios in the loosest and fully dense state, is generally termed as

a. degree of density

b. relativity

c. density index

d. all the above.

18. The weight of a pycnometer containing 400 g sand and water full to the top is 2150 g. The weight of pycnometer full of clean water is 1950 g. If specific gravity of the soil is 2.5, the water content is

a. 5%

b. 10%

c. 15%

d. 20%

19. The factor which affects the compaction, is

a. compacting content

b. method of compaction

c. type of soil

d. All the above.

20. For a homogeneous earth dam 50 m high having 2 m free broad, a flow net was constructed and the results were : Number of potential drops = 2.4 Number of flow channels = 0.4. If coefficiency of permeability of the dam mateiral is 3 x 10-3 cm3/sec, the discharge per metre length of dam, is

a. 12 x 10-5 m3/sec

b. 24 x 10-3 m3/sec

c. 6 x 10-5 m3/sec

d. 24 x 10-5 m3/sec

21. An R.C.C. beam of 25 cm width and 50 cm effective depth has a clear span of 6 metres and carries a U.D.L. of 3000 kg/m inclusive of its self weight. If the lever arm constant for the section is 0.865, the maximum intensity of shear stress, is

a. 8.3 kg/cm2

b. 7.6 kg/cm2

c. 21.5 kg/cm2

d. 11.4 kg/cm2

22. Minimum spacing between horizontal parallel reinforcement of different sizes, should not be less than

a. one diameter of thicker bar

b. sum of the diameters of ihinner and thicker bars

c. twice the diameter of thinner bar

d. none of these.

23. The toe projection of foundation slabs is taken

a. as one third of the base

b. as one sixth of overall height of the wall

c. equal to heel slab

d. below ground surface.

24. Top bars are extended to the projecting parts of the combined footing of two columns L distance apart for a distance of

a. 0.1 L from the outer edge of column

b. 0.1 L from the centre edge of column

c. half the distance of projection

d. one-fourth the distance of projection.

25. The formWork including the props can be removed from beams, only after

a. 3 day

b. 7 days

c. 14 days

d. 21 days.

26. For effective drainage, the finished surface of flat roof should have a minimum slope of

a. 1 in 20

b. 1 to 50

c. 1 in 10

d. 1 in 5.

27. For an ordinary Portland cement

a. soundness varies from 5 to 10 mm

b. initial setting time is not less than 30 minutes

c. compressive stress after 7 days, is not less than 175 kg/cm2

d. all the above.

28. Pick up the correct statement from the following:

a. The quantity of water required for concreting, depends upon the grading of aggregate amd method of compaction

b. 10% excess of water reduces the strength of concrete by 15%

c. 30% excess of water reduces the strength of concrete by 50%

d. All the above.

29. Suction lift of a pump depends upon

a. water temperature

b. velocity of water in suction pipe

c. frictional losses

d. all the above.

30. In a well planned city, the layout of distribution pipes generally adopted, is

a. grid-iron system

b. interlaced system

c. reticulation system

d. all the above.

31. A manhole is classified as shallow if its depth is between

a. 0.5 to 0.7 m

b. 0.7 to 0.9 m

c. 0.9 to 1.20 m

d. 1.2 to 1.50 m.

32. For design purposes, the normal rate of infiltration of ground water into the sewer, is

a. 1000 litres/km/cm

b. 1500 litres/km/cm

c. 2000 litres/km/cm

d. 2500 litres/km/cm.

33. Borrow pits should preferably be located in

a. field on the left side of the canal

b. field on the right side of the canal

c. fields on both sides of the canal

d. central half width of the section of the canal.

34. The velocity of drainage water in the barrels of a syphon-aqueduct, is normally limited to

a. 1 to 2 m per second

b. 2 to 3 m per second

c. 3 to 4 m per second

d. 4 to 5 m per second.

35. In gravity canals, F.S.L. is

a. always at the ground level

b. always below the ground level

c. generally 4 to 5 metres above the ground level

d. only a few cm above the ground level.

36. For a unique design of a channel by Kennedy's theory

a. its breadth must only be known

b. its depth must only be known

c. its breadth and depth ratio must only be known

d. all the above.

37. Longitudinal pavement lines marked broken in white paint

a. are for the guidance of the drivers

b. are not to be crossed over

c. may be crossed over at the discretion of the driver

d. all the above.

38. Excessive camber on pavements may cause

a. deterioration of central portion

b. slip of the speedy vehicles towards the edges

c. erosion of the berms

d. all the above.

39. When a number of hair pin bends are introduced, a minimum intervening distance in between is kept

a. 20 m

b. 40 m

c. 60 m

d. 100 m

40. For night travel, the length of a valley curve should be such that the head light beam distance is the same as

a. stopping sight distance

b. overtaking sight distance

c. sum of (a) and (b)

d. difference of (a) and (b)

41. The main factor to be considered while preparing a detailed estimate, is

a. Availability of materials

b. Transportation of materials

c. Location of site and local labour charges

d. All the above.

42. Pick up the correct statement from the following:

a. In a gully trap, a water seal of 6 to 7.5 cm is provided

b. The gully trap collects waste water from the kitchen, sink, wash basins, etc.

c. The gully trap disconnects the sullage drain from the main drainage system

d. The grating provided over gully traps is 23 cm square.

43. Pick up the correct statement regarding the centre line method of estimating a building

a. Product of the centre line of the walls and area of cross-section of any item, gives total quantity of the item

b. The centre line is worked out separately for different sections of walls of a building

c. The centre line length is reduced by half the layer of main wall joining the partition wall

d. All the above.

44. The area of a sloping surface of a protective embankment of mean height d, side slopes S : 1 and length L is

a. d x d x s

b. d2 x (ds)2

c. L.D 1 + s2

d. 2 LD 1 + s2

45. Pick up the PERT event from the following:

a. Digging of foundation completed

b. Laying of concrete started

c. Laying of concrete completed

d. All the above.

46. Power stations are generally treated as

a. light construction

b. heavy construction

c. industrial construction

d. electrical construction.

47. Pick up the incorrect statement from the following:

a. The beginning and end of a job, are called events

b. The activity which consumes maximum time, is called a node

c. Logically and sequentially connected activities and events form a network

d. None of these.

48. The object of technical planning, is

a. preparation of estimates

b. initiating the procurement action of resources

c. taking remedial action for likely bottleneck in the execution

d. all the above.

49. From the end of an instrumental runway, the approach surface rises outwards

a. 1 in 20

b. 1 in 30

c. 1 in 40

d. 1 in 50

50. Pick up the correct statement from the following:

a. Landing speed is directly proportional to the wing loading

b. Wing loading remaining constant, the take off distance is directly proportional to the powder loading

c. Neither (a) nor (b)

d. Both (a) and (b)