***Sub Engineer***

1. In case of a double line river, contours are

a. stopped at the banks of the river

b. stopped at the edge of the river

c. drawn across the water

d. drawn by parabolic curves having their vertex at the centre of the water.

2. The horizontal angle between true meridian and magnetic meridian, is known

a. bearing

b. magnetic declination

c. dip

d. convergence.

3. Determining the difference in elevation between two points on the surface of the earth, is known as

a. levelling

b. simple levelling

c. differential levelling

d. longitudinal levelling.

4. The combined effect of curvature and refraction over a distance L kilometres is

a. 67.2 L2 mm

b. 76.3 L2 mm

c. 64.5 L2 mm

d. none of these.

5. Stones used for the construction of retaining walls must be

a. soft

b. hard

c. light

d. heavy.

6. For the manufacture of stainless steel, steel is mixed with

a. chromium

b. nickel

c. tungsten

d. none of these.

7. A prime coat is given to steel work with

a. an oxide of iron paint

b. a mixture of white lead and lead paint

c. a special paint

d. cement paint.

8. The normal curing period for lime mortar, is:

a. 3 days

b. 7 days

c. 10 days

d. 14 days.

9. The tangential component of stress on an plane inclined θ° to the direction of the force, may be obtained by multiplying the normal stress by

a. cos θ

b. tan θ

c. sin θ cos θ

d. sin2θ

10. A material is said to be perfectly elastic if

a. it regains its original shape on removal of the load

b. It regains its original shape partially on removal of the load

c. it does not regain its original shape at all

d. none of these.

11. The slenderness ratio of a vertical column of a square cross-section of 2.5 cm sides and 300 cm length, is

a. 240

b. 360

c. 416

d. 500

12. When a rectangular beam is loaded longitudinally, shear develops on

a. bottom fibre

b. top fibre

c. middle fibre

d. every-horizontal plane.

13. For critical depth of flow of water in open channels, fc the specific energy must be :

a. minimum

b. maximum

c. average of maximum and minimum

d. None of these.

14. For the flow of liquid from an open ended tube (or nozzle) leading to the formation of spray of liquid drops, the number generally applied, is

a. Froude number

b. Weber number

c. Reynold number

d. Mach number.

15. 'Flow net' can be drawn only if the flow is

a. turbulent

b. rotational

c. distortion

d. none of these.

16. For the two dimensional flow, the stream function is given by ψ = 2xy. The velocity at a point (3, 4) is

a. 8 m/sec

b. 10 m/sec

c. 12 m/sec

d. 15 m/sec.

17. The angle between the directions of the failure and the major principal plane, is equal to

a. 90° + effective angle of shearing resistance

b. 90° + half of the angle of shearing resistance

c. 45° - half of the angle of shearing resistance

d. 45° + half of the angle of shearing resistance.

18. Pick up the correct statement from the following:

a. Kaolinite shows a very little sign of swelling on wetting

b. Kaolinite when wet, becomes moderately plastic because negative electro magnetic charges on platelets attrack water

c. Kaolinite is also called China clay

d. All the above.

19. Darcy's law is applicable to seepage if a soil is

a. homogeneous

b. isotropic

c. incompressible

d. all the above.

20. The factor which affects the compaction, is

a. compacting content

b. method of compaction

c. type of soil

d. All the above.

21. The length of the lap in a compression member is kept greater than bar diameter x (Permissible stress in bar / Five times the bond stress) or

a. 18 bar diameters

b. 24 bar diameters

c. 30 bar diameters

d. 36 bar diameters

22. Top bars are extended to the projecting parts of the combined footing of two columns L distance apart for a distance of

a. 0.1 L from the outer edge of column

b. 0.1 L from the centre edge of column

c. half the distance of projection

d. one-fourth the distance of projection.

23. In a singly reinforced beam, the effective depth is measured from its compression edge to

a. tensile edge

b. tensile reinforcement

c. neutral axis of the beam

d. longitudinal central axis.

24. The allowable tensile stress in mild steel stirrups, reinforced cement concrete, is

a. 1400 kg/cm2

b. 190 kg/cm2

c. 260 kg/cm2

d. 230 kg/cm2

25. A projecting piece usually provided to support a truss, is

a. cornice

b. coping

c. frieze

d. lintal.

26. A stair should not have pitch more than

a. 30°

b. 40°

c. 50°

d. 60°.

27. According to IS : 382-1963, a good aggregate should be

a. chemically inert

b. sufficiently strong

c. hard and durable

d. all the above.

28. Water required per bag of cement, is

a. 14 kg

b. 21 kg

c. 28 kg

d. 35 kg

29. In a rapid sand filter, air binding is caused due to excessive

a. negative pressure

b. pressure

c. turbidity

d. water pressure

30. The pH value of water is kept slightly less than 7 so that hydrochloride ions may combine with ammonia ions to form

a. Mono-chloramine (NHCl)

b. Di-chloramine (NH2Cl)

c. Nitrogen trichloramine (NCl3)

d. All the above.

31. The most effective arrangement for diverting excess storm water into a natural drainage, is

a. leaping weir

b. overflow weir

c. siphon spill way

d. none of these.

32. In a fully mechanised composting plant, involves

a. mechanized receipt

b. mechanized segregation

c. mechanized pulverising of refuse

d. all of these.

33. The difference in level between the top of a bank and supply level in a canal, is called

a. berm

b. free board

c. height of bank

d. none of these.

34. Pick up the incorrect statement from the following:

a. In free flooding irrigation, water is admitted at one corner of a field and is allowed to spread over the entire area

b. In check method of irrigation, the field is divided into smaller compartments and water is admitted to each in turn

c. In furrow irrigation water is admitted between the rows of plants in the field

d. None of these.

35. A fall which maintains its depth, is

a. a trapezoidal notch fall

b. a low weir fall

c. a rectangular notch fall

d. all the above.

36. For the conditions enumerated to provide a crossing at C1 You will probably provide

a. an aqueduct

b. a super-passage

c. a syphon aqueduct

d. none of these.

37. Setting out of Lemniscate transition curves, is done with

a. perpendicular offsets

b. radial offsets

c. deflection angles

d. polar deflection angles.

38. The total length of a valley formed by two gradients - 3% and + 2% curve between the two tangent points to provide a rate of change of centrifugal acceleration 0.6 m/sec2, for a design speed 100 km ph, is

a. 16.0 m

b. 42.3 m

c. 84.6 m

d. none of these.

39. If the cross slope of a country is 25% to 60%, the terrain is classified as

a. plain

b. rolling

c. steep

d. mountainous.

40. Full amount of extra width of a pavement on the curve, is provided at

a. beginning of the transition curve

b. centre of the transition curve

c. beginning of the circular curve

d. centre of the circular curve.

41. The concrete work for the following part of the building of specified thickness is measured in square metres

a. Floors

b. D.P.C.

c. Wall panels

d. All the above.

42. In long and short wall method of estimation, the length of long wall is the centre to centre distance between the walls and

a. breadth of the wall

b. half breadth of wall on each side

c. one fourth breadth of wall on each side

d. None of these.

43. Pick up the correct statement from the following:

a. The actual expenditure involved to complete a work including incidental, establishment and travelling charges, is called actual cost

b. The formal acceptance by the administrative department for incurring an expenditure on the work, is called administrative approval

c. The order of a competent authority sanctioning a properly detailed estimate of the cost of a work of construction or repair is called technical sanction

d. All the above.

44. The volume is measured correct to the nearest

a. 0.01 cum

b. 0.02 cum

c. 0.03 cum

d. 0.04 cum

45. Works costing less than Rs. 20,000 are treated as

a. projects

b. major projects

c. minor projects

d. all the above.

46. Optimistic time, most likely time and pessimistic times for the activities of a network in the given figure are written above their arrows. If the contractual obligation time for the project is 75, the latest occurrence time for the event 2, is

a. 20

b. 25

c. 35

d. 15

47. Pick up the correct statement from the following:

a. Backward pass is used for calculating the latest occurence time

b. Maximum value of earliest expected time is used if there are more than one value of any event

c. Minimum value of latest occurrence time is used if there are more than one value of any event

d. All the above.

48. Henry Gantt developed Bar charts for planning and scheduling of projects in

a. 1900

b. 1920

c. 1940

d. 1950

49. The fuse large includes

a. passengers chamber

b. pilot's cabin

c. tail of aircraft

d. all the above.

50. The best direction of a runway is along the direction of

a. longest line on wind rose diagram

b. shortest line on the wind rose diagram

c. line clear of wind rose diagram

d. none of these.