***Sub Engineer***

1. The distance between steps for measuring down hill to obtain better accuracy

a. decreases with decrease of slope

b. increases with increase of slope

c. decreases with increase of slope

d. decreases with decrease of weight of the chain.

2. The method of reversal

a. is usually directed to examine whether a certain part is truly parallel or perpendicular to another

b. makes the erroneous relationship between parts evident

c. both (a) and (b)

d. neither (a) nor (b).

3. The real image of an object formed by the objective, must lie

a. in the plane of cross hairs

b. at the centre of the telescope

c. at the optical centre of the eye-piece

d. anywhere inside the telescope.

4. For setting out a simple curve, using two theodolites.

a. offsets from chord produced are required

b. offsets from long chord are required

c. deflection angles from Rankine's formula are required

d. none of these.

5. Bitumen paints offer

a. pleasing surface

b. hard surface

c. smooth surface

d. protective surface

e. rough surface.

6. The timber having maximum resistance against white ants, is obtained from

a. chir

b. shisham

c. sal

d. teak.

7. Invar contains

a. 12% of nickel

b. 24% of nickel

c. 30% to nickel

d. 36% of nickel.

8. The slag which floats on the surface of the molten iron generally contains

a. Silica (SiO2) 35%

b. Alumina (Al2O3) 12%

c. MgO, CaSO4, KMnO2 and FeO 8%

d. all the above.

9. Pick up the correct statement from the following:

a. For I sections, the shear centre coincides with the centroid of the cross section of the beam

b. For channels, the shear centre does not concide its centroid

c. Bending loads should pass through the shear centre to avoid twisting

d. All the above.

10. Pick up the correct statement from the following:

a. If tensile stress is less than axial stress, the section experiences compressive stress

b. If tensile stress is equal to axial stress, the section experiences compressive stress

c. If tensile stress is more than axial stress, some portion of the section experiences a tensile stress

d. All the above.

11. At either end of a plane frame, maximum number of possible bending moments, are

a. two

b. three

c. four

d. zero.

12. A beam of length L is pinned at both ends and is subjected to a concentrated bending couple of moment M at its centre. The maximum bending moment in the beam is

a. M

b. M/2

c. M/3

d. ML/2

13. A short tube mouthpiece will not run full at its outlet if the head under which the orifice works, is

a. less than 12.2 m of the water

b. more than 12.2 m of the water

c. equal of 12.2 m of water

d. none of these.

14. Atmospheric pressure varies with

a. altitude

b. temperature

c. weather conditions

d. all of the above.

15. Mercury is generally used in barometers because

a. the height of the barometer will be less

b. it is a best liquid

c. both (a) and (b) above

d. both (b) and (c) above.

16. The discharge through an internal mouth piece is more if its length is

a. <  diameter

b. < diameter

c. ≥ diameter

d. none of these.

17. The passive earth pressure of a soil, is proportional to (where φ is the angle of internal friction of the soil.)

a. tan (45° - φ)

b. tan (45° + φ)

c. tan2 (45° - φ

d. tan2 (45° + φ)

18. The maximum net pressure intensity causing shear failure of soil, is known

a. safe bearing capacity

b. net safe bearing capacity

c. net ultimate bearing capacity

d. ultimate bearing capacity.

19. The quantity of seepage of water through soils is proportional to

a. coefficient of permeability of soil

b. total head loss through the soil

c. neither (a) nor (b)

d. both (a) and (b).

20. The change of moisture content of soils, changes the

a. value of the angle of repose

b. amount of compaction required

c. cohesive strength of soil

d. all the above.

21. Pick up the incorrect statement from the following. The intensity of horizontal shear stress at the elemental part of a beam section, is directly proportional to

a. area of the section

b. distance of the C.G. of the area from its neutral axis

c. moment of the beam section about its neutral axis

d. width of the beam.

22. A part of the slab may be considered as the flange of the T-beam if

a. flange has adequate reinforcement transverse to beam

b. it is built integrally with the beam

c. it is effectively bonded together with the beam

d. all the above.

23. An under-reinforced section means

a. Steel is provided at the under side only

b. Steel provided is insufficient

c. Steel provided on one face only

d. Steel will yield first.

24. In a simply supported slab, alternate bars are curtailed at

a. 1/5th of the span

b. 1/6th of the span

c. 1/7th of the span

d. none of these.

25. A wall constructed to resist the pressure of an earth filling, is called

a. retaining wall

b. breast wall

c. buttress

d. parapet wall.

26. The concrete slump recommended for foundations, is

a. 30 to 125 mm

b. 50 to 100 mm

c. 75 to 125 mm

d. none of these.

27. A concrete using an air entrained cement

a. has more resistance to weathering

b. is more plastic and workable

c. is free from segregation and bleeding

d. all the above.

28. Addition of pozzolana to ordinary port land cement, causes

a. reduction in chemical action with sulphates

b. increase in shrinkage

c. reduction bleeding

d. all the above.

29. Turbidity of water may be caused due to

a. Suspended clay

b. Suspended silt

c. finely divided organic material

d. all the above.

30. For controlling algae, the most commonly used chemical, is

a. copper sulphate

b. alum

c. lime

d. bleaching powder.

31. If the over land flow from the critical point to the drain is 8 km and the difference in level is 12.4 m, the inlet time is

a. 2 hours

b. 3 hours

c. 4 hours

d. 5 hours.

32. The digested sludge from septic tanks, is removed after a maximum period of

a. 3 years

b. 3.5 years

c. 4 years

d. 5 years.

33. While deciding the alignment of a proposed canal taking off from a river at B, three alignments of approximately equal lengths are available. These cross a drainages at C1, C2 and C3 where drainage bed levels C1 > C2 > C3, you will provide at site C3

a. an aqueduct

b. a syphon aqueduct

c. a super passage

d. a syphon.

34. If the straight sides of a triangular section of a lined canal with circular bottom of radius R, make an angle θ with horizontal, the perimeter of the canal is

a. 2R(θ + tan θ)

b. R(θ + cos θ)

c. 2R(θ + cos θ)

d. none of these.

35. In a Sarda type fall, the rectangular crest, may be used for discharge upto

a. 10 cumecs

b. 14 cumecs

c. 20 cumecs

d. 25 cumecs.

36. In Montague type fall

a. a straight glacis is provided

b. a circular glacis is provided

c. a parabolic glacis is provided

d. no glacis is provided.

37. If cross slope of a country is greater than 60%, the terrain is classified as

a. rolling

b. mountainous

c. steep

d. plain.

38. When each particle of aggregates is thinly coated with cement paste, a heterogeneous solid is formed, which is known as

a. hydration

b. gel

c. concrete

d. none of these.

39. Reference pillars fixed on the centre line of a proposed road, provide the following information:

a. horizontal distance of road from the centre line

b. reduced level at the top of pillar

c. formation level of the road

d. all the above.

40. Along high ways confirmatory route markers are generally fixed

a. before the crossing on the left side

b. after the crossing on the left side

c. before the crossing on the right side

d. after the crossing on the right side.

41. Due to change in price level, a revised estimate is prepared if the sanctioned estimate exceeds

a. 2.0%

b. 2.5%

c. 4.0%

d. 5.0%

42. Pick up the excavation where measurements are made in square metres for payment.

a. surface dressing up to 15 cm depths

b. Surface excavation up to 30 cm depths

c. Both (b) and (c)

d. Both (a) and (b)

43. Pick up the correct statement from the following:

a. The bent up bars at a support resist the negative bending moment

b. The bent up bars at a support resist the sharing force

c. The bending of bars near supports is generally at 45°

d. All the above.

44. In long and short wall method of estimation, the length of long wall is the centre to centre distance between the walls and

a. breadth of the wall

b. half breadth of wall on each side

c. one fourth breadth of wall on each side

d. None of these.

45. Pick up the correct statement from the following:

a. Pessimistic time estimate refers to activities

b. Most likely time estimate refers to activities

c. Expected time estimate refers to activities

d. All the above.

46. Modular co-ordination of construction means proper

a. planning

b. designing

c. execution

d. all the above.

47. A critical ratio scheduling

a. determines the status of each activity

b. adjusts automatically changes in activity progress

c. is a dynamic system

d. none of these.

48. For the supply of materials for concrete, form work reinforcing and placing of concrete, removal of form work and curing of concrete, number of bar(s) required on bar chart, is

a. 1

b. 2

c. 3

d. 4

49. International Civil Aviation Organisation (I.C.A.O.) was set up at Montreal (Canada), in

a. 1929

b. 1939

c. 1947

d. 1950

50. Conical surface of the approach area rises outwards

a. 1 in 10

b. 1 in 15

c. 1 in 20

d. 1 in 25