***Sub Engineer***

1. The reduced level of a floor is 99.995 m, the staff reading on the floor is 1.505 m. If the inverted staff reading against the roof is 1.795 m, the floor level below the slab, is

a. 3.290 m

b. 3.300 m

c. 3.275 m

d. 2.790 m

2. The latitude of a traverse leg is obtained by multiplying its length by

a. tangent of its reduced bearing

b. sign of its reduced bearing

c. cosine of its reduced bearing

d. cosecant of its reduced bearing.

3. The apparent error on reversal is

a. equal to the actual error

b. twice the actual error

c. thrice the actual error

d. none of these.

4. The direction of steepest slope on a contour, is

a. along the contour

b. at an angle of 45° to the contour

c. at right angles to the contour

d. none of these.

5. During smelting process, the combination of fuel in the furnace

a. forms carbon dioxide

b. carbon dioxide with carbon forms carbon mono-oxide

c. carbon mono-oxide reacts with Fe2O3 to form iron and liberates CO2

d. all the above.

6. Permanent magnets are made of high carbon steel and

a. 15% of cobalt

b. 20% of cobalt

c. 35% of cobalt

d. 45% of cobalt.

7. Pick up the correct statement from the following:

a. The average crushing strength of hand moulded bricks is 6000 t/m2

b. The average tensile strength of hand moulded brick is 200 t/m2

c. The average shearing strength of hand moulded brick is 600 t/m2

d. All the above.

8. Galvanising means covering iron with a thin coat of

a. tin

b. zinc

c. glaze

d. coal tar.

9. The ratio of crippling loads of a column having both the ends fixed to the column having both the ends hinged, is

a. 1

b. 2

c. 3

d. 4

10. A shaft rotating N.R.M. under a torque T, transmits a power

a.  Newton metres/sec

b.  Newton metres/min

c.  Newton metres/min

d.  Newton metres/sec

11. The section modulus of a rectangular section is proportional to

a. area of the section

b. square of the area of the section

c. product of the area and depth

d. product of the area and width

12. The radius of gyration of a rectangular section is not proportional to

a. square root of the moment of inertia

b. square root of the inverse of the area

c. square root of the moment of inertia divided by area of the section

d. none of these.

13. A floating body attains stable equilibrium if its metacentre is

a. at the centroid

b. above the centroid

c. below the centroid

d. anywhere.

14. The property of steam function ψ is :

a. the flow around any path in the fluid is zero for continuous flow

b. the rate of change of ψ with distance in an arbitrary direction, is proportional to the component of velocity normal to that direction

c. the velocity vector may be found by differentiating the stream function

d. all the above.

15. Flow of water in pipes of diameter more than 3 metres, can be measured by

a. pitot tube

b. venturimeter

c. orifice plate

d. rotameter.

16. Molecules of fluids get attracted due to

a. surface tension

b. adhesion

c. cohesion

d. none of these.

17. Pick up the correct statement from the following:

a. The percent settlement at any time is called degree of consolidation

b. Time factor is a dimensionless quantity

c. The initial curve on either side of the point of unloading and reloading is called 'virgin curve'

d. All the above.

18. The shear strength in plastic undrained clay, is due to

a. inter-granular friction

b. internal friction

c. cohesion

d. none of these.

19. In a fow net

a. flow lines and equipotential lines cross each other at right angles

b. fields are rectangles whose length is twice the breadth

c. smaller the dimensions of the field, smaller will be the hydraulic gradient and velocity of flow through it

d. for homogeneous soil, the curves are smooth and circular.

20. Pick up the correct statement from the following:

a. Kaolinite shows a very little sign of swelling on wetting

b. Kaolinite when wet, becomes moderately plastic because negative electro magnetic charges on platelets attrack water

c. Kaolinite is also called China clay

d. All the above.

21. For stairs spanning horizontally, the minimum waist provided is

a. 6 cm

b. 8 cm

c. 10 cm

d. 12 cm.

22. Pick up the incorrect statement from the following. The intensity of horizontal shear stress at the elemental part of a beam section, is directly proportional to

a. area of the section

b. distance of the C.G. of the area from its neutral axis

c. moment of the beam section about its neutral axis

d. width of the beam.

23. The maximum shear stress (qmax) in a rectangular beam is

a. 1.50 times the average

b. 1.75 times the average

c. 2.0 times the average

d. 2.5 times the average.

24. A short column 20 cm x 20 cm in section is reinforced with 4 bars whose area of cross section is 20 sq. cm. If permissible compressive stresses in concrete and steel are 40 kg/cm2 and 300 kg/cm2, the Safe load on the column, should not exceed

a. 4120 kg

b. 41, 200 kg

c. 412, 000 kg

d. none of these.

25. Raft foundations are used for :

a. providing increased area of foundation over poor bearing capacity of soil

b. spanning over small soft or loose pockets

c. counter acting the hydrostatic effect

d. all the above.

26. The single stage well point system of dewatering an excavation can be used if the depth of excavation does not exceed

a. 5 m

b. 10 m

c. 15 m

d. 20 m

27. Placing of concrete should preferably be done at a temperature of

a. 10°C

b. 20°C

c. 23°C

d. 27 ± 2°C

28. The percentage of the aggregate of F.M. 2.6 to be combined with coarse aggregate of F.M. 6.8 for obtaining the aggregates of F.M. 5.4, is

a. 30%

b. 40%

c. 50%

d. 60%.

29. The rate of silting in a reservoir

a. remains constant throughout

b. is more in the beginning

c. is more in the beginning and reduces in the end

d. none of these.

30. Estimates of a water supply project depends upon the rate of water supply per capita consumption and probable population estimated at the end of the design period of

a. 10 to 15 years

b. 15 to 20 years

c. 20 to 30 years

d. 25 years.

31. The liquid wastes from kitchens, bath rooms and wash basins, is not called

a. liquid waste

b. sullage

c. sewage

d. none of these.

32. During putrifaction process of sewage the gas given off, is

a. hydrogen

b. ammonia

c. methane

d. all the above.

33. Pick up the correct statement from the following:

a. The full supply level of a canal should be above ground level

b. According to Lacey, regime conditions require a particular slope for a given discharge and silt factor

c. In case the ground slope is less than the required bed slope, the silt factor must be reduced by permitting the entry of coarse silt

d. All the above.

34. Irrigation canals are generally aligned along

a. ridge line

b. contour line

c. valley line

d. straight line.

35. The Lacey's regime velocity is proportional to

a. R1/2 S3/4

b. Q3/4 S1/3

c. R3/4 S1/3

d. R2/3 S1/2

36. If the straight sides of a triangular section of a lined canal with circular bottom of radius R, make an angle θ with horizontal, the perimeter of the canal is

a. 2R(θ + tan θ)

b. R(θ + cos θ)

c. 2R(θ + cos θ)

d. none of these.

37. Full amount of extra width of a pavement on the curve, is provided at

a. beginning of the transition curve

b. centre of the transition curve

c. beginning of the circular curve

d. centre of the circular curve.

38. If the rate of change of the super-elevation along a curved portion of a 7 metre wide road is 1 in 150 and the maximum super-elevation allowed is 1 in 15, the maximum length of the transition curve to be provided at either end, is

a. 70 m

b. 75 m

c. 80 m

d. 90 m

39. The traffic manoeuvre means

a. diverging

b. merging

c. crossing

d. all the above.

40. Minimum number of 50 kg cement bags per cubic metre of concrete for a mix corresponding to crushing strength 280 kg/cm2 at 28 days, are

a. 6.5

b. 7.0

c. 7.5

d. 8.0

41. The measurement is not made in square metres in case of

a. D.P.C. (Damp proof course)

b. Form works

c. Concrete Jaffries

d. R.C. Chhajja.

42. Pick up the correct statement regarding the centre line method of estimating a building

a. Product of the centre line of the walls and area of cross-section of any item, gives total quantity of the item

b. The centre line is worked out separately for different sections of walls of a building

c. The centre line length is reduced by half the layer of main wall joining the partition wall

d. All the above.

43. Berms are provided in canals if these are

a. fully in excavation

b. partly in excavtion and partly in embankment

c. fully in embankment

d. All the above.

44. Referring of given figure, pick up the correct statement from the following:

a. The total length of centre line of four walls is 20 m

b. Length of long wall out-to-out is 6.80 m

c. Length of short walls in-to-in is 3.20 m

d. All the above.

45. Pick up the correct statement from the following:

a. If the float is positive and the activity is delayed by a period equal to its total float, the completion of project is not delayed

b. If the float of an activity is negative, delay in its performance is bound to delay the completion of project

c. If the float of an activity is zero, the activity is critical and any delay in its performance will delay the whole project

d. All the above.

46. The most popular type of organisation used for Civil Engineering Constructions, is

a. line organisation

b. line and staff organisation

c. functional organisation

d. effective organisation.

47. Pick up the PERT event from the following:

a. Digging of foundation completed

b. Laying of concrete started

c. Laying of concrete completed

d. All the above.

48. The main principle of an organisation, is

a. cohegency

b. effective control at all levels

c. delegation of authority

d. all the above.

49. To cope up high temperature of 196°C, the taxi ways and aprons are constructed with

a. rubberised tar concrete

b. plain concrete

c. epoxy asphalt concrete

d. all the above.

50. Pick up the correct statement from the following:

a. Landing speed is directly proportional to the wing loading

b. Wing loading remaining constant, the take off distance is directly proportional to the powder loading

c. Neither (a) nor (b)

d. Both (a) and (b)