***Sub Engineer***

1. The staff intercept will be

a. greater farther off the staff is held

b. smaller, farther off the staff is held

c. smaller, nearer the staff is held

d. same, wherever the staff is held.

2. Mistakes which may produce a very serious effect upon the final results arise due to

a. in attention

b. in experience

c. carelessness

d. all of these.

3. If R is the radius of the main curve, θ the angle of deflection, S the shift and L the length of the transition curve, then, total tangent length of the curve, is

a. (R + S) tan θ/2 - L/2

b. (R + S) tan θ/2 + L/2

c. (R - S) tan θ/2 + L/2

d. (R - S) cos θ/2 + L/2

4. A dumpy level is set up with its eye-piece vertically over a peg A. The height from the top of peg A to the centre of the eye-piece is 1.540 m and the reading on peg B is 0.705 m. The level is then setup over B. The height of the eye-piece above peg B is 1.490 m and a reading on A is 2.195 m. The difference in level between A and B is

a. 3.030 m

b. 0.770 m

c. 0.785 m

d. 1.770 m.

5. Vanadium steel is generally used for

a. railway switches and crossing

b. bearing balls

c. magnets

d. axles and springs.

6. Pig iron obtained from the furnance which is properly provided with fuel at a very high temperature, is called

a. Grey or foundry pig

b. White or forge pig

c. Mottled pig

d. None of the above.

7. Pick up the constituent of good brick earth whose excess causes the raw bricks shrink and warp during drying and burning, from the following:

a. Alumina

b. Lime

c. Ironoxide

d. Magnesia.

8. The foliated structure is very common in

a. sedimentary rocks

b. igneous rocks

c. metamorphic rocks

d. none of these.

9. The eccentricity (e) of a hollow circular column, external diameter 25 cm, internal diameter 15 cm for an eccentric load 100 t for non-development of tension, is

a. 3.00 cm

b. 3.50 cm

c. 4.25 cm

d. 5.0 cm

10.  is the equation of Euler's crippling load if

a. both at the ends are fixed

b. both the ends are hinged

c. one end is fixed and other end is free

d. one end is fixed and other end is hinged.

11. Shear force for a cantilever carrying a uniformly distributed load over its length, is

a. triangle

b. rectangle

c. parabola

d. cubic parabola.

12. A simply supported beam (l + 2a) with equal overhangs (a) carries a uniformly distributed load over the whole length, the B.M. changes sign if

a. l > 2a

b. l < 2a

c. l = 2a

d. l = 4a

13. 'Flow net' can be drawn only if the flow is

a. turbulent

b. rotational

c. distortion

d. none of these.

14. To avoid an interruption in the flow of a syphon, an air vessel is provided

a. at the inlet

b. at the outlet

c. at the summit

d. at any point between intet and outlet.

15. The horizontal component of the force on a curved surface is equal to

a. weight of liquid vertically below the curved surface

b. force on a vetical projection of the curved surface

c. product of pressure at its centroid and the area

d. weight of liquid retained by the curved area.

16. Cavitation is caused by

a. Low pressure

b. High pressure

c. Low velocity

d. High velocity

17. The phreatic line in an earth dam may be

a. elliptical

b. parabolic

c. a straight line

d. all the above.

18. If the failure of a finite slope occurs through the toe, it is known as

a. slope failure

b. face failure

c. base failure

d. toe failure.

19. The fundamental equation of specific gravity (G), dry density (γd), unit weight of water (γω) and void ratio (e), is

a. e =

b. G =

c. γd =

d. γω =

20. 'Drift' is the material picked up, mixed, disintegrated, transported and redeposited by

a. wind

b. gravitational force

c. glaciated water

d. all the above.

21. If the ratio of the span to the overall depth does not exceed 10, the stiffness of the beam will ordinarily be satisfactory in case of a

a. simply supported beam

b. continuous beam

c. cantilever beam

d. none of these.

22. Distribution reinforcement in a simply supported slab, is provided to distribute

a. load

b. temperature stress

c. shrinkage stress

d. all the above.

23. The stem of a cantilever retaining wall which retains earth level with top is 6 m. If the angle of repose and weight of the soil per cubic metre are 30° and 2000 kg respectively, the effective width of the stem at the bottom, is

a. 52.5

b. 53.5

c. 54.5

d. 55.5

24. Columns may be made of plain concrete if their unsupported lengths do not exceed their least lateral dimension

a. three times

b. four times

c. five times

d. six times.

25. The entrained concrete is used in lining walls and roofs for making

a. heat insulated

b. sound insulated

c. neither (a) nor (b)

d. both (a) and (b).

26. The strength of brick masonry in 1:6 cement mortar, is

a. 40 tonnes/m2

b. 50 tonnes/m2

c. 60 tonnes/m2

d. 75 tonnes/m2.

27. The shrinkage of concrete

a. is proportional to water content is the mix

b. is proportional to cement concrete

c. increases with age of concrete

d. all the above.

28. 'Ware house pack' of cement means

a. pressure exertion of the bags of upper layers

b. pressure compaction of the bags on lower layers

c. packing the ware house

d. none of these.

29. Pick up the correct statement from the following :

a. The amount of rainfall in one year, is known as annual rainfall

b. The rain cycle period in India is taken as 35 years

c. The ratio of the actual rainfall in the year to the normal rainfall at a place, is called index of the wetness

d. All the above.

30. The population growth curve is

a. S-shaped curve

b. parabolic curve

c. circular curve

d. straight line

31. Large diameter sewers subjected to external pressure alone, are reinforced

a. near the outer surface of the pipe

b. both (a) and (b)

c. with elliptical cage

d. none of these.

32. The stone ware sewers

a. are manufactured from clays and shales

b. after casting are dried and burnt at temperature 150°C, 700°C and 1200°C

c. are generally provided with a water proof cover by vaporising sodium chloride

d. all the above.

33. The intensity of irrigation means

a. percentage of culturable commanded area to be irrigated annually

b. percentage of gross commanded area to be irrigated annually

c. percentage of the mean of culturable commanded area and the gross commanded area to be irrigated annually

d. total depth of water supplied by the number of waterings.

34. The velocity of drainage water in the barrels of a syphon-aqueduct, is normally limited to

a. 1 to 2 m per second

b. 2 to 3 m per second

c. 3 to 4 m per second

d. 4 to 5 m per second.

35. In rigid module, the discharge

a. is independent of water levels in the distributary and water course

b. depends upon the water level in distributary

c. depends upon the water level in the water course

d. depends upon the water levels of both, i.e. distributary and water course

36. A hydraulic jump is generally formed when a stream moving with

a. a hyper-critical velocity meets a stream moving with a critical velocity

b. a hyper-critical velocity meets a stream moving with a hyper-critical velocity

c. A hyper-critical velocity meets a stream moving with a sub-critical velocity

d. a sub-critical velocity meets a stream moving with a hyper-critical velocity.

37. Retaining walls are generally constructed with dry rubble stones with 60 cm top width and

a. 1 : 2 front batter

b. 1 : 3 front batter

c. 1 : 4 front batter

d. 1 : 5 front batter

38. If R is the radius of a main curve and L is the length of the transition curve, the shift of the curve, is

a. L2/24 R

b. L3/24 R

c. L4/24 R

d. L/12 R

39. The minimum road width is taken

a. 9 m

b. 12 m

c. 16 m

d. 20 m

40. An ideal vertical curve is

a. cubic spiral

b. cubic parabala

c. lamniscate

d. none of these.

41. While estimating the qualities for the construction of a building, the correct metric unit is

a. Cubic metre for area

b. Square metres for volume

c. Kilogram for weight

d. Litre for capacity

42. For 12 mm thick cement plastering 1 : 6 on 100 sq.m new brick work, the quantity of cement required, is

a. 0.200 m3

b. 0.247 m3

c. 0.274 m3

d. 0.295 m3

43. While preparing a detailed estimate

a. Dimension should be measured correct to 0.01 m

b. Area should be measured correct to 0.01 sqm

c. Volume should be measured correct to O.Olcum

d. All the above.

44. The area of a sloping surface of a protective embankment of mean height d, side slopes S : 1 and length L is

a. d x d x s

b. d2 x (ds)2

c. L.D 1 + s2

d. 2 LD 1 + s2

45. If TL is the latest allowable event occurrence time, total activity slack(s), is equal to

a. LST-EST

b. LFT-EFT

c. TL-EFT

d. all the above.

46. Railway projects are treated as

a. light construction

b. heavy construction

c. industrial construction

d. none of these.

47. While filling the tender for any work, the contractor considers

a. availability of construction materials

b. availability of labour

c. study of specifications

d. all the above.

48. Pick up the correct statement from the following:

a. Latest occurrence time is denoted by TL

b. Contractual obligation time is denoted by Ts

c. Latest occurrence time is taken as contractual obligation time

d. All the above.

49. Pick up the correct statement from the following:

a. The distance between the points of intersection of the extreme tangents to the transition curve is kept greater than 7500 m x sum of grade changes at the point of intersection

b. The rate of change of grade is limited to 0.3% per 30 m length of the curve

c. According to I.C.A.O. the maximum longitudinal gradient along a runway is limited to 1.5%

d. All the above.

50. According to I.C.A.O. the slope of transitional surface at right angles to the centre line of runway, is kept

a. 1 in 5

b. 1 in 6

c. 1 in 7

d. 1 in 10