***Sub Engineer***

1. One of the tacheometric constants is additive, the other constant, is

a. subtractive constant

b. multiplying constant

c. dividing constant

d. indicative constant.

2. Angles to a given pivot station observed from a number of traverse stations when plotted, the lines to the pivot station intersect at a common point

a. angular measurements are correct and not the linear measurements

b. linear measurements are correct and not the angular measurements

c. angular and linear measurements are correct and not the plotting of traverse

d. angular and linear measurements and also plotting of the traverse are correct.

3. In a telescope the object glass of focal length 14 cm, is located at 20 cm from the diaphragm. The focussing lens is midway between them when a staff 16.50 m away is focussed. The focal length of the focussing lens, is

a. 5.24 cm

b. 6.24 cm

c. 7.24 cm

d. 8.24 cm

4. Perpendicular offset from a tangent to the junction of a transition curve and circular curve is equal to

a. shift

b. twice the shift

c. thrice the shift

d. four times the shift.

5. Cast iron is used for

a. structural works in beams

b. small sized water pipes

c. columns and struts

d. none to these.

6. Which one of the following polymers is obtained from condensation polymerization?

a. phenol formaldehyde

b. carbamide

c. melamine-formaldehyde

d. all of these.

7. The rocks which are formed due to cooling of magma at a considerable depth from earth's surface are called

a. Plutonic rocks

b. Hypabyssal rocks

c. Volcanic rocks

d. Igneous rocks.

8. The yield strength and tensile strength of low carbon steel may be improved by the addition of

a. chromium

b. nickel

c. venadium

d. tungsten.

9. If a solid shaft (diameter 20 cm, length 400 cm, N = 0.8 x 105 N/mm2) when subjected to a twisting moment, produces maximum shear stress of 50 N/mm2, the angle of twist in radians, is

a. 0.002

b. 0.0025

c. 0.003

d. 0.005

10. For beams of uniform strength, if depth is constant,

a. width b ∝ M

b. width b ∝ M

c. width b ∝ 3 M

d. width b ∝

11. The weakest section of a diamond riveting, is the section which passes through

a. first row

b. second row

c. central raw

d. one rivet hole of end row.

12. The ratio of the moment of inertia of a circular plate and that of a square plate for equal depth, is

a. equal to one

b. more than one

c. equal to 3π/16

d. none of these.

13. Uniform flow is said to occur when

a. size and shape of the cross-section in a particular length remain constant

b. size and shape of the cross-section change along a length

c. frictional loss in the particular length of the channel will the more than the drop in its elevation

d. frictional loss in the particular length of the channel, will be less than the drop in elevation.

14. To measure very low pressure, we use

a. barometers

b. piezometers

c. manometers

d. differential manometers.

15. The ratio of the percentage error in the discharge and percentage error in the measurement of head, over rectangular notch, is

a. 1/2

b. 2/3

c. 3/2

d. 3/4

16. The width of a weir with end contraction, is

a. equal to the width of the channel

b. less than the width of the channel

c. half the width of the channel

d. none of these.

17. Determination of water content of a soil sample suspected to contain gypsum is made by drying the sample for longer period at a temperature not more than

a. 60°C

b. 80°C

c. 100°C

d. 110°C

18. Pick up the incorrect statement from the following:

a. Below the water table, the pore water may be static.

b. The hydrostatic pressure depends on the depth below the water level.

c. The attractive forces between the particles, caused due to negative pressure of water held above the water table is called soil suction.

d. None of these.

19. Pick up the correct statement from the following:

a. Illite bond is stronger than montmorillonite bond

b. Illites do not swell when wet

c. Illites are composed of two silica tetrahedralsheets with a central octahedral sheet

d. All the above.

20. The specific gravity of Calcite is

a. 2.65

b. 2.72

c. 2.85

d. 2.90

21. The length of the lap in a compression member is kept greater than bar diameter x (Permissible stress in bar / Five times the bond stress) or

a. 18 bar diameters

b. 24 bar diameters

c. 30 bar diameters

d. 36 bar diameters

22. The effective span of a simply supported slab, is

a. distance between the centres of the bearings

b. clear distance between the inner faces of the walls plus twice the thickness of the wall

c. clear span plus effective depth of the slab

d. none of these.

23. In a prestressed beam carrying an external load W with a bent tendon is having angle of inclination θ and prestressed load P. The net downward load at the centre is

a. W - P cos θ

b. W - P sin θ

c.  W - 2P sin θ

d. W + 2P sin θ

24. A short column 20 cm x 20 cm in section is reinforced with 4 bars whose area of cross section is 20 sq. cm. If permissible compressive stresses in concrete and steel are 40 kg/cm2 and 300 kg/cm2, the Safe load on the column, should not exceed

a. 4120 kg

b. 41, 200 kg

c. 412, 000 kg

d. none of these.

25. The arrangement made to support an unsafe structure temporarily, is known as

a. shoring

b. scaffolding

c. underpinning

d. jacking

26. Black cotton soil is unsuitable for foundations because its

a. bearing capacity is low

b. permeability is uncertain

c. particles are cohesive

d. property to undergo a volumetric change due to variation of moisture content.

27. The type of aggregates not suitable for high strength concrete and for pavements subjected to tension, is

a. rounded aggregate

b. irregular aggregate

c. angular aggregate

d. flaky aggregate

28. The entrained air in concrete

a. increases workability

b. decreases workability

c. decreases resistance to weathering

d. increases strength

29. Pick up the correct statement from the following :

a. Lead and barium salts have toxic effect

b. Arsenic and selenium are poisonous to human health

c. Higher copper content affects the lungs

d. All the above.

30. Permanent hardness of water can be removed by

a. adding lime

b. adding chlorine

c. boiling

d. zeolite process.

31. Pick up the correct statement from the following :

a. In treated sewage, 4 PPm of D.O. is essential

b. Only very fresh sewage contains some dissolved oxygen

c. The solubility of oxygen in sewage is 95% that is in distilled water

d. All the above.

32. The amount of oxygen consumed by the aerobic bacterias which cause the aerobic biological decomposition of sewage, is known

a. Bio-Chemical Oxygen Demand (B.O.D.)

b. Dissolved Oxygen (D.O.)

c. Chemical Oxygen Demand (C.O.D.)

d. None of these.

33. Pick up the correct statement from the following

a. In a level crossing, a crest with its top at the canal F.S.L. is provided across the drainage at its up-stream junction with canal

b. In a level crossing a regulator is provided across the drainage at its down-stream

c. In a level crossing, a cross regulator is provided on the canal below the crossing

d. All the above.

34. Bligh's theory of seepage assumes

a. equal weightage to the horizontal and vertical creep

b. more weightage to horizontal creep than vertical creep

c. less weightage to horizontal creep than vertical creep

d. loss of head follows the sine curve.

35. If the straight sides of a triangular section of a lined canal with circular bottom of radius D, make an angle θ with horizontal, the hydraulic mean depth is

a. D/2

b. D/3

c. D/4

d. D/5

36. For a unique design of a channel by Kennedy's theory

a. its breadth must only be known

b. its depth must only be known

c. its breadth and depth ratio must only be known

d. all the above.

37. Round-abouts are not suitable if number of vehicles exceed

a. 3000

b. 4000

c. 5000

d. 6000

38. Length of vehicles does not affect

a. minimum radius of turning

b. passing sight distance

c. width of shoulders

d. none of these.

39. If P is the number of vehicles per day at last census, r is the increase in traffic and n is the number of years passed after last census, number of vehicles A per day for design, is

a. P (1 + r)n

b. P (1 - r)n

c. P (1 + r)-n+5

d. P (1 + r)5n

40. If the difference in elevation of an edge of the pavement 9 m wide and its crown is 15 cm, the camber of the pavement, is

a. 1 in 45

b. 1 in 30

c. 1 in 15

d. 1 in 7.5

41. The slope of the oultet of 'P trap' below the horizontal is kept

a. 8°

b. 10°

c. 12°

d. 14°

42. The expected out turn of 2.5 cm cement concrete floor per manson per day

a. 2.5 sqm

b. 5.0 sqm

c. 7.5 sqm

d. 10 sqm

43. Pick up the correct statement from the following:

a. The earth work of cutting in trenches or borrow pits in fairly uniform ground is measured with the help of average depths of the dead men

b. The earth work in trenches or borrow pits in irregular ground is measured by taking the difference in levels before and after completion of work

c. The earth work in trenches or borrow pits, where neither a nor b is feasible, are measured from the fillings after deduction of voids

d. All the above.

44. Pick up the correct statement from the following:

a. The actual expenditure involved to complete a work including incidental, establishment and travelling charges, is called actual cost

b. The formal acceptance by the administrative department for incurring an expenditure on the work, is called administrative approval

c. The order of a competent authority sanctioning a properly detailed estimate of the cost of a work of construction or repair is called technical sanction

d. All the above.

45. Frequency distribution curves

a. having a single lump, are called uninodal curves

b. if symmetrical, are called normal curves

c. if not symmetrical, are called skew curves

d. all the above.

46. The time by which activity completion time can be delayed without affecting the start of succeeding activities, is known as

a. duration

b. total flat

c. free float

d. interfering float.

47. The performance of a specific task in CPM, is known

a. Dummy

b. Event

c. Activity

d. Contract.

48. Pick up the incorrect statement from the following:

a. The beginning and end of a job, are called events

b. The activity which consumes maximum time, is called a node

c. Logically and sequentially connected activities and events form a network

d. None of these.

49. The threshold markings are

a. 1 m clear space between adjacents

b. 45 m in length

c. placed symmetrically on either side of the runway centre line

d. all the above.

50. According to I.C.A.O. the slope of transitional surface at right angles to the centre line of runway, is kept

a. 1 in 5

b. 1 in 6

c. 1 in 7

d. 1 in 10