***Sub Engineer***

1. The line normal to the plumb line is known as

a. horizontal line

b. level line

c. datum line

d. vertical line.

2. If h is the difference in level between end points separated by l, then the slope correction is  . The second term may be neglected if the value of h in a 20 m distance is less than

a.  m

b. 1 m

c. 2 m

d. 3 m

3. The angle of intersection of a contour and a ridge line, is

a. 30°

b. 45°

c. 60°

d. 90°.

4. The difference in the lengths of an arc and its subtended chord on the earth surface for a distance of 18.2 km, is only

a. 1 cm

b. 5 cm

c. 10 cm

d. 100 cm.

5. Portland pozzolana cement possesses

a. lower heat of hydration

b. lower shrinkage on drying

c. water tightness

d. all the above.

6. Laterite is a/an

a. volcanic rock

b. argillaceous rock

c. calcareous rock

d. silicious rock.

7. Portland cement manufactured from pure white chalk and clay but free from iron-oxide, is known as

a. quick setting cement

b. rapid hardening cement

c. white cement

d. low heat Portland cement.

8. The fire clay contains pure

a. lime

b. oxide of iron

c. hydrated aluminium silicate

d. magnesium.

9. The ratio of maximum and average shear stresses on a rectangular section, is

a. 1.25

b. 1.5

c. 2.0

d. 2.5

10. If a solid shaft (diameter 20 cm, length 400 cm, N = 0.8 x 105 N/mm2) when subjected to a twisting moment, produces maximum shear stress of 50 N/mm2, the angle of twist in radians, is

a. 0.002

b. 0.0025

c. 0.003

d. 0.005

11. Pick up the correct statement from the following :

a. In the standard rolled channels, the shear centre is on the horizontal line passing through and away from the C.G. beyond web

b. In equal angles, the shear centre is on the horizontal plane and away from the C.G., outside of the leg projection

c. In T-sections, the shear centre is at the C.G. of the section

d. All the above.

12. For a beam having fixed ends, the unknown element of the reactions, is

a. horizontal components at either end

b. vertical components at either end

c. horizontal component at one end and vertical component at the other

d. horizontal and vertical components at both the ends.

13. The intensity of pressure due to sudden closure of a valve of a pipe in which water flows with velocity v, is directly proportional to :

a. square root of the bulk modulus of elasticity of water

b. bulk modulus of elasticity of water

c. specific weight of water

d. none of these.

14. Specific weight of sea water is more than that of pure water because of

a. dissolved air

b. dissolved salts

c. suspended matter

d. all the above.

15. If cohesion between the molecules of a fluid is more than adhesion between the fluid and glass, the free level of fluid in a dipped glass tube will be

a. higher than the surface of liquid

b. same as the surface of liquid

c. lower than the surface of liquid

d. none of these.

16. Though angle of deviation of liquid is more in internal mouth piece, the contraction of the jet, is

a. more in the internal mouth piece

b. less in the internal mouth piece

c. equal to external mouth piece

d. none of these.

17. Number of piles required to support a column, is

a. 1

b. 2

c. 3

d. 4

18. Failure of a slope occurs only when total shear force is

a. equal to total shearing strength

b. greater than total shearing strength

c. less than total shearing strength

d. none of these.

19. The seepage exit gradient in a soil is the ratio of

a. flow line to slope

b. head upstream to that at downstream

c. head loss to the length of the seepage

d. none of these.

20. The specific gravity of Calcite is

a. 2.65

b. 2.72

c. 2.85

d. 2.90

21. An R.C.C. lintel is spanning an opening of 2 m span in a brick wall. The height of the roof is 2.9 m above the floor level and that of the opening is 2.1 m above the floor level. The lintel is to be designed for self weight plus

a. triangular load of the wall

b. UDL of wall

c. UDL of wall + load from the roof

d. triangular load + load from the roof.

22. If the tendon is placed at an eccentricity e below the centroidal axis of the lon-gitudial axis of a rectangular beam (sectional modulus Z and stressed load P in tendon) the stress at the extreme top edge

a. is increased by

b. is increased by

c. is decreased by

d. remains unchanged.

23. In the zone of R.C.C. beam where shear stress is less than 5 kg/cm2, nominal reinforcement is provided at a pitch of

a. one-half lever arm of the section

b. one-third lever arm of the section

c. lever arm of the section

d. one and half lever arm of the section.

24. A simply supported beam , 6 m long and of effective depth 50 cm, carries a uniformly distributed load 2400 kg/m including its self weight. If the lever arm factor is 0.85 and permissible tensile stress of steel is 1400 kg/cm2, the area of steel required, is

a. 15 cm2

b. 16 cm2

c. 17 cm2

d. 18 cm2

25. The alignment of a cross joint along the plumb line is

a. bed block

b. perpend

c. lintel

d. vertical line.

26. Pick up the incorrect statement from the following :

a. No  timbering is  required  for  shallow trenches

b. Shallow foundations can be constructed on made-up soil

c. Grillage   foundation   is   classified as a shallow foundation

d. Black cotton soil is very good for foundation bed.

27. Pick up the correct statement from the following:

a. The free water is the amount of water added while mixing and the amount of water held on the surface of the aggregates prior to mixing

b. The total water is the free water and the amount actually absorbed by the aggregates

c. Neither (a) nor (b)

d. Both (a) and (b).

28. To hydrate 500kg of cement full water needed, is

a. 110 kg

b. 120 kg

c. 130 kg

d. 140 kg

29. Pickup the incorrect statement from the following :

a. The invert of pressure conduit is independent of the grade of the hydraulic gradient line

b. The pressure conduits may be taken uphill upto a maximum height of 8.3 m

c. Aqueducts and tunnels sections are generally kept circular

d. None of these.

30. The diameter of pipes in bath rooms and lavatories in domestic water supply, is

a. 6 mm

b. 12 mm

c. 18 mm

d. 24 mm.

31. For design purposes, the normal rate of infiltration of ground water into the sewer, is

a. 1000 litres/km/cm

b. 1500 litres/km/cm

c. 2000 litres/km/cm

d. 2500 litres/km/cm.

32. Aerobic activity is maximum

a. in freshly produced sewage

b. at sewer pipes

c. in sewer treatment plants

d. none of these.

33. Pick up the correct sequence of the part of a canal system from the following

a. Head work-distributary-branch canal-minor

b. Head works-main canal-branch canal-dis-tributary-minor

c. Head works-main canal-branch canal-minor-distributary

d. Heads works-branch canal-main canal distributary, minor.

34. Lacy's regime condition is obtained if

a. silt grade in the channel is variable

b. discharge in the channel is variable

c. silt charge in the channel is variable

d. channel flows in unlimited, incoherent alluvium of the same character as that transported material.

35. Canals taken off from ice-fed perennial rivers, are known

a. ridge canals

b. perennial canals

c. inundation canals

d. ice canals.

36. Garnett's diagrams are used for graphical solution of design equations of a canal by

a. Lacey's theory

b. Kennedy's theory

c. Gibb's theory

d. Lindlay theory.

37. The minimum cross fall of shoulders is kept

a. 1.0%

b. 1.5%

c. 2.5%

d. 3%

38. Side drains on both sides of a hill road, are essential when the road is

a. along the spur curves

b. along the re-entrant curves

c. in cutting

d. none of these.

39. If cross slope of a country is greater than 60%, the terrain is classified as

a. rolling

b. mountainous

c. steep

d. plain.

40. Driving vehicles on wet surfaced roads, is dangerous because it may

a. skid

b. slip

c. overturn

d. all the above.

41. Brick walls are measured in sq. m if the thickness of the wall is

a. 10 cm

b. 15 cm

c. 20 cm

d. None of these.

42. Pick up the correct statement from the following:

a. Cement is paid per 50 kg bag

b. Lime is paid per quintal

c. Brick aggregates is paid per cum

d. All the above.

43. If tensile stress of a steel rod of diameter D is 1400 kg/cm2 and bond stress is 6 kg/cm2, the required bond length of the rod is

a. 40 D

b. 50 D

c. 53 D

d. 59 D

44. Pick up the incorrect statement from the following:

a. Lead is the average horizontal straight distance between the borrow pit and the place of spreading soil

b. The lead is calculated for each block of the excavated area

c. The unit of lead is 50 m for a distance upto 500 m

d. The unit of lead is 1 km where the lead exceeds 2 km.

45. While scheduling a project by C.P.M.

a. required time for each activity is established

b. sequence of various activities is made according to their importance

c. net work is drawn by connecting the activities and the events

d. All the above.

46. A goiden rule for the procurement of construction stones, suggests

a. 67% at the site and 33% under procurement

b. 50% at the site and 50% under procurement

c. 33% at the site and 67% under procurement

d. none of these.

47. Bar charts are suitable for

a. minor works

b. major works

c. large projects

d. all the Above.

48. The salient feature of functional organisation is

a. separation of planning and design part

b. each individual maintains functional efficiency

c. work is properly planned and distributed

d. all the above.

49. Pick up the correct statement from the following:

a. Landing speed is directly proportional to the wing loading

b. Wing loading remaining constant, the take off distance is directly proportional to the powder loading

c. Neither (a) nor (b)

d. Both (a) and (b)

50. The bearing of the longest line of a wind rose is S 45° E, the bearing of the runway will be numbered

a. 315°

b. 13

c. 31

d. both (c) and (d)