***Sub Engineer***

1. Tacheometric formula for horizontal distances using horizontal sights can also suitable be employed for inclined sights through θ by multiplying

a. the constants by cos2 θ

b. the constants by cos θ

c. the constants by sin θ

d. the multiplying constant by cos2 θ and additive constant by cos θ.

2. In levelling operation

a. when the instrument is being shifted, the staff must not be moved

b. when the staff is being carried forward, the instrument must remain stationary

c. both (a) and (b)

d. neither (a) nor (b).

3. The properties of autogenous curve for automobiles are given by

a. true spiral

b. cubic parabola

c. Bernoulli's Lemniscate

d. clothoid spiral.

4. If vertical angles of inclined sights do not exceed 10° and non-verticality of the staff remains within 1°, stadia system of tacheometric observations are made on

a. staff normal

b. staff vertical

c. staff normal as well as vertical

d. none of these.

5. The most important constituent of varnish, is

a. drier

b. solvent

c. resin

d. all the above.

6. The commonly used base for iron and steel work, is

a. red lead

b. zinc white

c. white lead

d. titanium white.

7. Steel contains carbon approximately

a. 1.50% to 5.6%

b. 0.05% to 1.75%

c. 0.25%

d. none to these.

8. Cast iron is used for

a. structural works in beams

b. small sized water pipes

c. columns and struts

d. none to these.

9. A compound truss may be formed by connecting two simple rigid frames, by

a. two bars

b. three bars

c. three parallel bars

d. three bars intersecting at a point.

10. In a shaft, the shear stress is not directly proportional to

a. radius of the shaft

b. angle of twist

c. length of the shaft

d. modulus of rigidity.

11. A simply supported wooden beam 150 cm long and having a cross section 16 cm x 24 cm carries a concentrated load, at the centre. If the permissible stress ft = 75 kg/cm2 and fs = 10 kg/cm2 the safe load is

a. 3025 kg

b. 3050 kg

c. 3075 kg

d. 3100 kg.

12. The phenomenon of slow extension of materials having constant load, i.e. increasing with the time is called

a. creeping

b. yielding

c. breaking

d. none of these.

13. Mach number is the ratio of inertia force to

a. viscosity

b. surface tension

c. gravitational force

d. elasticity.

14. Shear stress is directly proportional to

a. the velocity

b. the shear strain

c. the viscosity

d. the velocity.

15. Poise is the unit of

a. viscosity

b. velocity gradient

c. mass density

d. kinematic viscous.

16. The total pressure force on a plane area is equal to the area multiplied by the intensity of pressure at its centroid, if

a. area is horizontal

b. area is vertical

c. area is inclined

d. all the above.

17. The angle between the directions of the failure and the major principal plane, is equal to

a. 90° + effective angle of shearing resistance

b. 90° + half of the angle of shearing resistance

c. 45° - half of the angle of shearing resistance

d. 45° + half of the angle of shearing resistance.

18. The consistency index of a soil is defined as the ratio of

a. liquid limit plus the natural water content to the plasticity index of the soil

b. liquid limit minus the natural water content to the plasticity index of the soil

c. natural water content of a soil minus plastic limit to the plasticity index of the soil

d. natural water content of a soil plus its plastic limit to the plasticity index of the soil.

19. If the unit weight of sand particles is 2.696 g/cc. and porosity in loose state is 44%, the critical hydraulic gradient for quick sand condition, is

a. 0.92

b. 0.93

c. 0.94

d. 0.95

20. Transporting and redepositing soils, is done by

a. glacier

b. gravity

c. wind

d. all the above.

21. For stairs spanning horizontally, the minimum waist provided is

a. 6 cm

b. 8 cm

c. 10 cm

d. 12 cm.

22. The maximum diameter of a bar used in a ribbed slab, is

a. 6 mm

b. 20 mm

c. 22 mm

d. 24 mm

23. The spacing of transverse reinforcement of column is decided by the following consideration.

a. The least lateral dimension of the column

b. Sixteen times the diameter of the smallest longitudinal reinforcing rods in the column

c. Forty-eight times the diameter of transverse reinforcement

d. All the above.

24. Thickened part of a flat slab over its supporting column, is technically known as

a. drop panel

b. capital

c. column head

d. none of these.

25. For providing a raft foundation, the following activities are involved ramming the foundation bedexcavation of the soil upto required depthlaying the reinforcement over the foundation bedcuring the cement concrete placed over reinforcementpouring the cement concrete over the reinforcement. The correct sequence is

a. 1, 2, 3, 4, 5

b. 5, 4, 3, 2, 1

c. 2, 1, 3, 5, 4

d. 3, 2, 5, 1, 4.

26. Pick up the correct statements from the following :

a. cracks appear on the plastered surface in the form of hair cracks

b. in brick work, the efflorescence is removed by applying a solution of zinc sulphate and water

c. excessive thermal variations in the backing or plaster causes the plaster to fall

d. all the above.

27. According to Water-Cement Ratio Law, the strength of workable plastic concrete

a. depends upon the amount of water used in the mix

b. does not depend upon the quality of cement mixed with aggregates

c. does not depend upon the quanity of cement mixed with aggregates

d. all the above.

28. Pick up the correct statement from the following:

a. According to the petrological characteristics, concrete aggregates are classified as heavy weight, normal weight and light weight

b. According to the shape of the particles, concrete aggregates are classified as rounded irregular, angular and flaky

c. According to the surface texture of the particles, the concrete aggregates are classified as glassy, smooth, granular, rough, crystalline, honey combed and porous

d. All the above.

29. The percentage of chlorine in fresh bleaching powder is roughly

a. 50 to 60

b. 30 to 35

c. 40 to 50

d. 20 to 25

30. The process of passing water through beds of granular materials, is called

a. screening

b. sedimentation

c. filtration

d. none of these.

31. In areas where rainy season is limited to a few months, the type of sewerage system recommended is

a. combined system

b. partially separate system

c. separate system

d. none of these.

32. 1000 kg of sewage contains

a. 0.112 kg in solution

b. 0.225 kg in solution

c. 0.450 kg in suspension

d. both (a) and (c) of above.

33. Pick up the correct statement from the following:

a. Gravity water is harmful to crops

b. Hygroscopic water remains attached to soil molecules by chemical bond

c. Capillary moisture held in the soil pores against gravity by surface tension, is utilised by plants

d. All the above.

34. When a canal flowing under pressure is carried below a natural drainage such that its F.S.L. does not touch the underside of the supporting structure, the structure so provided, is called

a. syphon

b. aqueduct

c. super passage

d. syphon-aqueduct.

35. If A is the area of the surface, x is the depth of its C.G. from the surface of the water and ω is the density of water, then

a. total pressure on the surface is equal to ω x

b. depth of the point at which total pressure acts is equal to its moment of inertia divided by A x

c. depth of the centre of pressure is 2/3H vertically below the surface

d. all the above.

36. The state of the soil when plants fail to extract sufficient water for their requirements, is

a. maximum saturated point

b. permanent wilting point

c. ultimate utilisation point

d. none of these.

37. The minimum cross fall of shoulders is kept

a. 1.0%

b. 1.5%

c. 2.5%

d. 3%

38. For the administration of road transport, a Motor Vehicle Act was enacted in

a. 1934

b. 1939

c. 1947

d. 1950

39. The minimum superelevation in rolling terrain in plains, is limited to

a. 5%

b. 6%

c. 7%

d. 10%

40. An Executive Engineer of roads, executes works under direct control of

a. Superintending Engineer

b. Secretary to the Govt

c. Chief Engineer

d. None of these.

41. A portion of an embankment having a uniform up-gradient 1 in 500 is circular with radius 1000 m of the centre line. It subtends 180° at the centre. If the height of the bank is 1 m at the lower end, and side slopes 2:1, the earth work involved.

a. 26, 000 m3

b. 26, 500 m3

c. 27, 000 m3

d. 27, 500 m3

42. The damp proof course (D.P.C.) is measured in

a. Cub.m

b. Sq m

c. Metres

d. None of these

43. The order of booking dimensions is

a. Length, breadth, height

b. Breadth, length, height

c. Height, breadth, length

d. None of these.

44. The height of the sink of wash basin above floor level is kept

a. 60 cm

b. 70 cm

c. 75 cm to 80 cm

d. 80 cm

45. Pick up the correct statement from the following:

a. If the float is positive and the activity is delayed by a period equal to its total float, the completion of project is not delayed

b. If the float of an activity is negative, delay in its performance is bound to delay the completion of project

c. If the float of an activity is zero, the activity is critical and any delay in its performance will delay the whole project

d. All the above.

46. Time and progress chart of a construction, is also known as

a. Gantt chart

b. Modified Mile stone chart

c. Critical path method chart

d. All the above.

47. Modular co-ordination of construction means proper

a. planning

b. designing

c. execution

d. all the above.

48. The difference between the time avail-to do a job and the time required to do the job, is known as

a. event

b. float

c. duration

d. constraint.

49. The air is blowing at a speed of 75 knots in the direction of the aircraft flying at a ground speed of 775 knots. The air speed of the aircraft, is

a. 775 knots

b. 75 knots

c. 850 knots

d. 675 knots

50. The length of a runway under standard atmospheric conditions is 1800 m. If the actual reduced level of the site is 1200 m, the design length of the runway is

a. 2360 m

b. 2460 m

c. 2560 m

d. 2660 m