***Sub Engineer***

1. The intercept of a staff

a. is maximum if the staff is held truly normal to the line of sight.

b. is minimum if the staff is held truly normal to the line of sight.

c. decreases if the staff is tilted away from normal

d. increases if the staff is tilted towards normal.

2. The probable error of the adjusted bearing at the middle is

a.  rn

b.  rn

c.  rn

d.  rn.

3. The angle of intersection of a curve is the angle between

a. back tangent and forward tangent

b. prolongation of back tangent and forward tangent

c. forward tangent and long chord

d. back tangent and long chord.

4. The ratio of the length of long chord and the tangent length of a circular curve of radius R deflecting through angle Δ, is

a. cos

b. tan

c. 2 sin

d. 2 cos .

5. Bulking of sand is caused due to

a. surface moisture

b. air voids

c. viscosity

d. clay contents

6. Pick up the non-inflammable plastic from the following:

a. Cellulose acetate plastics

b. Polyvinyl chloride plastics

c. Phenol formaldehyde plastic

d. Urea formaldehyde plastic.

7. The PVC doors and windows are preferred as they are

a. rot proof

b. termile proof

c. water proof

d. all of these.

8. Clay and silt content in a good brick earth must be at least

a. 50%

b. 40%

c. 30%

d. 25%

9. If a solid shaft (diameter 20 cm, length 400 cm, N = 0.8 x 105 N/mm2) when subjected to a twisting moment, produces maximum shear stress of 50 N/mm2, the angle of twist in radians, is

a. 0.002

b. 0.0025

c. 0.003

d. 0.005

10. Pick up the correct statement from the following:

a. Mcg = (M M2 + r2) where letters carry their usual meanings

b. Tcp = m2 + T2 where letters carry their usual meanings

c. The torque which when acting alone would produce maximum shear stress equal to the maximum shear stress caused by the combined bending and torsion, is called equivalent torque

d. All the above.

11. The value of Poisson's ratio always remains

a. greater than one

b. less than one

c. equal to one

d. none of these.

12. A column is said to be of medium size if its slenderness ratio is between

a. 32 and 120

b. 120 and 160

c. 160 and 180

d. 180 and 200

13. When the whole fluid mass rotates either due to fluid pressure or gravity or rotation previously imparted, the motion is known as

a. free vortex

b. forced vortex

c. non-potential vortex

d. rotational vortex.

14. The specific weight of water is 1000 kg/m3

a. at normal pressure of 760 mm

b. at 4°C temperature

c. at mean sea level

d. all the above.

15. For a most economical trapezoidal open channel, the half of the top width must be equal to

a. the bed width

b. one sloping side

c. the depth of flow

d. None of these.

16. Water flows through a convergent mouthpiece of diameter 4 cm at convergence under a head of 3 metres. If the maximum vacuum pressure is 9 metres of water, the maximum diameter of divergence, to aviod separation of flow, is

a. 4 cm

b. 6 cm

c. 2 cm

d. 23 cm.

17. The ultimate Settlement of a soil is directly proportional to:

a. compressive index

b. void ratio

c. both (a) and (b)

d. none of these.

18. Pick up the incorrect statement from the following:

a. Below the water table, the pore water may be static.

b. The hydrostatic pressure depends on the depth below the water level.

c. The attractive forces between the particles, caused due to negative pressure of water held above the water table is called soil suction.

d. None of these.

19. The ratio of the difference between the void ratio of the soil in its loosest state and its natural void ratio (e) to the difference between the void ratios in the loosest and fully dense state, is generally termed as

a. degree of density

b. relativity

c. density index

d. all the above.

20. Rankine's theory of active earth pressure assumes

a. ground surface is a plane which may be horizontal or inclined

b. back of the wall is vertical and smooth

c. wall yields about the base

d. all the above.

21. After prestressing process is completed, a loss of stress is due to

a. elastic shortening of concrete

b. creep of concrete

c. creep of steel

d. all the above.

22. An R.C.C. column is treated as long if its slenderness ratio is greater than

a. 35

b. 40

c. 50

d. 60

23. If the tendon is placed at an eccentricity e below the centroidal axis of the lon-gitudial axis of a rectangular beam (sectional modulus Z and stressed load P in tendon) the stress at the extreme top edge

a. is increased by

b. is increased by

c. is decreased by

d. remains unchanged.

24. The live load to be considered for an inaccessible roof, is

a. Nil

b. 75 kg/m2

c. 150 kg/cm2

d. 200 kg/m2

25. Pile foundations are suitable for

a. water logged soils

b. soft rocks

c. compact soils

d. multistoreyed buildings

26. In high mountaneous region, the type of roof generally recommended for buildings, is

a. shed type

b. gable type

c. gambrel type

d. mansord type.

27. ISI has specified full strength of concrete after

a. 14 days

b. 21 days

c. 28 days

d. none of these.

28. Bulking of sand is

a. mixing of different sizes of sand particles

b. mixing of lime with sand

c. maximum water with sand

d. swelling of sand when wetted.

29. Water supply system includes

a. digging a well for water

b. construction of dams

c. construction of canals

d. entire arrangement from source to distribution.

30. Pick up the correct statement from the following :

a. Head loss in smaller size pipes at equal velocities, is less

b. Cost of pumping is less in smaller size pipes

c. Cost of smaller pipes is comparatively more

d. None of these.

31. 1000 kg of sewage contains

a. 0.112 kg in solution

b. 0.225 kg in solution

c. 0.450 kg in suspension

d. both (a) and (c) of above.

32. Pick up the incorrect statement from the following :

a. Septic tanks are generally provided a detention period of 12 to 36 hours

b. Septic tanks are completely covered and high vent shafts are provided for the escape of foul gases

c. Septic tanks are generally classified as tanks which work on the principle of anaerobic decomposition

d. None of these.

33. The difference in level between the top of a bank and supply level in a canal, is called

a. berm

b. free board

c. height of bank

d. none of these.

34. If the straight sides of a triangular section of a lined canal with circular bottom of radius D, make an angle θ with horizontal, the hydraulic mean depth is

a. D/2

b. D/3

c. D/4

d. D/5

35. Canals constructed for draining off water from water logged areas, are known

a. drains

b. inundation canals

c. valley canals

d. contour canals

36. Effective precipitation for a crop may be defined as

a. total precipitation minus the loss due to evaporation

b. total precipitation minus the loss due to infiltration

c. total precipitation during the crop period

d. available water stored in soil within root zone of the crop.

37. Roadway width for a National highways and State highways (two-lanes) is

a. 12 m

b. 9 m

c. 9.5 m

d. 15 m

38. If brakes of vehicles are effective, the vehicle-running at 30 km/hour comes to a stop in

a. 10 metres

b. 12 metres

c. 15 metres

d. 18 metres

39. Pick up the correct statement from the following:

a. Curve length should be at least 150 metres for a deflection angle of 5 degress

b. For every degree decrease in the deflection angle, 30 metre length of curve to be increased

c. If the deflection angle is less than 1°, no curve is designed

d. All the above.

40. If the number of lanes on the carriageway of a road is more than two, the total width of lane ways is equal to 3.0 m

a. + 0.70 m

b. + 0.80 xn

c. + 0.90 m

d. + 1.50 m

41. The following item of earth work is not measured separately.

a. Site clearence

b. dead men

c. Steps in deep exavaction

d. All the above.

42. The excavation exceeding 1.5 m in width and 10 sq.m in plan area with a depth not exceeding 30 cm, is termed as

a. Excavation

b. Surface dressing

c. Cutting

d. Surface excavation.

43. While preparing a detailed estimate

a. Dimension should be measured correct to 0.01 m

b. Area should be measured correct to 0.01 sqm

c. Volume should be measured correct to O.Olcum

d. All the above.

44. The diameter of a domestic sewer pipe laid at gradient 1 in 100 is recommended

a. 100 mm

b. 150 mm

c. 200 mm

d. 175 mm

45. PERT is

a. limited of event oriented diagrams

b. used for research and development projects

c. based on three time estimates for activities linking up two events

d. all the above.

46. The performance of a specific task in CPM, is known

a. Dummy

b. Event

c. Activity

d. Contract.

47. A critical ratio scheduling

a. determines the status of each activity

b. adjusts automatically changes in activity progress

c. is a dynamic system

d. none of these.

48. Pre-tender stage requires

a. selection of site

b. formalisation of alignment of work

c. formalisation of designs and preparation of estimate

d. all the above.

49. Airport elevation is the reduced level above M.S.L. of

a. control tower

b. highest point of the landing area

c. lowest point of the landing area

d. none of these.

50. From the end of an instrumental runway, the approach surface rises outwards

a. 1 in 20

b. 1 in 30

c. 1 in 40

d. 1 in 50