***Sub Engineer***

1. The apparent error on reversal is

a. equal to the actual error

b. twice the actual error

c. thrice the actual error

d. none of these.

2. Determining the difference in elevation between two points on the surface of the earth, is known as

a. levelling

b. simple levelling

c. differential levelling

d. longitudinal levelling.

3. The latitude of a traverse leg is obtained by multiplying its length by

a. tangent of its reduced bearing

b. sign of its reduced bearing

c. cosine of its reduced bearing

d. cosecant of its reduced bearing.

4. If the whole circle bearing of a line is 270°, its reduced bearing is

a. N 90° W

b. S 90° W

c. W 90°

d. 90° W.

5. Pick up the constituent of good brick earth whose excess causes the raw bricks shrink and warp during drying and burning, from the following:

a. Alumina

b. Lime

c. Ironoxide

d. Magnesia.

6. Plywood is obtained by gluing wooden sheets at

a. 100 to 150 N/cm2

b. 100 to 130°C

c. both (a) and (b)

d. Neither (a) nor (b).

7. Pozzolana (or surkhi) is used in lime

a. to impart hydraulicity

b. to prevent shrinkage

c. to decrease the cost of construction

d. to decrease the setting time.

8. The cast iron when heated to red heat with powdered red haemetite in an oven for increasing its toughness, is converted to

a. grey cast iron

b. white cast iron

c. mottled cast iron

d. toughed cast iron.

9. For beams of uniform strength, if depth is constant,

a. width b ∝ M

b. width b ∝ M

c. width b ∝ 3 M

d. width b ∝

10. The yield moment of a cross section is defined as the moment that will just produce the yield stress in

a. the outer most fibre of the section

b. the inner most fibre of the section

c. the neutral fibre of the section

d. the fibre everywhere

11. An arch with three hinges, is a structure

a. statically determinate

b. statically indeterminate

c. geometrically unstable

d. structurally sound but indeterminate

12. The cross sections of the beams of equal length are a circle and a square whose permissible bending stress are same under same maximum bending. The ratio of their flexural weights is,

a. 1.118

b. 1.338

c. 1.228

d. 1.108

13. The hydrostatic force acts through

a. centre of pressure

b. centre of top edge

c. centre of bottom edge

d. metacentre.

14. For most economical rectangular section of a channel, the depth is kept

a. three times the hydraulic radius

b. half the width

c. hydraulic mean depth

d. none of these.

15. The pressure less than atmospheric , pressure, is known

a. suction pressure

b. vacuum pressure

c. negative gauge pressure

d. all the above.

16. For the flow of liquid from an open ended tube (or nozzle) leading to the formation of spray of liquid drops, the number generally applied, is

a. Froude number

b. Weber number

c. Reynold number

d. Mach number.

17. Pick up the correct statement from the following:

a. The freezing of water is accompained by a volume increase of 9%.

b. Below freezing point, higher soil suction develops.

c. The magnitude of frost heave decreases as the degree of saturation of soil decreases

d. All the above.

18. The critical exist gradient of seepage water in soils, is

a. inversely proportional to the specific gravity

b. directly proportional to the specific gravity

c. inversely proportional to the voids ratio

d. none of these.

19. The consolidation time for soils

a. decreases with increasing permeability

b. increases rapidly with increasing size of soil mass

c. is independent of the magnitude of the stress change.

d. All the above.

20. Pick up the correct statement from the following:

a. At low value of water content most soils tend to be stiff.

b. At high water content, the dry density decreases with an increase of water content.

c. An airless soil will have maximum possible value of dry density for the given water content

d. All the above.

21. [A + (m - 1)ASC] known as equivalent concrete area of R.C.C. is given by

a. modular ratio method

b. load factor method

c. ultimate load method

d. none of these.

22. An R.C.C. column is treated as short column if its slenderness ratio is less than

a. 35

b. 40

c. 50

d. 60

23. An R.C.C. roof slab is designed as a two way slab if

a. it supports live loads in both directions

b. the ratio of spans in two directions is less than 2

c. the slab is continuous over two supports

d. the slab is discontinuous at edges.

24. Minimum spacing between horizontal parallel reinforcement of the same size should not be less than

a. one diameter

b. 2.5 diameters

c. 3 diameters

d. 3.5 diameters

25. A pointed arch which forms isosceles or equilateral triangle, is generally known as

a. three centred arch

b. two centred arch

c. Lancet arch

d. Bull's eye arch.

26. Raft foundation are generally preferred to when the area required for individual footing, is more than

a. 25% to total area

b. 30% of total area

c. 40% to total area

d. 50% of total area.

27. Pick up the incorrect statement from the following:

a. The process of changing cement paste into hard mass, is known as setting of cement

b. The phenomenon by virtue of which cement does not allow transmission of sound, is known as soundness of cement

c. The heat generated during chemical reaction of cement with water, is known as heat of hydration

d. None of these.

28. The mixture of different ingredients of cement, is burnt at

a. 1200°C

b. 1400°C

c. 1600°C

d. 1800°C

29. Disappearance of pink colour of water of a well due of KMnO4 indicates that water contains

a. alkalinity

b. turbidity

c. organic matter

d. all the above.

30. The most commonly used chemical for dechlorination of water, is

a. sodium bisulphate

b. sodium sulphite

c. sulphur-dioxide

d. all the above.

31. Rate of flow of sewage is generally assumed

a. more than the rate of water supply

b. equal to the rate of water supply

c. less than the rate of water supply

d. at 150 litres per capita.

32. If the peak discharge of a storm water drain (S.W. Drain) is e.xpected to exceed 150 cumecs, the free board to be provided, is

a. 100 cm

b. 90 cm

c. 80 cm

d. 50 cm.

33. Canals constructed for draining off water from water logged areas, are known

a. drains

b. inundation canals

c. valley canals

d. contour canals

34. In a syphon aqueduct

a. drainage passes over the canal and F.S.L. of the canal is below the bottom of the drainage trough

b. drainage passes over the canal and F.S.L. of the canal is above the bottom of the drainage trough

c. canal passes over the drainage and H.F.L. of the drainage is above the bottom of the canal trough

d. canal passes over the drainage and H.F.L. of the drainage is below the bottom of the canal trough.

35. The intensity of irrigation means

a. percentage of culturable commanded area to be irrigated annually

b. percentage of gross commanded area to be irrigated annually

c. percentage of the mean of culturable commanded area and the gross commanded area to be irrigated annually

d. total depth of water supplied by the number of waterings.

36. The saturation line is the line up to which banks get saturated after the canal runs for some time. The saturation gradient in ordinary loam soil, is generally

a. 3 : 1

b. 4 : 1

c. 5 : 1

d. 6 : 1

37. The type of curves generally provided on highways, is

a. critical curve

b. transition curve

c. vertical curve

d. all the above.

38. If the elevations along a road increase, the slope of the road along the longitudinal direction, is known as

a. gradient

b. grade

c. positive grade

d. negative grade.

39. The width of the right of way in urban area, is kept between 24 m to 60 m for

a. National Highways

b. State Highways

c. Both (a) and (b)

d. None of these.

40. For a 6.6 m wide two lane pavement, berms are provided on either side having a width of

a. 1.25 m

b. 1.50 m

c. 1.75 m

d. 2.00 m

41. The most reliable estiamte is

a. Detailed estimate

b. Preliminary estimate

c. Plinth area estimate

d. Cube rate estimate

42. The ground surface slopes 1 in 50 along a proposed railway embankment 150 m in length. The height of the embankment at zero chainage is 0.5 m, the width is 11 m and side slopes 2:1. If the falling gradient of the embankment is 1 in 150, the quantity of the earthwork calculated by prismoidal formula, is

a. 3250 m3

b. 3225 m3

c. 3275 m3

d. 3300 m3

43. The plinth area of a building not includes

a. area of stair cover

b. internal shaft for sanitary installations up to 2 sq m. in area

c. lift and wall including landing

d. area of cantilevered porch.

44. The floor area includes the area of the balcony up to

a. 100%

b. 75%

c. 50%

d. 25%

45. If the total float and duration of an activity are 5 and 10 days respectively, the particular activity can be

a. started 5 days later

b. completed 5 days later

c. performed at slower rate in 15 days

d. all the above.

46. The technique for establishing and maintaining priorities among the various jobs of a project, is known

a. Event flow scheduling technique

b. Critical ratio scheduling

c. Slotting technique for scheduling

d. Short interval scheduling.

47. Pick up the correct statement from the following:

a. Backward pass is used for calculating the latest occurence time

b. Maximum value of earliest expected time is used if there are more than one value of any event

c. Minimum value of latest occurrence time is used if there are more than one value of any event

d. All the above.

48. Various activities of a project, are shown on bar charts by

a. vertical lines

b. horizontal lines

c. dots

d. crosses.

49. The lift off distance is the distance along the centre of the runway between the starting point and

a. end of the runway

b. end of stopway

c. point where air craft becomes air borne

d. point where air craft attains a height of 10.7 m

50. According to the International Civil Aviation Organisation (I.C.A.O.) the strength of runway pavements, have been coded by

a. Last Seven English alphabets

b. First Seven English alphabets

c. First seven numbers

d. None of these.