***Sub Engineer***

1. Ranging is an operation of

a. reconnaissance

b. judging the distance

c. determination of slope

d. establishing intermediate points between terminals.

2. For orientation of a plane table with three points A, B and C, Bessel's drill is

a. Align b through a and draw a ray towards c, align a through b and draw a ray towards c, finally align c through the point of intersection of the previously drawn rays

b. Align c through a and draw a ray towards b, align a through c and draw a ray towards b, finally align b through the point of intersection of the previously drawn rays

c. Align c through b and draw a ray towards a, align b through c and draw a ray towards a, finally align a, through the point of intersection of the previously, drawn rays

d. In the first two steps any two of the points may be used and a ray drawn towards the third point, which is sighted through the point of intersection of previously drawn rays in the final step.

3. If the angular measurements of a traverse are more precise than its linear measurements, balancing of the traverse, is done by

a. Bowditch's rule

b. Transit rule

c. Empirical rule

d. all of the above.

4. Pick up the correct statement from the following :

a. The vertical distance between a maximum ordinate and the next forward maximum ordinate represents the whole volume of the embankment

b. The vertical distance between a minimum ordinate and the next forward maximum ordinate represents the whole volume of a cutting

c. The area enclosed by a loop of the curve and balancing line, measures the haul in that direction.

d. all the above.

5. Bituminous fells are used for

a. covering A.C. sheets

b. covering sloping roofs

c. D.P.C.

d. none to these.

6. Linseed oil is used in paints as

a. thinner

b. vehicle

c. base

d. drier.

7. Bessemer process is used for the manufacture of

a. Pig iron

b. cast iron

c. Wrought iron

d. Steel.

8. Which one of the following polymers is obtained from condensation polymerization?

a. phenol formaldehyde

b. carbamide

c. melamine-formaldehyde

d. all of these.

9. In plastic analysis, the shape factor for a circular section, is

a. 1.5

b. 1.6

c. 1.7

d. 1.75

10. For calculating the allowable stress of long columns  is the empirical formula, known as

a. Straight line formula

b. Parabolic formula

c. Perry's formula

d. Rankine's formula.

11. The reaction at support A of the beam shown in below figure, is

a. zero

b. 5 T

c. 10 T

d. 1 T

12. A rectangular log of wood is floating in water with a load of 100 N at its centre. The maximum shear force in the wooden log is

a. 50 N at each end

b. 50 N at the centre

c. 100 N at the centre

d. none of these.

13. The momentum correction factor (β) for the viscous flow through a circular pipe is

a. 1.33

b. 1.50

c. 1.66

d. 2.00

14. An ideal flow of a liquid obeys

a. Continuity equation

b. Newton's law of viscosity

c. Newton's second law of motion

d. dynamic viscosity law,

15. Chezy's formula is used to determine

a. head loss due to friction in pipe

b. velocity of flow in pipe

c. velocity of flow in open channels

d. none of these.

16. A steady uniform flow is through

a. a long pipe at constant rate

b. an expanding tube at constant rate

c. an expanding tube at increasing rate

d. a long pipe at increasing rate.

17. The fundamental equation of air content (ac), degree of saturation (Sr) and void ratio (e), is

a. ac =

b. ac =

c. ac =

d. ac = 1 + e/Sr

18. The clay soil mainly consists of

a. Montomorillonite

b. Illites

c. Vermiculite

d. All the above.

19. If the coefficient of the active pressure Ka is 1/3, the coefficient of passive pressure Kp, is

a. 2/3

b. 1

c. 3/2

d. 3

20. Negative skin friction on piles

a. is caused due to relative settlement of the soil

b. is caused in soft clays

c. decreases the pile capacity

d. all of the above.

21. The transverse reinforcements provided at right angles to the main reinforcement

a. distribute the load

b. resist the temperature stresses

c. resist the shrinkage stress

d. all the above.

22. If P kg/m2 is the upward pressure on the slab of a plain concrete footing whose projection on either side of the wall is a cm, the depth of foundation D is given by

a. D = 0.00775 aP

b. D = 0.0775 aP

c. D = 0.07775 aP

d. D = 0.775 Pa

23. The maximum ratio of span to depth of a slab simply supported and spanning in two directions, is

a. 30

b. 35

c. 40

d. 15

24. Minimum spacing between horizontal parallel reinforcement of different sizes, should not be less than

a. one diameter of thicker bar

b. sum of the diameters of ihinner and thicker bars

c. twice the diameter of thinner bar

d. none of these.

25. The rock formed from the solidification of molten matter (magma) is called :

a. sedimentary rock

b. metamorphic rock

c. igneous rock

d. none of the above.

26. The opening provided in sloping roof with its top parallel to the roof surface, is called

a. dormer window

b. sky light window

c. lantern window

d. louvered window.

27. The process of proper and accurate measurement of concrete ingredients for uniformity of proportion, is known

a. curing

b. mixing

c. batching

d. none of these.

28. Vicat apparatus is used for

a. consistency test

b. test for setting time

c. test for tensile strength

d. none of these.

29. The bacterias which may survive with or without free oxygen, are called

a. Aerobic bacterias

b. Anaerobic bacterias

c. Faculative bacteria's

d. None of these.

30. Pressure relief valves are provided in water mains

a. to reduce the pressure

b. at low points

c. upstream of sluice

d. all the above.

31. The small sewers are cleaned by

a. flushing

b. cane rodding

c. wooden pills

d. none of these.

32. Stone ware pipes are

a. highly impervious

b. hydraulically efficient because of their smooth interior surface

c. specially suited to pressure pipes

d. none of these.

33. The consumptive use of water for a crop

a. is measured as the volume of water per unit area

b. is measured as depth of water on irrigated area

c. may be supplied partly by precipitation and partly by irrigation

d. all the above.

34. Retrogression of the bed level of a river downstream a weir, occurs due to

a. heavy impact of water

b. increase of the bed level

c. less percentage of silt

d. soft soil strata.

35. If D is the depth of water upstream of the throat above its sill, B is the width of the throat, to achieve critical flow in an open venturi flume, the theoretical maximum flow Q, is

a. Q = 1.71 BD

b. Q = 1.71 BD3/2

c. Q = 1/71 BD2/3

d. none of these.

36. Pick up the correct statement from the following:

a. Static level of water in an open well indicates the position of the water table

b. Saturated pores of a soil in the capillary zone above water table have negative pressure

c. At water table, hydrostatic pressure is zero

d. All the above.

37. The minimum value of camber provided for thin bituminous surface hill roads, is

a. 2.2%

b. 2.5%

c. 3.0%

d. 3.5%

38. From the point of tangency before an intersection, the route markers are fixed at a distance of

a. 20 m to 35 m

b. 40 m to 50 m

c. 50 m to 75 m

d. 100 m to 150

39. If R is the radius of a main circular curve, Δ is the angle of deflection and α is the polar deflection angle of any point, its radial distance is

a. 3 R sin Δ/3 sin 2α

b. 3 R sin Δ/2 sin 3α

c. 3 R sin Δ/3 sin α/2

d. 3 R sin 2Δ sin α

40. The most commonly adopted method to provide super-elevation on roads, is by pivoting the road surface about

a. outer edge so that the inner edge is lowered

b. crown so that outer edge is raised and inner edge is lowered

c. inner edge so that outer edge is raised

d. none of these.

41. Brick walls are measured in sq. m if the thickness of the wall is

a. 10 cm

b. 15 cm

c. 20 cm

d. None of these.

42. Carpet area does not include the area of

a. verandah, corridor and passage

b. bath room and lavatory

c. kitchen and pantry

d. All the above.

43. The detention period in a septic tank is assumed

a. 20 minutes

b. 25 minutes

c. 30 minutes

d. 40 minutes

44. The main factor to be considered while preparing a detailed estimate, is

a. Availability of materials

b. Transportation of materials

c. Location of site and local labour charges

d. All the above.

45. The first stage of a construction, is

a. survey of the site

b. initiation of proposal

c. preparation of tender

d. allotment of funds.

46. Railway projects are treated as

a. light construction

b. heavy construction

c. industrial construction

d. none of these.

47. Which one of the following represents an event ?

a. concrete cured

b. fixing of door

c. plastering of walls

d. selecting sites

48. Modular co-ordination of construction means proper

a. planning

b. designing

c. execution

d. all the above.

49. If the width of the approach area near the runway end is 150 m, the width of the approach area at a distance of 3 kilometres from runway end will be

a. 1500 m

b. 1200 m

c. 1000 m

d. 800 m

50. Pick up the correct statement from the following:

a. The wind data of an air port is depicted in the form of a chart known as wind rose

b. The landing and take off of the air craft is made against the wind direction

c. The product of wind loading and power loading is called air craft rating

d. All the above.