***Sub Engineer***

1. The whole circle bearing of a line is 290°. Its reduced bearing is

a. N 20° E

b. N 20° W

c. N 70° W

d. S 70° E

2. In levelling operation

a. when the instrument is being shifted, the staff must not be moved

b. when the staff is being carried forward, the instrument must remain stationary

c. both (a) and (b)

d. neither (a) nor (b).

3. In chain surveying tie lines are primarily provided

a. to check the accuracy of the survey

b. to take offsets for detail survey

c. to avoid long offsets from chain lines

d. to increase the number of chain lines.

4. Which one of the following procedures for getting accurate orientation is the most distinctive feature of the art of plane tabling

a. radiation

b. intersection

c. traversing

d. resection.

5. If P is the percentage of water required for normal consistency, water to be added for determination of initial setting time, is

a. 0.75 P

b. 0.80 P

c. 0.85 P

d. 0.90 P.

6. Pick up the correct statement from the following:

a. Lime is available by dissolving calcium carbonate in water

b. Lime is available by calcining calcium carbonate at 900°C

c. Lime is nothing but calcium chloride

d. None of these.

7. The cracks which extend from bark towards the sap wood in the cross section of a tree, are called

a. radial shakes

b. star shakes

c. heart shakes

d. cup shakes.

8. Linseed oil is used in paints as

a. thinner

b. vehicle

c. base

d. drier.

9. The point of contraflexure is the point where

a. B.M. changes sign

b. B.M. is maximum

c. B.M. is minimum

d. S.F. is zero.

10. Pick up the correct statement from the following:

a. The pitch of a close coil spring, is very small

b. The angle made by the coil with horizontal, is called the angle of helix

c. In the open coil helical spring, the angle of helix is comparatively large

d. All the above.

11. The reaction at the supports will be vertical to the plane of the support if the frame structure rests on

a. roller supports

b. free supports

c. hinged supports

d. all the above.

12. The maximum compressive stress at the top of a beam is 1600 kg/cm2 and the corresponding tensile stress at its bottom is 400 kg/cm2 . If the depth of the beam is 10 cm, the neutral axis from the top, is

a. 4 cm

b. 6 cm

c. 8 cm

d. 10 cm.

13. In a two dimensional flow if the components of the velocity are u = ax ; v = by, the point where no motion occurs, is known as

a. neutral point

b. stagnation point

c. stationary point

d. none of these.

14. For a long pipe, the head loss

a. at the entrance is ignored

b. at the outlet is ignored

c. at the entrance and outlet both are ignored

d. due to friction is ignored.

15. If the forces are due to inertia and gravity, and frictional resistance plays only a minor role, the design of the channels is made by comparing

a. Reynold number

b. Froude number

c. Weber number

d. Mach number.

16. To avoid the tendency of separation of liquid flow, the most suitable ratio of the diameters of the throat and the pipe, is

a. 1/4 to 1/8

b. 1/3 to 1/2

c. 1/2 to 3/4.

d. none of these.

17. The minimum water content at which the soil retains its liquid state and also possesses a small shearing strength against flowing, is known

a. liquid limit

b. plastic limit

c. shrinkage limit

d. permeability limit.

18. A flow net may be utilised for the determination of

a. seepage

b. hydrostatic pressure

c. seepage pressure

d. all the above.

19. 260 g of wet soil was taken in a pycnometer jar of weight 400 g in order to find the moisture content in the soil, with specific gravity of soil particles 2.75. The weight of soil and remaining water filled in pycnometer without air bubbles was 1415 g and the weight of pycnometer filled with water alone was 1275 g. The moisture content in the soil is

a. 24.2%

b. 18.2%

c. 53.8%

d. none of these.

20. The compression index of a soil

a. decreases with an increase in the liquid limit

b. increases with an increase in the liquid limit

c. decreases with an increase in the plastic limit

d. is not related with plastic limit.

21. If the shear stress in a R.C.C. beam is

a. equal or less than 5 kg/cm2, no shear reinforcement is provided

b. greater than 4 kg/cm2, but less than 20 kg/cm2, shear reinforcement is provided

c. greater than 20 kg/cm2, the size of the section is changed

d. all the above.

22. If C is creep coefficient, f is original prestress in concrete, m is modular ratio, E is Young's modulus of steel and e is shrinkage strain, the combined effect of creep and shrinkage is:

a. (1 - C)mf - eE

b. (C - 1)mf + eE

c. (C - 1)mf - eE

d. (1 - C)mf + eE

23. In a prestressed beam carrying an external load W with a bent tendon is having angle of inclination θ and prestressed load P. The net downward load at the centre is

a. W - P cos θ

b. W - P sin θ

c.  W - 2P sin θ

d. W + 2P sin θ

24. If the bearing capacity of soil is 10 tonnes/cm2 and the projection of plain concrete footing from walls, is a cm, the depth D of footing is

a. D = 0.0775 a

b. D = 0.775 a

c. D = 0.775a

d. D = 0.775 a2

25. The rock formed from the solidification of molten matter (magma) is called :

a. sedimentary rock

b. metamorphic rock

c. igneous rock

d. none of the above.

26. The least bearing capacity of soil is that of

a. moist clay

b. soft rock

c. laminated

d. coarse sandy soil.

27. The following proportion of the ingredients of concrete mix, is not in conformation to arbitrary method of proportioning

a. 1 : 2 : 4

b. 1 : 3 : 6

c. 1 : 2 : 8

d. 1 : 4 : 10

28. Pick up the correct statement from the following:

a. Water cement paste hardens due to hydration

b. During hardening cement binds the aggregates together

c. Cement provides strength, durability and water tightness to the concrete

d. All the above.

29. Chlorination of water does not remove

a. ammonia content

b. B.O.D.

c. organic matter content

d. dissolved oxygen.

30. The gaseous form of chlorine gets converted into liquid form when subjected to a pressure of

a. 6 kg/cm2

b. 7 kg/cm2

c. 8 kg/cm2

d. 10 kg/cm2.

31. Which of the following pumps in used to pump sewage solids with liquid sewage without clogging the pump is

a. centrifugal pump

b. pneumatic ejector

c. reciprocating pump

d. none of these.

32. In case of sewer lines

a. water test is carried out to check water tightness of the joints

b. test for straightness is carried out with the help of a lamp and mirror

c. obstruction test is carried out with the. help of smooth ball of diameter 13 mm

d. all the above.

33. For the design of major hydraulic structures on the canals, the method generally preferred to, is based on

a. Bligh's theory

b. Electrical analogy method

c. The relaxation method

d. Khosla's method of independent variables.

34. The main function of a diversion head works of a canal from a river, is

a. to control floods

b. to store water

c. to raise water level

d. all the above.

35. For a given discharge in a channel, Blench curves give the relationship between the loss of head (HL) and

a. specific energy up-stream

b. specific energy down-stream

c. critical depth of water down-stream

d. depth of water down-stream

36. The saturation line is the line up to which banks get saturated after the canal runs for some time. The saturation gradient in ordinary loam soil, is generally

a. 3 : 1

b. 4 : 1

c. 5 : 1

d. 6 : 1

37. Minimum permissible speed on high speed roads, is decided on the basis of

a. 15 percentile cumulative frequency

b. 20 percentile cumulative frequency

c. 30 percentile cumulative frequency

d. 40 percentile cumulative frequency.

38. The difference in gradients after full super-elevation and the initial alignment of a road, is known as

a. rising gradient

b. compensated gradient

c. differential gradient

d. none of these.

39. The head light of vehicles should be such that its lower beam illuminates objects at

a. 20 m

b. 30 m

c. 40 m

d. 50 m

40. Volume of traffic which is due to improvement carried out in adjacent area, is known as

a. development traffic

b. generated traffic growth

c. normal traffic growth

d. current traffic.

41. Pick up the correct statement from the following:

a. In order to check up the average depth of excavation, 'Dead mans' are left at the mid-widths of borrow pits

b. The earthwork calculation in excavation is made from the difference in levels obtained with a level

c. The earth work in excavation to form the road embankment includes the formation of correct profile and depositing the soil in layers

d. All the above.

42. If the formation level of a highway has a uniform gradient for a particular length, and the ground is also having a longitudinal slope, the earthwork may be calculated by

a. Mid-section formula

b. Trapezoidal formula

c. Prismoidal formula

d. All the above.

43. The brick work is not measured in cu m in case of

a. One or more than one brick wall

b. Brick work in arches

c. Reinforced brick work

d. Half brick wall.

44. The floor area includes the area of the balcony up to

a. 100%

b. 75%

c. 50%

d. 25%

45. Pick up the incorrect statement from the following:

a. The tail of the arrow indicates the start of the activity

b. The head of the arrow indicates the end of the activity

c. The arrows are drawn to scale from left to right

d. Each activity consumes a given time.

46. Frederick W. Taylor introduced a system of working known as

a. line organisation

b. line and staff organisation

c. functional organisation

d. effective organisation.

47. Completion of an activity on CPM network diagram, is generally known

a. Event

b. Node

c. Connector

d. All the above.

48. Critical path method

a. provides a realistic approach to daily problems

b. avoids delays which are very common in bar charts

c. was invented by Morgan R. Walker of Dupot and James E. Kalley or Remington U.S.A. in 1957

d. All the above.

49. Effective length of a runway is the distance between

a. ends of the runway

b. point of intersection of the obstruction clearance line and the extended plane of the runway surface, and the other end of the runway

c. point of intersection of the glide path and the extended plane of the runway surface and the other end of the runway

d. ends of the clear way on either side.

50. For the taxiways, the following statement is true

a. The maximum longitudinal grade is 3%

b. The permissible rate of change of grade is 1%

c. The permissible transverse grade is 1.5%

d. All the above.