***Sub Engineer***

1. In levelling operation,

a. The first sight on any change point is a back sight

b. The second sight on any change point is a fore sight

c. The line commences with a fore sight and closes with a back sight

d. The line commences with a back sight and closes with a foresight.

2. If the rate of gain of radial acceleration is 0.3 m per sec3 and full centrifugal ratio is developed. On the curve the ratio of the length of the transition curve of same radius on road and railway, is

a. 2.828

b. 3.828

c. 1.828

d. 0.828.

3. The theodolites used for making tacheometric observations by optical wedge system, are

a. provided with stadia hairs in front of eye piece

b. not provided with stadia hairs at all

c. fitted with a glass wedge inside the telescope

d. fitted with a glass wedge in front of telescope.

4. The surface of zero elevation around the earth, which is slightly irregular and curved, is known as

a. mean sea level

b. geoid surface

c. level surface

d. horizontal surface.

5. In order of increasing percentage of silica, the correct sequence is

a. sandy clay, calcareous clay, pure clay

b. calcareous clay, pure clay, sandy clay

c. pure clay, sandy clay, calcareous clay

d. None of these.

6. Pick up the correct statement from the following:

a. The silica in excess makes the cement stronger but its setting time also increases

b. The excess amount of alumina weakens the cement

c. The addition of gypsum increases the initial setting time of cement

d. All the above.

7. A bull nose brick is not used for

a. rounding off sharp corners

b. pillars

c. decoration purpose

d. arches.

8. The presence of original rounded surface on the manufactured piece of timber, is called

a. Wane

b. Torn grain

c. Diagonal grain

d. Chipmark.

9. Keeping the depth d constant, the width of a cantilever of length l of uniform strength loaded with a uniformly distributed load w varies from zero at the free end and

a.  at the fixed end

b.  at the fixed end

c.  at the fixed end

d.  at the fixed end

10. Pick up the incorrect statement from the following : The torsional resistance of a shaft is directly proportional to

a. modulus of rigidity

b. angle of twist

c. reciprocal of the length of the shaft

d. moment of inertia of the shaft section.

11. A simply supported wooden beam 150 cm long and having a cross section 16 cm x 24 cm carries a concentrated load, at the centre. If the permissible stress ft = 75 kg/cm2 and fs = 10 kg/cm2 the safe load is

a. 3025 kg

b. 3050 kg

c. 3075 kg

d. 3100 kg.

12. The ratio of strengths of solid to hollow shafts, both having outside diameter D and hollow having inside diameter D/2, in torsion, is

a. 1/2

b. 1/16

c. 15/16

d. 3/8

13. If total head available at the inlet of pipe and f1 is the loss of head due to friction in the pipe, the maximum efficiency of transmission of power (ηmax) is

a. 1/2

b. 2/3

c. 3/4

d. 4/5

14. The ratio of the inertia and gravitational force acting in any flow, ignoring other forces, is called

a. Euler number

b. Frode number

c. Reynold number

d. Weber number.

15. Discharge through a totally submerged orifice, is directly proportional to

a. square root of the difference in elevation of water surface

b. square root of the opening

c. reciprocal of the area of the opening

d. none of these.

16. A non-uniform steady flow is through

a. a long tube at a decreasing rate

b. an expanding tube at constant rate

c. an expanding tube at increasing rate

d. a long pipe at increasing rate.

17. Water formed transported soil is

a. alluvial

b. marine

c. lacustrine

d. loess.

18. The Mohr's straight theory is based on the following fact :

a. Material fails essentially by shear

b. Ultimate strength of the material is determined by the stress in the plane of slip

c. Failure criterion is independent of the intermediate principal stress

d. All the above.

19. Pick up the cohesive soil from the following:

a. Red earth

b. Clay

c. Black cotton soil

d. Compacted ground.

20. Pick up the correct definition from the following:

a. The lateral pressure exerted by the soil when the retaining wall moves away from the back fill, is generally known as active earth pressure of the soil

b. The lateral pressure exerted by the soil when the retaining wail moves towards the soil, is generally known as 'Passive earth pressure of the soil'

c. The lateral pressure exerted by the soil when the retaining wall has no movement relative to the back fill, is known as 'earth pressure at rest of the soil'

d. All the above.

21. By over-reinforcing a beam, the moment of resistance can be increased not more than

a. 10%

b. 15%

c. 20%

d. 25%

22. The maximum area of tension reinforcement in beams shall not exceed

a. 0.15%

b. 1.5%

c. 4%

d. 1%

23. Top bars are extended to the projecting parts of the combined footing of two columns L distance apart for a distance of

a. 0.1 L from the outer edge of column

b. 0.1 L from the centre edge of column

c. half the distance of projection

d. one-fourth the distance of projection.

24. In a combined footing for two columns carrying unequal loads, the maximum hogging bending moment occurs at

a. more loaded column

b. a point equidistant from either column

c. a point of the maximum shear force

d. a point of zero shear force.

25. In grillage foundations a minimum 15 cm cover is provided on

a. upper flange of top tier

b. lower beam of lower tier

c. ends of external beams

d. none to these.

26. For plastering the exposed brick walls, the cement sand mortar should be

a. 1:3

b. 1:4

c. 1:6

d. 1:8.

27. Concrete containing

a. silicious aggregates, has higher co-efficient of expansion

b. igneous aggregates, has intermediate coefficient of expansion

c. lime stones, has lowest co-efficient of expansion

d. All the above.

28. Specified compressive strengh of concrete is obtained from cube tests at the end of

a. 7 days

b. 14 days

c. 21 days

d. 28 days.

29. An ideal sand for filters should be

a. free from dirt and other impurities

b. uniform in nature and size

c. hard and resistant

d. all the above.

30. The four major water supply distribution systems, are

a. dead end, tree, grid iron and reticulation

b. dead end, tree, grid iron and circular

c. tree, grid iron, ring and radial

d. tree, reticulation, circular and ring.

31. The self-cleansing velocity of water flowing through pipe lines, is

a. 2 metres/sec

b. 1 metre/sec

c. 0.5 metre/sec

d. 0.25 metre/sec.

32. If the depletion of oxygen is found to be 5 ppm after incubating a 2.5% solution of sewage sample for 5 days at 21°C, B.O.D. of the sewage is

a. 100 ppm

b. 150 ppm

c. 200 ppm

d. 250 ppm.

33. Pick up the incorrect statement from the following

a. A bank in the cross-section of a canal is provided on the right

b. A bank in the cross-section of a canal is provided on the left

c. A 'dowla' in the cross-section of a canal is provided on the left

d. None of these.

34. Pick up the incorrect statement from the following. Culturable commanded area is the gross area of an irrigation canal system less

a. populated area

b. alkaline area

c. forest area

d. fallow land.

35. In rigid module, the discharge

a. is independent of water levels in the distributary and water course

b. depends upon the water level in distributary

c. depends upon the water level in the water course

d. depends upon the water levels of both, i.e. distributary and water course

36. When a canal is carried over a natural drainage, the structure provided, is known as

a. syphon

b. aqueduct

c. super passage

d. syphon-aqueduct.

37. If the elevations along a road increase, the slope of the road along the longitudinal direction, is known as

a. gradient

b. grade

c. positive grade

d. negative grade.

38. At a road junction, 16 cross conflict points are severe, if

a. both are one-way roads

b. both are two-way roads

c. one is two-way road and other is one-way road

d. none of these.

39. Traffic engineering only includes

a. design and application of control devices

b. analysis of traffic characteristics

c. traffic opeation

d. all the above.

40. An Executive Engineer of roads, executes works under direct control of

a. Superintending Engineer

b. Secretary to the Govt

c. Chief Engineer

d. None of these.

41. The total length of a cranked bar through a distance (d) at 45° in case of a beam of effective length L, is

a. L + 0.42 d

b. L + 2 x 0.42 d

c. L - 0.42 d

d. L - 2 x 0.4 d

42. The floor area includes the area of the balcony up to

a. 100%

b. 75%

c. 50%

d. 25%

43. The detention period in a septic tank is assumed

a. 20 minutes

b. 25 minutes

c. 30 minutes

d. 40 minutes

44. The diameter of a domestic sewer pipe laid at gradient 1 in 100 is recommended

a. 100 mm

b. 150 mm

c. 200 mm

d. 175 mm

45. PERT analysis is based on

a. optimistic time

b. pessimistic time

c. most likely time

d. all the above.

46. The time by which activity completion time can be delayed without affecting the start of succeeding activities, is known as

a. duration

b. total flat

c. free float

d. interfering float.

47. If TL is the latest allowable event occurrence time, total activity slack(s), is equal to

a. LST-EST

b. LFT-EFT

c. TL-EFT

d. all the above.

48. Construction team means

a. an architect

b. an owner

c. a contractor

d. all the above.

49. The bearing of the runway at threshold is 290°, the runway number is

a. N 70° W

b. 290°

c. 29°

d. W 20° N

50. The air is blowing at a speed of 75 knots in the direction of the aircraft flying at a ground speed of 775 knots. The air speed of the aircraft, is

a. 775 knots

b. 75 knots

c. 850 knots

d. 675 knots