***Sub Engineer***

1. The reduced level of a floor is 99.995 m, the staff reading on the floor is 1.505 m. If the inverted staff reading against the roof is 1.795 m, the floor level below the slab, is

a. 3.290 m

b. 3.300 m

c. 3.275 m

d. 2.790 m

2. Deviation of the actual road gradient from the proposed contou'r gradient up hill side, involves

a. embankment on the centre line

b. excavation on the centre line

c. earth work on the centre line

d. none of these.

3. The ratio of the distances at which a stated length can be distinguished by the telescope and the human eye, respectively, is called

a. brightness of telescope

b. magnification of telescope

c. resolving power of telescope

d. none of these.

4. In quadrantal bearing system, back bearing of a line may be obtained from its forward bearing, by

a. adding 180°, if the given bearing is less than 180°

b. subtracting 180°, if the given bearing, is more than 180°

c. changing the cardinal points, i.e. substituting N for S and E for W and vice-versa

d. none of these.

5. Porcelain is used as :

a. electric insulators

b. storage vessels

c. reactor chambers

d. All of the above.

6. Dextrin is

a. starch glue

b. albumin glue

c. rubber based adhesive

d. none to these.

7. The compound of Portland cement which reacts immediately with water and also sets first is

a. Tri-calcium silicate

b. Di-calcium silicate

c. Tri-calcium aluminate

d. Tetra calcium alumino ferrite.

8. Pick up the compound responsible for early strength of cement from the following:

a. Tetra-calcium alumino-ferrite

b. Tricalcium silicate

c. Tricalcium aluminate

d. Dicalcium silicate.

9. For determining the support reactions at A and B of a three hinged arch, points B and C are joined and produced to intersect the load line at D and a line parallel to the load line through A at D'. Distances AD, DD' and AD' when measured were 4 cm, 3 cm and 5 cm respectively. The angle between the reactions at A and B is

a. 30°

b. 45°

c. 60°

d. 90°

10. A compound bar consists of two bars of equal length. Steel bar cross-section is 3500 mm2 and that of brass bar is 3000 mm2. These are subjected to a compressive load 100,000 N. If Eb = 0.2 MN/mm2 and Eb = 0.1 MN/mm2, the stresses developed are:

a. σb = 10 N/mm2, σs = 20 N/mm2

b. σb = 8 N/mm2, σs = 16 N/mm2

c. σb = 6 N/mm2, σs = 12 N/mm2

d. σb = 5 N/mm2, σs = 10 N/mm2

11. The moment diagram for a cantilever which is subjected to a uniformly distributed load will be a

a. triangle

b. rectangle

c. parabola

d. cubic parabola.

12. The rise of a parabolic arch at quarter points, is equal to

a.  times the rise of the crown

b.  times the rise of the crown

c.  times the rise of the crown

d.  times the rise of the crown.

13. For the two dimensional flow, the stream function is given by ψ = 2xy. The velocity at a point (3, 4) is

a. 8 m/sec

b. 10 m/sec

c. 12 m/sec

d. 15 m/sec.

14. The ratio of the inertia and gravitational force acting in any flow, ignoring other forces, is called

a. Euler number

b. Frode number

c. Reynold number

d. Weber number.

15. Pick up the incorrect statement from to following regarding triangular notch :

a. For measuring low discharge, it gives more accurate result

b. Only one reading (i.e. H) is required for computation of discharge

c. Ventilation is necessary

d. None of these.

16. To avoid the force of surface tension in an inclined manometer, the minimum angle of inclination is

a. 2°

b. 3°

c. 4°

d. 5°

17. A suspended particle falls through a height H cm in water in t minutes. If the viscosity of water is η and specific gravity of the particle is G, the diameter of the particle is (where M is a constant factor)

a. 103M

b. 104M

c. 105M

d. 102M

18. The maximum load carried by a pile, when it continues to sink without further increase of load, is known as

a. ultimate load carrying capacity

b. ultimate bearing capacity

c. ultimate bearing resistant

d. all the above.

19. The bearing capacity of a soil depends upon

a. shape of the particles

b. cohesive properties of particles

c. internal frictional resistance of particles

d. all the above.

20. Pick up the correct statement from the following:

a. At critical hydraulic gradient, the saturated sand becomes quick

b. The critical gradient depends on the void ratio and the specific gravity

c. The quick sand occurs more in fine sand and silt than coarse material

d. All the above.

21. A raft foundation is provided if its area exceeds the plan area of the building by

a. 20%

b. 30%

c. 40%

d. 50%

22. The minimum cube strength of concrete used for a prestressed member, is

a. 150 kg/cm2

b. 250 kg/cm2

c. 350 kg/cm2

d. 400 kg/cm2

23. The diameter of longitudinal bars of a column should never be less than

a. 8 mm

b. 10 mm

c. 12 mm

d. none of these.

24. By over-reinforcing a beam, the moment of resistance can be increased not more than

a. 10%

b. 15%

c. 20%

d. 25%

25. Grillage foundation

a. is light and economical

b. does not require deep cutting as the required base area with required pressure intensity is obtained at a shallow depth

c. is constructed by rolled steel joists (R.S.J.) placed in single or double tier

d. all the above.

26. The portion of a brick cut across the width, is called

a. closer

b. half brick

c. bed

d. bat.

27. An ideal ware house, is provided

a. water proof roof

b. few windows which remain generally closed

c. 15 cm thick concrete floor laid on a dry course of soling

d. all the above.

28. If P, Y and Z are the weights of cement, fine aggregates and coarse aggregates respetively and W/C is the water cement ratio, the minimum quantity of water to be added to first batch, is obtained by the equation

a. 0.3P + 0.1Y + 0.01Z = W/C x P

b. 0.4P + 0.2Y + 0.01Z = W/C x P

c. 0.5P + 0.3Y + 0.01Z = W/C x P

d. 0.2P + 0.5Y + 0.1Z = W/C x P

29. Mostly used coagulant, is

a. chlorine

b. alum

c. lime

d. bleaching powder.

30. To pump water from a water reservoir 3 m deep and maximum water level at 135 m, a pump is installed to lift water up to R.L. 175 m at a constant rate of 36, 00, 000 litres per hour. If the length of the pipe is 1506 m and f = 0.01, ignoring other minor losses and assuming the economical diameter from Lea's formula D = 1.2 Q, the water horse power of the pump is

a. 450

b. 500

c. 580

d. 600

31. For efficient working of a sewer, it must be ensured that

a. minimum velocity of 0.45 m/sec, is maintained at its minimum flow

b. a maximum velocity of 0.90 m/sec, is maintained at its maximum flow

c. both (a) and (b)

d. neither (a) nor (b).

32. Pick up the correct statement from the following :

a. The automatic purification of polluted water, is known self purification phenomenon

b. The photosynthesis is carried out in the presence of sun light

c. Grab sample is generally obtained from the surface where turbulence mixes the sewage particles

d. All the above.

33. Lacy's regime condition is obtained if

a. silt grade in the channel is variable

b. discharge in the channel is variable

c. silt charge in the channel is variable

d. channel flows in unlimited, incoherent alluvium of the same character as that transported material.

34. If the optimum depth of kor watering for a crop is 15.12 cm, the outlet factor for the crop for four week period in hectares per cumec, is

a. 1200

b. 1400

c. 1600

d. 2000

35. A river training work is generally required when the river is

a. meandering

b. aggrading

c. degrading

d. all the above.

36. The depth of rice root zone, is

a. 60 cm

b. 70 cm

c. 80 cm

d. 90 cm

37. Enoscope is used to determine

a. spot speed

b. average speed

c. travel time

d. none of these.

38. The full width of land acquired before finalising a highway, alignment is known

a. width of formation

b. right of way

c. carriage way

d. roadway.

39. In a braking test, a vehicle travelling at 36 km ph was stopped at a braking distance of 8.0 m. The average value of the vehicle's skid resistance (friction coefficient) is

a. 0.64

b. 6.25

c. 0.16

d. none of these

40. Driving vehicles on wet surfaced roads, is dangerous because it may

a. skid

b. slip

c. overturn

d. all the above.

41. The order of booking dimensions is

a. Length, breadth, height

b. Breadth, length, height

c. Height, breadth, length

d. None of these.

42. The damp proof course (D.P.C.) is measured in

a. Cub.m

b. Sq m

c. Metres

d. None of these

43. Pick up the correct statement from the following:

a. The earth work of cutting in trenches or borrow pits in fairly uniform ground is measured with the help of average depths of the dead men

b. The earth work in trenches or borrow pits in irregular ground is measured by taking the difference in levels before and after completion of work

c. The earth work in trenches or borrow pits, where neither a nor b is feasible, are measured from the fillings after deduction of voids

d. All the above.

44. The expected out turn for earth work in excavation in ordinary soil per mazdoor per day is

a. 1.00 cubic meter

b. 2.00 cubic meter

c. 3.00 cubic meter

d. 4.00 cubic meter

45. While filling the tender for any work, the contractor considers

a. availability of construction materials

b. availability of labour

c. study of specifications

d. all the above.

46. A CPM family includes

a. CPP (Critical Path Plotted)

b. MCE (Minimum Cost Expenditure)

c. CPS (Critical Path Scheduling)

d. All the above.

47. The Overall in-charge of an organisation at the site responsible for the execution of the works, is

a. Engineer

b. Junior Engineer

c. Sub overseer

d. Assistant Engineer.

48. Bar charts are suitable for

a. minor works

b. major works

c. large projects

d. all the Above.

49. Pick up the correct statement from the following:

a. The propellers which are driven by turbine engines, are technically called turboprops

b. The aircrafts which obtain the thrust directly from turbine engine, are called turbo-jets

c. The aircraft which is filled with a fan infront of turbo-jet engine, is called turbofan propelled aeroplane

d. All the above.

50. Pick up the correct statement from the following:

a. The wind data of an air port is depicted in the form of a chart known as wind rose

b. The landing and take off of the air craft is made against the wind direction

c. The product of wind loading and power loading is called air craft rating

d. All the above.