***Sub Engineer***

1. While surveying a plot of land by plane tabling, the field observations

a. and plotting proceed simultaneously

b. and plotting do not proceed simultaneously

c. and recorded in field books to be plotted later

d. all the above.

2. Imaginary line passing through points having equal magnetic declination is termed as

a. isogon

b. agonic line

c. isoclinic line

d. none of these.

3. The representation of general topography of a very flat terrain is possible only

a. by drawing contours at large interval

b. by drawing contours at small interval

c. by giving spot levels at large interval

d. by giving spot levels to salient features at close interval.

4. In a constant level tube, size of the bubble remains constant because upper wall is

a. of relatively larger radius

b. of relatively smaller radius

c. flat

d. convex downwards.

5. Commonly used thinner in

a. cellulose paints is ethyle acetate

b. oil paints, is naptha

c. distemper, is water

d. all the above.

6. Seasoning of timber is done

a. to make it water proof

b. to paint its surface

c. to increase its temperature

d. to remove water.

7. Brittleness of cold is due to an excess of

a. sulphur

b. carbon

c. phosphorus

d. silicon.

8. During pudding

a. carbon is converted into carbonic acid gas

b. silicon forms a slag

c. metal is heated by the burning of gases

d. all the above.

9. A three hinged arch is generally hinged at its supports and

a. at one quarter span

b. at the crown

c. any where in the rib

d. none of these.

10. A steel bar 5 m x 50 mm is loaded with 250, 000 N. If the modulus of elasticity of the material is 0.2 MN/mm2 and Poisson's ratio is 0.25, the change in the volume of the bar is :

a. 1.125 cm3

b. 2.125 cm3

c. 3.125 cm3

d. 4.125 cm2

11. A beam of length L supported on two intermediate rollers carries a uniformly distributed load on its entire length. If sagging B.M. and hogging B.M. of the beam are equal, the length of each overhang, is

a. 0.207 L

b. 0.307 L

c. 0.407 L

d. 0.5 L.

12. If a steel rod of 20 mm diameter and 5 metres long elongates by 2.275 mm when subjected to an axial pull of 3000 kg, the stress developed, is

a. 9.5541 kg/cm2

b. 95.541 kg/cm2

c. 955.41 kg/cm2

d. 9554.1 kg/cm2.

13. For uniform flow in canals

a. there is a balance between the frictional loss and drop in elevation of the channel

b. bed and free water surfaces of a channel are parallel to each other

c. bed of channel represents the hydraulic gradient

d. all the above.

14. In an inclined position, a venturimeter records

a. more reading

b. less reading

c. same reading

d. none of these.

15. The main assumption of Bernoulli's equation is :

a. The velocity of energy of liquid particle, across any cross-section of a pipe is uniform

b. No external force except the gravity acts on the liquid

c. There is no loss of energy of the liquid while flowing

d. All the above.

16. Most economical section of a circular channel for maximum velocity, is if,

a. depth of water = 0.810 diameter

b. hydraulic mean depth = 0.304 diameter

c. wetted perimeter = 2.245 diameters

d. all the above.

17. The vane shear test is used for the in-situ determination of the undrained strength of the intact fully saturated

a. sands

b. clays

c. gravels

d. highly organic soils.

18. The soil moisture driven off by heat, is called

a. free water

b. hydroscopic water

c. gravity water

d. none of these.

19. 'Talus' is the soil transported by

a. wind

b. water

c. glacier

d. gravitational force.

20. Determination of water content of a soil sample suspected to contain gypsum is made by drying the sample for longer period at a temperature not more than

a. 60°C

b. 80°C

c. 100°C

d. 110°C

21. To ensure that the hogging bending moment at two points of suspension of a pile of length L equals the sagging moment at its centre, the distances of the points of suspension from either end, is

a. 0.107 L

b. 0.207 L

c. 0.307 L

d. 0.407 L

22. For a continuous floor slab supported on beams, the ratio of end span length and intermediate span length, is

a. 0.6

b. 0.7

c. 0.8

d. 0.9

23. A per IS : 1343, total shrinkage for a pretensioned beam, is

a. 3.0 x 10-3

b. 3.0 x 10-4

c. 3.0 x 10-5

d. 3.5 x 10-5

24. An R.C.C. beam not provided with shear reinforcement may develop cracks in its bottom inclined roughly to the horizontal at

a. 35°

b. 45°

c. 55°

d. 60°

25. Slate

a. is a metamorphic rock

b. splits into thin sheets along its bedding planes

c. has a smooth surface and contains alumina and silica

d. possesses good water absorption capacity

26. Pick up the correct statement from the following :

a. the lime with surkhi used as mortor for construction reduces the cost and provides equal strength to wall

b. for very cold or very hot climate, a compact and closed plan should be provided

c. on the sea coast, an exposed and open house is generally preferred

d. all the above.

27. Pick up the correct statement from the following:

a. The free water is the amount of water added while mixing and the amount of water held on the surface of the aggregates prior to mixing

b. The total water is the free water and the amount actually absorbed by the aggregates

c. Neither (a) nor (b)

d. Both (a) and (b).

28. For a good concrete

a. cement should be sufficient to produce the required strength

b. water should be free from organic materials

c. mixing of ingredients should be done thoroughly so as to produce homogeneity

d. All the above.

29. A well is considered to be good if it is sunk into

a. Clay

b. Sand

c. Coarse gravel

d. Silt.

30. The bed slope in slow sand filters, is generally kept

a. 1 in 50

b. 1 in 75

c. 1 in 100

d. 1 : 200

31. 'Cowl' is provided at

a. lower end of ventilating column

b. upper end of ventilating column

c. upper end of the manhole

d. first step in manhole.

32. Drop manholes are the manholes

a. without entry ladders

b. without manhole covers

c. with depths more than 3.5 m

d. having drains at different levels.

33. The crest level of a canal diversion head work, depends upon

a. discharge perimeters

b. pond level

c. all the above

d. none these.

34. In rigid module, the discharge

a. is independent of water levels in the distributary and water course

b. depends upon the water level in distributary

c. depends upon the water level in the water course

d. depends upon the water levels of both, i.e. distributary and water course

35. The useful moisture of soil, is equal to its

a. field capacity

b. saturation capacity

c. moisture content at permanent wilting point

d. difference between filed capacity and permanent wilting point within the root zone of plants.

36. The optimum depth of kor watering for a rice crop, is

a. 19.0 cm

b. 17.5 cm

c. 13.5 cm

d. 12.0 cm

37. The minimum superelevation in rolling terrain in plains, is limited to

a. 5%

b. 6%

c. 7%

d. 10%

38. To prevent movement of moisture from subgrade to road pavement at the same level as that of water-table, thickness of a cut off layer of coarse sand, is

a. 15 cm

b. 20 cm

c. 30 cm

d. 45 cm

39. Longitudinal pavement lines marked broken in white paint

a. are for the guidance of the drivers

b. are not to be crossed over

c. may be crossed over at the discretion of the driver

d. all the above.

40. If the radius of curvature of a hill road is 50 m, the percentage grade compensation should be

a. 70/R

b. 75/R

c. 80/R

d. 100/R

41. The assumption on which the trapezoidal formula for volumes is based, is

a. The end sections are parallel planes

b. The mid-area of a pyramid is half the average area of the ends

c. The volume of the prismoidal is over-es-timted and hence a prismoidal correction is applied

d. All the above.

42. The height of the sink of wash basin above floor level is kept

a. 60 cm

b. 70 cm

c. 75 cm to 80 cm

d. 80 cm

43. A portion of an embankment having a uniform up-gradient 1 in 500 is circular with radius 1000 m of the centre line. It subtends 180° at the centre. If the height of the bank is 1 m at the lower end, and side slopes 2:1, the earth work involved.

a. 26, 000 m3

b. 26, 500 m3

c. 27, 000 m3

d. 27, 500 m3

44. The brick work is measured in sq metre, in case of

a. Honey comb brick work

b. Brick flat soling

c. Half brick walls or the partition

d. All the above.

45. In CPM analysis,

a. uncertainties are not allowed

b. activities are represented by arrows

c. beginning and end of an activity, are denoted by nodes

d. all the above.

46. The Overall in-charge of an organisation at the site responsible for the execution of the works, is

a. Engineer

b. Junior Engineer

c. Sub overseer

d. Assistant Engineer.

47. Pick up the correct statement from the following:

a. Pessimistic time estimate refers to activities

b. Most likely time estimate refers to activities

c. Expected time estimate refers to activities

d. All the above.

48. Which one of the following represents an event ?

a. concrete cured

b. fixing of door

c. plastering of walls

d. selecting sites

49. For the taxiways, the following statement is true

a. The maximum longitudinal grade is 3%

b. The permissible rate of change of grade is 1%

c. The permissible transverse grade is 1.5%

d. All the above.

50. Pick up the incorrect statement from the following:

a. Channelization of pavement is caused by constant use of tri-cycle gears of aircraft

b. Blast pads are used over cohesionless soils to resist erosion due to tremendous speed of the jet blast

c. Over-run areas of at least 300 m length on either side of the runway are provided

d. None of these.