***Sub Engineer***

1. Accuracy of 'fix' by two point problem, is

a. bad

b. good

c. not reliable

d. unique.

2. For setting out a simple curve, using two theodolites.

a. offsets from chord produced are required

b. offsets from long chord are required

c. deflection angles from Rankine's formula are required

d. none of these.

3. In chain surveying, perpendiculars to the chain line, are set out by

a. a theodolite

b. a prismatic compass

c. a level

d. an optical square

4. Closed contours of decreasing values towards their centre, represent

a. a hill

b. a depression

c. a saddle or pass

d. a river bed.

5. The commonly used colour pigment in paints, is

a. carbon black

b. iron oxide

c. lamp black

d. all the above.

6. Ground glass

a. is made by grinding its one side

b. is made by melting powdered glass paints surface

c. is used for getting light without transparency

d. all the above.

7. Pick up the correct statement from the following:

a. Quick lime is obtained by burning pure lime stone:

b. Hydraulic lime is obtained by burning lime stone containing clay 5% to 30%

c. Poor lime is obtained by burning lime stone containing impurities more than 5%

d. All the above.

8. PVC stands for

a. plastic very compact

b. polythene vinyl chloride

c. polythene vinyl carbon

d. polythene vanadium carbide.

9. Pick up the incorrect statement from the following : The torsional resistance of a shaft is directly proportional to

a. modulus of rigidity

b. angle of twist

c. reciprocal of the length of the shaft

d. moment of inertia of the shaft section.

10. Maximum principal stress theory for the failure of a material at elastic point, is known

a. St. Venant's theory

b. Rankine's theory

c. Haig's theory

d. Von Mises's theory.

11. A member which is subjected to reversible tensile or compressive stress may fail at a stress lower than the ultimate stress of the material. This property of metal, is called

a. plasticity of the metal

b. elasticity of the metal

c. fatigue of the metal

d. workability of the metal.

12. Ties are load carrying members of a frame, which are subjected to

a. transverse loads

b. axial tension loads

c. axial compressive loads

d. torsional loads.

13. Pick up the correct statement regarding Borda's mouth piece running full from the following :

a. Theoretical velocity at the outlet is 2gH

b. Coefficient of velocity is 1/2

c. Coefficient of contraction is 1

d. All the above.

14. The centre of pressure of a vertical plane immersed in a liquid is at

a. centre of higher edge

b. centre of lower edge

c. centroid of the area

d. none of these.

15. The discharge through a V-notch weir varies as

a. 1/H

b. H3/2

c. H7/2

d. H5/2.

16. In an inclined pipe, the pressure difference at its two ends is due to

a. exit head drop

b. frictional loss head

c. elevation head

d. all the above.

17. Pick up the correct statement from the following:

a. If the ratio of depth to width is more than 2, it is deep foundation

b. If the ratio of the length to width is between 1 and 2, it is spread foundation

c. If the length is large as compared to width, it is a strip foundation

d. All the above.

18. Pick up the correct statement from the following:

a. The permeability of the coarse-grained soils may be reduced by grouting.

b. The process of injecting fluids (i.e. grouts) into the pores space of the soil, is called grouting.

c. The grouting increases the soil strength.

d. All the above.

19. The coefficient of curvature for a well graded soil, must be between

a. 0.5 to 1.0

b. 1.0 to 3.0

c. 3.0 to 4.0

d. 4.0 to 5.0

20. Pick up the correct statement from the following:

a. In hydrometer method, weight Wd per ml of suspension is found directly

b. In pipette analysis, weight Wd per ml of suspension is found indirectly

c. In pipette analysis, weight Wd per ml of suspension is found directly

d. None of these.

21. The ratio of the breadth to effective depth of a beam is kept

a. 0.50

b. 0.70

c. 0.75

d. none of these.

22. The diameter of main bars in R.C.C. columns, shall not be less than

a. 6 mm

b. 8 mm

c. 10 mm

d. 12 mm

23. The steel generally used in R.C.C. work, is

a. stainless

b. mildsteel

c. high carbon steel

d. high tension steel.

24. If the diameter of longitudinal bars of a square column is 16 mm, the diameter of lateral ties should not be less than

a. 5 mm

b. 6 mm

c. 8 mm

d. 10 mm

25. Weep holes are provided in retaining and breast walls

a. to drain off the water from the filling

b. to ventilate the stone masonry

c. to add architectural beauty

d. to reduce the weight of the earth retained

26. If height of the first storey of a building is 3.2 m and riser is 13 cm, the number of treads required, is

a. 18

b. 24

c. 25

d. 30.

27. Pick up the correct proportions of chemical ingredients of cement

a. Lime : Silica : Alumina : Iron oxide : 63 : 22 : 6 : 3

b. Silica : Lime : Alumina : Iron oxide : 63 : 22 : 6 : 3

c. Alumina : Silica : Lime : Iron oxide : 63 : 22 : 6 : 3

d. Iron oxide : Alumina : Silica : Lime : 63 : 22 : 6 : 3

28. Ordinary concrete is not used for concrete grade

a. M 150

b. M 200

c. M 250

d. M 400

29. Pick up the incorrect statement from the following. The underground sources of water, is from

a. springs

b. infiltration wells

c. storage reservoirs

d. none of these.

30. The U.C. (uniformity coefficient)  for the best filter media sand should be

a. 2

b. 3

c. 4

d. 5

31. The discharge per unit plan area of a sedimentation tank, is generally called

a. over flow rate

b. surface loading

c. over flow velocity

d. all the above.

32. Pick up the correct statement from the following :

a. The screenings are disposed off either by burning or by burial or by dumping

b. The process of burning the screenings, is called incineration

c. The process of burning the screenings, is known as compositing

d. All the above.

33. Groynes are generally built

a. perpendicular to the bank

b. inclined up stream up to 30°

c. inclined down stream upto 30°

d. all the above.

34. Side slopes generally kept for canals in loam soil, are :

a. 1 : 1 in cutting and  : 1 in filling

b.  : 1 in cutting and 2 : 1 in filling

c. Neither (a) nor (b)

d. Both (a) and (b)

35. Pick up the incorrect statement from the following:

a. In free flooding irrigation, water is admitted at one corner of a field and is allowed to spread over the entire area

b. In check method of irrigation, the field is divided into smaller compartments and water is admitted to each in turn

c. In furrow irrigation water is admitted between the rows of plants in the field

d. None of these.

36. Process of meandering is due to

a. sediment load of streams

b. discharge and hydraulic properties of streams

c. relative erodibility of the bed and banks

d. all the above.

37. When each particle of aggregates is thinly coated with cement paste, a heterogeneous solid is formed, which is known as

a. hydration

b. gel

c. concrete

d. none of these.

38. Length of vehicles does not affect

a. minimum radius of turning

b. passing sight distance

c. width of shoulders

d. none of these.

39. To prevent a head-on-collision of vehicles travelling in opposite directions along four-lane roads

a. physical dividers are provided

b. area dividers are provided

c. medians of wide area are provided

d. none of these.

40. Pick up the incorrect statement from the following:

a. Highways are always constructed in straight line

b. Highways may be provided horizontal curves

c. Highways may be provided vertical curves

d. Highways may be provided both horizontal and vertical curves.

41. The order of booking dimensions is

a. Length, breadth, height

b. Breadth, length, height

c. Height, breadth, length

d. None of these.

42. Pick up the correct statement from the following:

a. The earth work of cutting in trenches or borrow pits in fairly uniform ground is measured with the help of average depths of the dead men

b. The earth work in trenches or borrow pits in irregular ground is measured by taking the difference in levels before and after completion of work

c. The earth work in trenches or borrow pits, where neither a nor b is feasible, are measured from the fillings after deduction of voids

d. All the above.

43. The area of a sloping surface of a protective embankment of mean height d, side slopes S : 1 and length L is

a. d x d x s

b. d2 x (ds)2

c. L.D 1 + s2

d. 2 LD 1 + s2

44. Pick up the incorrect statement from the following:

a. No deduction is made for the openings upto 0.1 sq.m

b. No deduction is made for volumes occupied by pipes, not exceeding 100 sq.cm in cross-section

c. No deductions are made for the ends of dissimilar structures up to 500 sq.cm cross-sectional area

d. None of these.

45. Final technical authority of a project lies with

a. Assistant Engineer

b. Executive Engineer

c. Superintending Engineer

d. Chief Engineer.

46. Pick up the incorrect statement from the following:

a. The tail of the arrow indicates the start of the activity

b. The head of the arrow indicates the end of the activity

c. The arrows are drawn to scale from left to right

d. Each activity consumes a given time.

47. A goiden rule for the procurement of construction stones, suggests

a. 67% at the site and 33% under procurement

b. 50% at the site and 50% under procurement

c. 33% at the site and 67% under procurement

d. none of these.

48. If the total float and duration of an activity are 5 and 10 days respectively, the particular activity can be

a. started 5 days later

b. completed 5 days later

c. performed at slower rate in 15 days

d. all the above.

49. Effective length of a runway is the distance between

a. ends of the runway

b. point of intersection of the obstruction clearance line and the extended plane of the runway surface, and the other end of the runway

c. point of intersection of the glide path and the extended plane of the runway surface and the other end of the runway

d. ends of the clear way on either side.

50. Pick up the correct statement from the following:

a. The wind data of an air port is depicted in the form of a chart known as wind rose

b. The landing and take off of the air craft is made against the wind direction

c. The product of wind loading and power loading is called air craft rating

d. All the above.