***Sub Engineer***

1. The defect of a lens whereby rays of white light proceeding from a point get dispersed into their components and conveyed to various foci, forming a blurred and coloured image is known as

a. chromatic aberration

b. spherical aberration

c. astigmatism

d. coma.

2. The staff reading at a distance of 80 m from a level with the bubble at its centre is 1.31 m. When the bubble is moved by 5 divisions out of the centre, the reading is 1.39 m. The angular value of the one division of the bubble, is

a. 41.25 secs

b. 14.52 secs

c. 25.05

d. none of these

3. Which of the following introduces an error of about 1 in 1000 if 20 m chain is used

a. one end of the chain 0.9 m off the line

b. one end of chain 0.9 m higher than the other

c. middle of the chain 0.45 m off the line

d. all the above.

4. Diaphragm of a surveying telescope is held inside

a. objective

b. telescope tube at its mid point

c. telescope at the end nearer the eye-piece

d. telescope at its end nearer the objective.

5. Pick up the correct statement from the following:

a. Blistering may be cured by applying water paint finished with oil paint dried with a little copal varnish

b. Cracked paints may be cured by removing paint and giving a fresh coat of paint

c. Crawling paints may be cured by sand preparing the surface and giving a fresh coat with plenty of turps

d. All the above.

6. Magnese steels

a. possess high electrical resistance

b. possess low coefficient of expansion

c. are used for the manufacture of rails

d. all the above.

7. Laterite is found in

a. U.P.

b. Punjab

c. West Bengal

d. Kerala.

8. Spirit varnish generally consists of

a. alcohol, wax and turpentine

b. pigment and synthetic resin

c. spirit and shellac

d. none of these.

9. A shaft rotating N.R.M. under a torque T, transmits a power

a.  Newton metres/sec

b.  Newton metres/min

c.  Newton metres/min

d.  Newton metres/sec

10. At any point of a beam, the section modulus may be obtained by dividing the moment of inertia of the section by

a. depth of the neutral axis

b. maximum tensile stress at the section

c. maximum compressive stress at the section

d. none of these.

11. The effect of arching a beam, is

a. to reduce the bending moment throughout

b. to increase the bending moment throughout

c. nothing on the bending throughout

d. ail the above.

12. A cantilever beam rectangular in cross-section is subjected to an isolated load at its free end. If the width of the beam is doubled, the deflection of the free end will be changed in the ratio of

a. 1/8

b. 1/2

c. 2

d. 3

13. For a most economical rectangular channel, the hydraulic mean depth, is equal to

a. the depth of flow

b. half the depth of flow

c. one-third depth of flow

d. None of these.

14. The shear stress distribution in viscous fluid through a circular pipe is :

a. maximum at the centre

b. maximum at the inside of surface

c. same throughout the section

d. none of these.

15. The acceleration f required to accelerate a rectangular tank containing water horizontally so that the slope of its free surface is 45°, is

a. g

b. 2 g

c. 2.5 g

d. 3 g.

16. A jet projected at an angle of 45θ, 40 m from the foot of a vertical column, just reaches the top of the column. The height of the column is

a. 20 m

b. 30 m

c. 40 m

d. 60 m.

17. 260 g of wet soil was taken in a pycnometer jar of weight 400 g in order to find the moisture content in the soil, with specific gravity of soil particles 2.75. The weight of soil and remaining water filled in pycnometer without air bubbles was 1415 g and the weight of pycnometer filled with water alone was 1275 g. The moisture content in the soil is

a. 24.2%

b. 18.2%

c. 53.8%

d. none of these.

18. Water content of a soil sample is the difference of the weight of the given sample at the given temperature and the weight determined after drying it for 24 hours at temperature ranging from

a. 90° to 95°C

b. 95° to 100°C

c. 103° to 105°C

d. 105° to 110°C

19. For slopes of limited extent the surface of slippage, is usually along

a. a parabolic arc

b. an elliptical arc

c. a straight line

d. a circular arc.

20. Darcy's law is applicable to seepage if a soil is

a. homogeneous

b. isotropic

c. incompressible

d. all the above.

21. The minimum number of main steel bars provided in R.C.C.

a. rectangular columns is 4

b. circular columns is 6

c. octagonal columns is 8

d. all the above.

22. For a continuous floor slab supported on beams, the ratio of end span length and intermediate span length, is

a. 0.6

b. 0.7

c. 0.8

d. 0.9

23. In a cantilever retaining wall without a heel slab

a. thickness of the stem is kept same throughout

b. base slab is made 10 cm thicker than the stem

c. width of the base slab is kept 0.7 time the total height of the wall

d. all the above.

24. The thickness of the topping of a ribbed slab, varies between

a. 5 cm to 8 cm

b. 8 cm to 10 cm

c. 12 cm to 15 cm

d. 12 cm to 18 cm

25. The art of bringing the floor to a true level surface by means of screads, is called

a. topping

b. bedding

c. screading

d. none of these.

26. The arrangement of supporting an existing structure by providing supports underneath, is known as

a. shoring

b. underpinning

c. jacking

d. piling

27. After casting, an ordinary cement concrete on drying

a. expands

b. mix

c. shrinks

d. none of these.

28. Pick up the incorrect statement from the following:

a. Cement bags should preferably be piled on wooden planks

b. Cement bags should be placed such that bags of one layer does not touch the bags of the adjacent layer

c. Width and height of the pile should not exceed 3 m and 2.70 m respectively

d. None of these.

29. Surface water is obtained from

a. well

b. springs

c. artesian well

d. rain.

30. The maximum pressure to which a pipe is subjected to during its operation, is known

a. working pressure

b. design pressure

c. test pressure

d. pipe pressure

31. Oxidation process results in the formation of

a. carbon dioxide

b. nitrates

c. sulphates

d. all the above.

32. Which one of the following gases is most significant as air pollutant

a. Carbondioxide

b. Oxygen

c. Nitrogen

d. Sulphurdioxide.

33. A current meter measures the velocity of flow, if it is held

a. on the surface of water

b. at the bottom of channel

c. at any point within the cross-section

d. none of these.

34. For smooth entry of water in a canal, the angle between head regulator and water is generally kept

a. 80°

b. 90°

c. 110°

d. 120°

35. Pick up the correct statement from the following

a. The capacity of escapes should not be less than the capacity of the canal at its location

b. The escapes must lead the surplus water to natural drainages

c. The escapes are aligned to take advantage of contours of lower values

d. All the above.

36. The width of a dowla is generally kept between 30 to 60 cm and its height above the road level should invariably be more than

a. 10 cm

b. 20 cm

c. 30 cm

d. 40 cm

37. In cement concrete pavements, tensile stress is due to :

a. bending or deflection under wheel loads

b. difference in temperature of the top and bottom of pavement

c. contraction of slab during falling temperature

d. all the above.

38. The top height of a route marker above crown level is

a. 1.50 m

b. 1.75 m

c. 2.00 m

d. 2.25 m

39. Normal formation width of a hill road for one-way traffic, is

a. 4.8 m

b. 6.6 m

c. 7.2 m

d. 8 m

40. The camber on pavements, is provided by

a. parabolic method

b. straight line and parabolic at crown

c. elliptical method

d. circular method.

41. In the mid-section formula

a. The area of mid-sections is calculated by using mean depth

b. The volume of the earth work is calculated by multiplying the mid-section area by the distance between the two sections

c. The volume of the earth work is calculated by multiplying the mid-section area by the distance between the two original sections

d. (a), (b) and (c) of the above.

42. The item of steel work which is measured in sq.m, is

a. Rolling shutters

b. Steel doors

c. Ventilators and glazing.

d. All the above.

43. Pick up the excavation where measurements are made in square metres for payment.

a. surface dressing up to 15 cm depths

b. Surface excavation up to 30 cm depths

c. Both (b) and (c)

d. Both (a) and (b)

44. The expected out turn of 12 mm plastering with cement mortar is

a. 4.0 sq m

b. 6.0 sq m

c. 8.0 sq m

d. 10 sq m

45. A Milestone chart

a. shows the interdependencies of various jobs

b. depicts the delay of jobs, if any

c. points outgoing ahead of schedule of jobs, if any

d. none of these.

46. Pick up the correct statement from the following:

a. Latest occurrence time is denoted by TL

b. Contractual obligation time is denoted by Ts

c. Latest occurrence time is taken as contractual obligation time

d. All the above.

47. While scheduling a project by C.P.M.

a. required time for each activity is established

b. sequence of various activities is made according to their importance

c. net work is drawn by connecting the activities and the events

d. All the above.

48. Pre-tender stage requires

a. selection of site

b. formalisation of alignment of work

c. formalisation of designs and preparation of estimate

d. all the above.

49. Pick up the correct statement from the following:

a. The basic length of a runway is increased at a rate of 7% per 300 m of elevation of M.S.L.

b. The standard temperature at the site is obtained by reducing the standard sea level temperature of 15°C at the rate of 6.5°C per 1000 m rise in elevation

c. The aerodrome reference temperature is the monthly mean of the mean daily temperature for the hottest month of the year

d. All the above.

50. Two single runways may be arranged so as to have

a. L-shape

b. T-shape

c. X-shape

d. All the above.