***Sub Engineer***

1. If the plane table is not horizontal in a direction at right angles to the alidade, the line of sight is parallel to the fiducial edge only for

a. horizontal sights

b. inclined sights upward

c. inclined sight downward

d. none of these.

2. Accuracy of elevation of various points obtained from contour map is limited to

a.  of the contour interval

b.  th of the contour interval

c.  rd of the contour interval

d.  th of the contour interval.

3. A uniform slope was measured by the method of stepping. If the difference in level between two points is 1.8 m and the slope distance between them is 15 m, the error is approximately equal to

a. cumulative, + 0.11 m

b. compensating, ± 0.11 m

c. cumulative, - 0.11 m

d. none of these

4. The intercept of a staff

a. is maximum if the staff is held truly normal to the line of sight.

b. is minimum if the staff is held truly normal to the line of sight.

c. decreases if the staff is tilted away from normal

d. increases if the staff is tilted towards normal.

5. Pick up the correct statement from the following:

a. Lime is available by dissolving calcium carbonate in water

b. Lime is available by calcining calcium carbonate at 900°C

c. Lime is nothing but calcium chloride

d. None of these.

6. The sequence of refractory materials according to increasing melting points is :

a. Dolomite, Magnesia, Bauxite, Chromite

b. Bauxite, Chromite, Dolomite, Magnesia

c. Magnesia, Bauxite, Dolomite, Chromite.

7. Wrought iron is manufactured from pig iron by

a. puddling

b. shingling

c. rolling

d. all the above.

8. Porcelain is used as :

a. electric insulators

b. storage vessels

c. reactor chambers

d. All of the above.

9. Beams composed of more than one material, rigidly connected together so as to behave as one piece, are known as

a. Compound beams

b. Indeterminate beams

c. Determinate beams

d. Composite beams.

10. The locus of the end point of the resultant of the normal and tangential components of the stress on an inclined plane, is

a. circle

b. parabola

c. ellipse

d. straight line.

11. If all the dimensions of a bar are increased in the proportion n : 1, the proportion with which the maximum stress produced in the prismatic bar by its own weight, will increase in the ratio

a. n : 1

b. 1 :

c.  : 1

d. 1 : n.

12. A diagram which shows the variations of the axial load for all sections of the span of a beam, is called

a. shear force diagram

b. thrust diagram

c. stress diagram

d. none of these.

13. The phenomenon occuring in an open channel when a rapidly flowing stream abruptly changes to a slowly flowing stream causing a distinct rise of liquid surface, is

a. water hammer

b. hydraulic jump

c. critical discharge

d. none of these.

14. For solving the problems is hydraulic engineering, the velocity used is

a. velocity at the centre of pipe section

b. average velocity of flow over a section

c. mean of the velocities at the centre and that along the pipe surface

d. none of these.

15. To avoid the tendency of separation of liquid flow, the most suitable ratio of the diameters of the throat and the pipe, is

a. 1/4 to 1/8

b. 1/3 to 1/2

c. 1/2 to 3/4.

d. none of these.

16. A rise or fall of liquid in a glass tube of a very small diameter when dipped is

a. directly proportional to the force per unit length of periphery

b. direcly proportional to the sine of the angle of contact

c. directly proportional to the specific weight of liquid

d. inversely proportional to the diameter of the glass tube.

17. The zero atmospheric pressure is at

a. mean sea level

b. water table

c. phreatic surface

d. both (c) and (d) of the above.

18. Soils containing organic matters

a. are of spongy nature

b. swell with decrease of moisture

c. shrink with increase of moisture content

d. none of these.

19. The direct shear test suffers from the following disadvantage:

a. Pore water pressure cannot be measured

b. Shear stress on the failure plane is not uniform.

c. The area under the shear and vertical loads does not remain constant throughout the test

d. All the above.

20. Pick up the correct statement from the following:

a. The dry density reduces by addition of water after attaining optimum moisture content

b. The line joining the peak of three moisture content graphs obtained by using three compactive energies, is called line of optimus

c. Well graded coarse grained soils can be compacted to a very high density as compared to fine grained soils

d. All the above. .

21. A pre-stressed concrete member is preferred because

a. its dimensions are not decided from the diagonal tensile stress

b. large size of long beams carrying large shear force need not be adopted

c. removal of cracks in the members due to shrinkage

d. all the above.

22. The diameter of the column head support a flat slab, is generally kept

a. 0.25 times the span length

b. 0.25 times the diameter of the column

c. 4.0 cm larger than the diameter of the column

d. 5.0 cm larger than the diameter of the column

23. For M 150 grade concrete (1:2:4) the moment of resistance factor is

a. 8.50

b. 7.50

c. 5.80

d. none of these.

24. Design of a two way slab simply supported on edges and having no provision to prevent the corners from lifting, is made by

a. Marcus formula

b. Rankine Grashoff formula

c. Grashoff formula

d. Rankine-Marcus formula.

25. The portion of a brick cut across the width, is called

a. closer

b. half brick

c. bed

d. bat.

26. A wall constructed to resist the pressure of an earth filling, is called

a. retaining wall

b. breast wall

c. buttress

d. parapet wall.

27. Slump test of concrete is a measure ofits

a. consistency

b. compressive strength

c. tensile strength

d. impact value.

28. To obtain cement dry powder, lime stones and shales or their slurry, is burnt in a rotary kiln at a temperature between

a. 1200° and 1300°C

b. 1300° and 1400°C

c. 1400° and 1500°C

d. 1500° and 1600°C

29. Mud balls may be removed by

a. breaking and washing

b. washing the filter with a solution of caustic soda

c. removing, cleaning and replacing the damaged sand

d. all the above.

30. Efficiency of removing bacterias from raw water by a slow sand filter, is

a. 80% to 81%

b. 85% to 86%

c. 90% to 97%

d. 98% to 99%.

31. Hazin's formula  is used for the settlement velocity of the particles of diameter

a. less than 0.01 mm

b. less than 0.05 mm

c. less than 0.1 mm

d. greater than 0.1 mm.

32. Aerobic activity is maximum

a. in freshly produced sewage

b. at sewer pipes

c. in sewer treatment plants

d. none of these.

33. Pick up the correct statement from the following:

a. Gravity water is harmful to crops

b. Hygroscopic water remains attached to soil molecules by chemical bond

c. Capillary moisture held in the soil pores against gravity by surface tension, is utilised by plants

d. All the above.

34. F.S.L. of a canal at its head with respect to parent channel is kept

a. at the same level

b. 15 cm lower

c. 15 cm higher

d. none of these.

35. The level of the fioor of a syphon aqueduct can be obtained

a. by subtracting the depth of the culvert from the canal bed level

b. by subtracting the thickness of culvert plus the depth of the culvert from the canal bed level

c. both (a) and (b)

d. none of these.

36. According to Lacey, depth of scour in a river depends upon the straightness of the reach. If D is the depth of scour in regime flow in a right angled bend, it is

a. 1.25 D

b. 1.50 D

c. 1.75 D

d. 2.00 D

37. In a braking test, a vehicle travelling at 36 km ph was stopped at a braking distance of 8.0 m. The average value of the vehicle's skid resistance (friction coefficient) is

a. 0.64

b. 6.25

c. 0.16

d. none of these

38. Formation width of a hill road is the distance between

a. outer end of side drain to outer end of parapet wall

b. outer end of side drain to inner end of parapet wall

c. inner end of side drain to outer end of parapet wall

d. inner end of side drain to inner end of parapet wall

39. On earth roads, the camber should preferably be

a. 1 in 20 to 1 in 24

b. 1 in 30 to 1 in 48

c. 1 in 15 to 1 in 20

d. 1 in 10 to 1 in 15

40. The distance travelled by a moving vehicle during perception and brake reaction times, is known as

a. sight distance

b. stopping distance

c. lag distance

d. none of these.

41. The measurement is made in square metre in case of

a. R.C.C. structure

b. Hollow concrete block wall

c. Concrete fencing posts

d. None of these.

42. A portion of an embankment having a uniform up-gradient 1 in 500 is circular with radius 1000 m of the centre line. It subtends 180° at the centre. If the height of the bank is 1 m at the lower end, and side slopes 2:1, the earth work involved.

a. 26, 000 m3

b. 26, 500 m3

c. 27, 000 m3

d. 27, 500 m3

43. The area is measured correct to the nearest

a. 0.01 sqm

b. 0.02 sqm

c. 0.03 sqm

d. 0.04 sqm

44. The assumption on which the trapezoidal formula for volumes is based, is

a. The end sections are parallel planes

b. The mid-area of a pyramid is half the average area of the ends

c. The volume of the prismoidal is over-es-timted and hence a prismoidal correction is applied

d. All the above.

45. Railway projects are treated as

a. light construction

b. heavy construction

c. industrial construction

d. none of these.

46. Pick up the correct statement from the following:

a. If the float is positive and the activity is delayed by a period equal to its total float, the completion of project is not delayed

b. If the float of an activity is negative, delay in its performance is bound to delay the completion of project

c. If the float of an activity is zero, the activity is critical and any delay in its performance will delay the whole project

d. All the above.

47. Time and progress chart of a construction, is also known as

a. Gantt chart

b. Modified Mile stone chart

c. Critical path method chart

d. All the above.

48. Site order book is used for recording

a. instructions by the executive engineers

b. construction measurements

c. issue of store equipments

d. names of the casual labour.

49. In instrument landing system

a. L.O.M. and L.M.M. are installed on one side and Localizer antenna on the other side

b. L.O.M. and Localizer are installed on one side and L.M.M. on the other side

c. Localizer and L.M.M. are installed on one side and L.O.M. on the other side

d. None of these.

50. From the end of an instrumental runway, the approach surface rises outwards

a. 1 in 20

b. 1 in 30

c. 1 in 40

d. 1 in 50