***Sub Engineer***

1. Diopter is the power of a lens having a focal length of

a. 50 cm

b. 75 cm

c. 100 cm

d. 125 cm

2. A lemniscate curve will not be transitional throughout, if its deflection angle, is

a. 45°

b. 60°

c. 90°

d. 120°

3. If θ is the slope of the ground and l is the measured distance, the correction is

a. 2l sin2 θ/2

b. 2l cos2 θ/2

c. 2l tan2 θ/2

d. 2l cot2 θ/2.

4.  is the conventional sign of

a. temple

b. mosque

c. idgah

d. church

5. In the cement the compound quickest to react with water, is

a. Tricalcium aluminate

b. Tetra-calcium alumino-ferrite

c. Tricalcium silicate

d. Dicalcium silicate.

6. Duco is one of the patent forms of

a. plastic paints

b. bituminous paints

c. aluminium paints

d. cellulose paints.

7. A good brick earth should contain :

a. about 50% to 60% of silica

b. not more than 5% of lime

c. about 5 to 6% of oxide of lime

d. All the above.

8. The frog of a brick is normally made on its

a. top face

b. bottom face

c. longer face

d. shorter side.

9. The force in BC of the truss shown in the given figure, is

a. 3.0t tension

b. t tension

c. t compression

d. None of these.

10. The eccentricity (e) of a hollow circular column, external diameter 25 cm, internal diameter 15 cm for an eccentric load 100 t for non-development of tension, is

a. 3.00 cm

b. 3.50 cm

c. 4.25 cm

d. 5.0 cm

11. The ratio of the effective length of a column and minimum radius of gyration of its cross-sectional area, is known

a. buckling factor

b. slenderness ratio

c. crippling factor

d. none of these.

12. If the width b and depth d of a beam simply supported with a central load are interchanged, the deflection at the centre of the beam will be changed in the ratio of

a. d/b

b. (d/b)2

c. (b/d)2

d. (b/d)3.

13. Fluids change the volume under external presssure due to

a. plasticity

b. viscosity

c. compressibility

d. none of these.

14. A body of dimensions 1.5 m x 1.0 m x 2 m weighs 3000 kg in water. Its specific gravity is

a. 0.8

b. 0.9

c. 1.0

d. 1.1

15. Uniform flow is said to occur when

a. size and shape of the cross-section in a particular length remain constant

b. size and shape of the cross-section change along a length

c. frictional loss in the particular length of the channel will the more than the drop in its elevation

d. frictional loss in the particular length of the channel, will be less than the drop in elevation.

16. An independent mass of a fluid does not posses

a. elevation energy

b. kinetic energy

c. pressure energy

d. none of these.

17. 260 g of wet soil was taken in a pycnometer jar of weight 400 g in order to find the moisture content in the soil, with specific gravity of soil particles 2.75. The weight of soil and remaining water filled in pycnometer without air bubbles was 1415 g and the weight of pycnometer filled with water alone was 1275 g. The moisture content in the soil is

a. 24.2%

b. 18.2%

c. 53.8%

d. none of these.

18. Pick up the correct statement from the following:

a. The permeability of the coarse-grained soils may be reduced by grouting.

b. The process of injecting fluids (i.e. grouts) into the pores space of the soil, is called grouting.

c. The grouting increases the soil strength.

d. All the above.

19. According to IS : 2720 - 1965, the composition of a dispersing solution used in pipette analysis for determining the size of particles, is

a. sodium-hexametaphosphate 33 g, sodium carbonate 7 g and distilled water one litre

b. sodium-hexametaphosphate 7 g, sodium carbonate 33 g and distilled water one litre

c. sodium-hexametaphosphate 23 g, sodium carbonate 17 g and distilled water one litre

d. none of these.

20. The soil moisture driven off by heat, is called

a. free water

b. hydroscopic water

c. gravity water

d. none of these.

21. The number of treads in a flight is equal to

a. risers in the flight

b. risers plus one

c. risers minus one

d. none of these.

22. A foundation rests on

a. base of the foundation

b. subgrade

c. foundation soil

d. both (b) and (c)

23. If the bearing capacity of soil is 10 tonnes/cm2 and the projection of plain concrete footing from walls, is a cm, the depth D of footing is

a. D = 0.0775 a

b. D = 0.775 a

c. D = 0.775a

d. D = 0.775 a2

24. The angle of repose of a soil is the maximum angle which the outer face of the soil mass makes

a. with the horizontal

b. with the vertical

c. with the perpendicular to. the inclined plane of the soil

d. none of these.

25. The vertical faces of a door opening which support frame of the door, are

a. jambs

b. posts

c. reveals

d. styles.

26. Auger boring

a. is generally employed in cohesive and other self soils above water table

b. is most economical upto a depth of 5 metres

c. is done by portable power driven helical augers those diameters range from 7.5 to 30 cm

d. all the above.

27. Strength of concrete with passage of time

a. increases

b. decreases

c. fluctuates

d. remains constant.

28. If aggregates completely pass through a sieve of size 75 mm and are retained on a sieve of size 60 mm, the particular aggregate will be flaky if its minimum dimension is less than

a. 30.5 mm

b. 40.5 mm

c. 50.5 mm

d. none of these.

29. Pick up the correct statement from the following regarding the pressure conduits :

a. Pressure conduits are permitted to run  th full

b. Pressure conduits are always laid along down grades

c. The hydraulic gradient line always coincides the invert of the conduit

d. None of these.

30. Pick up the correct statement from the following :

a. The amount of rainfall in one year, is known as annual rainfall

b. The rain cycle period in India is taken as 35 years

c. The ratio of the actual rainfall in the year to the normal rainfall at a place, is called index of the wetness

d. All the above.

31. If D.O. concentration falls down to zero in any natural drainage, it indicates the zone of

a. active decomposition

b. recovery

c. cleaner water

d. none of these.

32. For house drainage minimum gradient is

a. 1 in 60

b. 1 in 80

c. 1 in 10

d. 1 in 400

33. A current meter measures the velocity of flow, if it is held

a. on the surface of water

b. at the bottom of channel

c. at any point within the cross-section

d. none of these.

34. Irrigation canals are generally aligned along

a. ridge line

b. contour line

c. valley line

d. straight line.

35. In rigid module, the discharge

a. is independent of water levels in the distributary and water course

b. depends upon the water level in distributary

c. depends upon the water level in the water course

d. depends upon the water levels of both, i.e. distributary and water course

36. In Montague type fall

a. a straight glacis is provided

b. a circular glacis is provided

c. a parabolic glacis is provided

d. no glacis is provided.

37. Following type of pavement is generally known as flexible pavement

a. water-bound macadam roads

b. stabilised soil roads

c. road constructed with various layers of building material well compacted

d. all the above.

38. Degree of a road curve is defined as the angle in degrees subtended at the centre by an arc of

a. 20 metres

b. 25 metres

c. 30 metres

d. 50 metres

39. Longitudinal pavement lines marked broken in white paint

a. are for the guidance of the drivers

b. are not to be crossed over

c. may be crossed over at the discretion of the driver

d. all the above.

40. Transverse joints are provided at distances varying from

a. 12 m to 18 m

b. 16 m to 24 m

c. 17 m to 27 m

d. 25 m to 35 m

41. The height of the sink of wash basin above floor level is kept

a. 60 cm

b. 70 cm

c. 75 cm to 80 cm

d. 80 cm

42. Cost of fittings and their fixing is specified for the following sanitary fittings

a. Flushing pipes

b. Lavatary basins

c. Sinks

d. All the above.

43. The following item of earth work is not measured separately.

a. Site clearence

b. dead men

c. Steps in deep exavaction

d. All the above.

44. If the formation level of a highway has a uniform gradient for a particular length, and the ground is also having a longitudinal slope, the earthwork may be calculated by

a. Mid-section formula

b. Trapezoidal formula

c. Prismoidal formula

d. All the above.

45. The object of technical planning, is

a. preparation of estimates

b. initiating the procurement action of resources

c. taking remedial action for likely bottleneck in the execution

d. all the above.

46. Time and progress chart of a construction, is also known as

a. Gantt chart

b. Modified Mile stone chart

c. Critical path method chart

d. All the above.

47. Pick up the correct statement from the following:

a. Latest occurrence time is denoted by TL

b. Contractual obligation time is denoted by Ts

c. Latest occurrence time is taken as contractual obligation time

d. All the above.

48. Pick up the correct statement from the following with regards to C.P.M. network analysis of projects

a. Earliest occurrence time of the event from which the activity arrow originates plus the duration of the activity, is called earliest finish time of the activity

b. The latest occurrence time of the node of which the activity arrow terminates minus the duration of the activity, is called latest start time

c. The latest occurrence time for the node at which the activity arrow terminates, is called latest finish time

d. All the above.

49. The depressions and undulations in the pavement, are caused due to

a. impact of heavy wheel loads

b. punching effect

c. flexible gears

d. all the above.

50. The thickness design of the pavement, is decided on the load carried by

a. main gears

b. nose wheel

c. tail wheel

d. all the above.