***Sub Engineer***

1. While rotating the theodolite in the horizontal plane, the bubble of the bubble tube takes up the same position in its tube, it indicates

a. the rotation axis is vertical

b. the trummion axis is horizontal

c. the line of collimation is perpendicular to vertical axis

d. none of the above

2. In chain surveying field work is limited to

a. linear measurements only

b. angular measurements only

c. both linear and angular measurements

d. all the above.

3. With usual notations, the expression  represents

a. centrifugal force

b. centrifugal ratio

c. super elevation

d. radial acceleration.

4. The angle between two plane mirrors of optical square, is

a. 30°

b. 45°

c. 60°

d. 90°.

5. Pick up the correct statement from the following:

a. Blistering may be cured by applying water paint finished with oil paint dried with a little copal varnish

b. Cracked paints may be cured by removing paint and giving a fresh coat of paint

c. Crawling paints may be cured by sand preparing the surface and giving a fresh coat with plenty of turps

d. All the above.

6. The weight of 1 m3 of brick earth, is about

a. 1200 kg

b. 1500 kg

c. 1800 kg

d. 2000 kg.

7. Inner part of a timber log surrounding the pith, is called

a. sapwood

b. cambium layer

c. heart wood

d. none to these.

8. Name the type of cement from the following for canal linings :

a. sulphate resisting cement

b. rapid hardening cement

c. quick setting cement

d. pozzuolana cement.

9. A truss containing j joints and m members, will be a simple truss if

a. m = 2j - 3

b. j = 2m - 3

c. m = 3j - 2

d. j = 3m - 2

10. The locus of the end point of the resultant of the normal and tangential components of the stress on an inclined plane, is

a. circle

b. parabola

c. ellipse

d. straight line.

11. When loads are applied proportionately to a frame structure containing its members in one plane, the structure is called

a. grid frame

b. plane frame

c. space frame

d. truss frame.

12. The value of Poisson's ratio always remains

a. greater than one

b. less than one

c. equal to one

d. none of these.

13. Kinematic viscosity of liquids by equation  is determined by :

a. Redwood viscometer

b. Engler viscometer

c. Saybolt universal viscometer

d. None of these.

14. In C.G.S. system the unit of visocity is

a. joule

b. poise

c. Newton

d. none of these.

15. If cohesion between the molecules of a fluid is more than adhesion between the fluid and glass, the free level of fluid in a dipped glass tube will be

a. higher than the surface of liquid

b. same as the surface of liquid

c. lower than the surface of liquid

d. none of these.

16. A steady uniform flow is through

a. a long pipe at constant rate

b. an expanding tube at constant rate

c. an expanding tube at increasing rate

d. a long pipe at increasing rate.

17. The Terzaghi's general bearing capacity equation for a continuous footing is given by (where Nc, 2V& and Ny are bearing capacity factors.)

a. qf = cNc + γDNq + 0.5γBNγ

b. qf = cNc - γDNq + 0.5γBNγ

c. qf = cNc + γDNq - 0.5γBNγ

d. qf = cNc - γDNq - 0.5γBNγ

18. The compression index of a soil

a. decreases with an increase in the liquid limit

b. increases with an increase in the liquid limit

c. decreases with an increase in the plastic limit

d. is not related with plastic limit.

19. The angle of internal friction of clays, is usually

a. 0° to 5°

b. 5° to 20°

c. 20° to 30°

d. 30° to 45°

20. A partially saturated soil is classified as

a. one phase soil

b. two phase soil

c. three phase soil

d. four phase soil.

21. An intermediate T-beam reinforced with two layers of tensile steel with clear cover 13 cm encasted with the floor of a hall 12 metres by 7 metres, is spaced at 3 metres from adjoining beams and if the width of the beam is 20 cm, the breadth of the flange is

a. 233 cm

b. 176 cm

c. 236 cm

d. 255 cm

22. To ensure that the hogging bending moment at two points of suspension of a pile of length L equals the sagging moment at its centre, the distances of the points of suspension from either end, is

a. 0.107 L

b. 0.207 L

c. 0.307 L

d. 0.407 L

23. A foundation is called shallow if its depth, is

a. half of its width

b. three-fourth of its width

c. equal to its width

d. all the above.

24. The stresses developed in concrete and steel in reinforced concrete beam 25 cm width and 70 cm effective depth, are 62.5 kg/cm2 and 250 kg/cm2 respectively. If m = 15, the depth of its neutral axis is

a. 25 cm

b. 30 cm

c. 35 cm

d. 40 cm

25. The type of pointing in which a V-shaped projection outside the wall surface, is provided, is called

a. recessed pointing

b. weather pointing

c. V-pointing

d. tuck pointing.

26. While investigating the site, a thick layer of fairly firm clay over a deep layer of soft clay is encountered. In such a situation, the following type of foundation is useful :

a. pile formation

b. raft foundation

c. grillage foundation

d. none of these.

27. Curing

a. reduces the shrinkage of concrete

b. preserves the properties of concrete

c. prevents the loss of water by evaporation

d. all of the above.

28. Segregation is responsible for

a. porous layers in concrete

b. surface scaling in concrete

c. sand streaks in concrete

d. all the above.

29. The equation  in which Q is yearly run off in cm, P is yearly total rainfall in cm, H is difference of R.L.s. of lowest and highest points and A is area of catchment in square metres, is known as

a. English formula

b. Khosla's formula

c. Justin's formula

d. Vermule's formula.

30. Water may not contain much impurities if its source is

a. stream flowing in plains

b. lakes in lower regions

c. spring along hill slopes

d. none of these.

31. To prevent settling down of sewage both at the bottom and on the sides of a large sewer, self-cleaning velocity recommended for Indian conditions, is

a. 0.60 m/sec

b. 0.70 m/sec

c. 0.75 m/sec

d. 1.0 m/sec.

32. For laying a sewer line in a trench of 2 m width, an offset line is marked on the ground parallel to the given centre line at a distance of

a. 120 cm

b. 140 cm

c. 160 cm

d. 180 cm.

33. The main function of a diversion head works of a canal from a river, is

a. to control floods

b. to store water

c. to raise water level

d. all the above.

34. If A is the area of the surface, x is the depth of its C.G. from the surface of the water and ω is the density of water, then

a. total pressure on the surface is equal to ω x

b. depth of the point at which total pressure acts is equal to its moment of inertia divided by A x

c. depth of the centre of pressure is 2/3H vertically below the surface

d. all the above.

35. The length of a meander is the distance along the river between the tangent point of one curve to the tangent point of

a. reverse curve

b. next curve of the same order

c. reverse curve plus the width of the river

d. none of these.

36. The useful moisture of soil, is equal to its

a. field capacity

b. saturation capacity

c. moisture content at permanent wilting point

d. difference between filed capacity and permanent wilting point within the root zone of plants.

37. The G.T. road from Lahore to Calcutta in undivided India, was constructed during

a. 3495-3500 BC

b. 265-270 BC

c. 1540-1545 AD

d. 1440-1450

38. Design of flexible pavement, involves

a. intensity of traffic

b. climate of the region

c. sub-grade conditions

d. all the above.

39. Pick up the correct statement from the following:

a. Various geometric design features and generally guided by ruling design speed

b. The design speed for a given highway should preferably be uniform

c. Abrupt change in the design speed should not be permitted

d. all the above.

40. If the velocity of moving vehicles on a road is 24 km/per hour, stopping distance is 19 metres and average length of vehicles is 6 metres, the basic capacity of lane, is

a. 500 vehicles per hour

b. 700 vehicles per hour

c. 1000 vehicles per hour

d. 1250 vehicles per hour

41. The expected out turn of half brick partition wall per mason per day is

a. 1.5 m3

b. 2.0 m3

c. 4.0 m2

d. 5.0 m2

42. Pick up the correct statement from the following:

a. Plastering is measured in sq.m

b. Glazing is measured in sq.m

c. Striking is measured in sq.m

d. All the above.

43. If B is the width of formation, d is the height of the embankment, side slope S : 1, for a highway with no transverse slope, the area of cross-section is

a. B + d + Sd

b. Bd + Sd2

c. B x d - Sd1/2

d. 12(Bd + Sd2)

44. Size, capacity and materials need be specified for

a. Stop-cocks

b. Ferrules

c. Ball valves

d. All the above.

45. PERT is

a. limited of event oriented diagrams

b. used for research and development projects

c. based on three time estimates for activities linking up two events

d. all the above.

46. Pick up the PERT event from the following:

a. Digging of foundation completed

b. Laying of concrete started

c. Laying of concrete completed

d. All the above.

47. Completion of an activity on CPM network diagram, is generally known

a. Event

b. Node

c. Connector

d. All the above.

48. The most popular type of organisation used for Civil Engineering Constructions, is

a. line organisation

b. line and staff organisation

c. functional organisation

d. effective organisation.

49. Pick up the correct statement from the following:

a. Minimum turning radius of aircrafts decides the size of the apron and the radius of the curves at taxi-ends

b. Take off and landing distances for an aircraft, determine the minimum runway length

c. The length of the normal haul of the air craft decides the frequency of operation

d. All the above.

50. The meterological condition which influences the size and location of an air port is

a. air density

b. reduced level

c. wind direction

d. all the above.