***Sub Engineer***

1. The sum of the interior angles of a geometrical figure laid on the surface of the earth differs from that of the corresponding plane figure only to the extent of one second for every

a. 100 sq. km of area

b. 150 sq. km of area

c. 200 sq. km of area

d. none of these.

2. Which one of the following statements is correct ?

a. when the axes of rotation of the graduated circle and the verniers are not coincident, the instrument possesses eccentricity

b. the mean of the readings of the two verniers gives correct reading free from the eccentricity

c. one vernier may be used if the readings of two verniers differ by a constant

d. all the above.

3. Prolongation of chain line across an obstruction in chain surveying, is done by

a. making angular measurements

b. drawing perpendiculars with a chain

c. solution of triangles

d. all the above.

4. Ranging in chain survey means

a. establishing an intermediate point on the line

b. determining the distance between end points

c. determining the offset distance

d. none of these

5. Bitumen paints offer

a. pleasing surface

b. hard surface

c. smooth surface

d. protective surface

e. rough surface.

6. During pudding

a. carbon is converted into carbonic acid gas

b. silicon forms a slag

c. metal is heated by the burning of gases

d. all the above.

7. Cast iron contains carbon approximately

a. 1.5% to 5.5%

b. 0.05% to 1.75%

c. 0.250%

d. none to these.

8. Bitumen may be dissolved in

a. water

b. sodium chloride

c. carbon disulphide

d. none of these.

9. The ratio of the area of cross-section of a circular section to the area of its core, is

a. 4

b. 8

c. 12

d. 16

10. Stress may be defined as

a. force per unit length

b. force per unit volume

c. force per unit area

d. none of these.

11. The tension coefficient of any member is

a. force divided by the length

b. tension divided by the length

c. tension per unit area

d. tension in the member.

12. If a three hinged parabolic arch carries a uniformly distributed load on its entire span, every section of the arch resists.

a. compressive force

b. tensile force

c. shear force

d. bending moment.

13. Mercury is generally used in barometers because

a. the height of the barometer will be less

b. it is a best liquid

c. both (a) and (b) above

d. both (b) and (c) above.

14. Which one of the following statements is true ?

a. The value of kinetic energy correction factor for turbulent flow lies between 1.03 to 1.06

b. The value of kinetic energy correction factor for laminar flow is 2

c. The practical value of kinetic energy correction factor for turbulent flow is unity

d. all the above.

15. Pick up the incorrect statement from the following :

a. In radial flow, fluid flows such that pressure and velocity at any point change with respect to its distance from the central axis

b. In radial flow, velocity of flow is in a radial direction

c. In radial flow, flow may take place radially inward to or outward from the centre

d. In radial flow, flow is one dimensional with stream lines parallel.

16. A syphon is used

a. to connect water reservoirs at different levels intervened by a hill

b. to supply water to a town from higher level to lower level

c. to fill up a tank with water at higher level from a lower level

d. none of these.

17. If the specific gravity of a soil particle of 0.05 cm diameter is 2.67, its terminal velocity while settling in distilled water of viscosity, 0.01 poise, is

a. 0.2225 cm/sec

b. 0.2250 cm/sec

c. 0.2275 cm/sec

d. 0.2300 cm/sec

18. The maximum net pressure intensity causing shear failure of soil, is known

a. safe bearing capacity

b. net safe bearing capacity

c. net ultimate bearing capacity

d. ultimate bearing capacity.

19. In a purely cohesive soil, the critical centre lies at the intersection of

a. perpendicular drawn at l/3rd slope from toe and the locus of the centre

b. perpendicular drawn at 2/3rd slope from toe and the locus of the centre

c. directional angles

d. none of these.

20. Pick up the correct statement from the following:

a. An unconfined compression test is a special case of triaxial compression test

b. An unconfined compression test is a special case of direct shear test

c. The confining pressure is maximum during an unconfined compression test

d. The cylindrical specimen of a soil is subjected to major principal stress till it fails due to shearing along the plane of the failure.

21. The maximum ratio of span to depth of a cantilever slab, is

a. 10

b. 12

c. 14

d. 16

22. Pick up the incorrect statement from the following: Tensile reinforcement bars of a rectangular beam

a. are curtailed if not required to resist the bending moment

b. are bent up at suitable places to serve as shear reinforcement

c. are bent down at suitable places to serve as shear reinforcement

d. are maintained at bottom to provide at least local bond stress.

23. For initial estimate for a beam design, the width is assumed

a. 1/20th of span

b. 1/25th of span

c. 1/30th of span

d. 1/40th of span.

24. According to I.S. : 456, slabs which span in two directions with corners held down, are assumed to be divided in each direction into middle strips and edge strips such that the width of the middle strip, is

a. two-third of the width of the slab

b. three-fourth of the width of the slab

c. four-fifth of the width of the slab

d. three-fifth of the width of the slab.

25. The additional piles which are driven to increase the capacity of supporting loads on vertical piles, are known

a. raking piles

b. eccentric piles

c. sinking piles

d. none of these.

26. You are asked to design and supervise a truss for a factory to have spans 6 m to 9 m. The type of the truss you will use, is

a. queen post truss

b. king post truss

c. collar truss

d. none of these.

27. Pozzolanic properties exist in

a. fly ash

b. pumicite

c. diatomaceous clay

d. all the above.

28. The produce impermeable concrete

a. proper compaction of concrete is required

b. proper curing of concrete is required

c. properly graded and non-porous aggregates are requied

d. all the above.

29. Steel pipe are

a. connected by riveted or welded joints

b. generally laid under ground and no expansion joint is required

c. likely to last 100 years under ordinary conditions

d. None of these.

30. In a reservoir the average volume of sediment deposition is 0.15 million cubic per year. If the dead storage and total capacity of the reservoir are 8 million cubic metres and 36 million cubic metres respectively,

a. the reservoir will theoretically be silted in 240 years

b. the reservoir will start reducing after 60 years

c. both (a) and (b)

d. neither (a) nor (b).

31. If D is the diameter of upper circular portion, the overall depth of New Egg shaped sewer section, is

a. 1.250 D

b. 1.350 D

c. 1.425 D

d. 1.625 D.

32. The disintegrating pump in which solid matter is broken up before passing out, is

a. centrifugal pump

b. reciprocating pump

c. pneumatic ejector

d. none of these.

33. The depth of the crest of a scouring sluice below the crest of a head regulator, is generally kept

a. 1.20 m

b. 2.20 m

c. 3.20 m

d. 4.90 m

34. The most suitable section of a lined canal, is

a. trapezoidal section with rounded corners for large canals

b. rectangular section with rounded corners for large canals

c. both (a) and (b)

d. both (a) and (c)

35. A land is said to be water-logged if its soil pores within

a. a depth of 40 cm are saturated

b. a depth of 50 cm are saturated

c. root zone of the crops are saturated

d. all the above.

36. A fall which maintains its depth, is

a. a trapezoidal notch fall

b. a low weir fall

c. a rectangular notch fall

d. all the above.

37. The radius of curvature provided along a transition curve, is

a. minimum at the beginning

b. same throughout its length

c. equal to the radius of circular curve

d. varying from infinity to the radius of circular curve.

38. Set-back distance is the distance between

a. road land boundary and building line

b. road land boundary and control line

c. building line and control line

d. road land boundary and control line.

39. For a properly designed vehicle, the resistance generally ignored, is

a. rolling resistance

b. grade resistance

c. axle resistance

d. none of these.

40. Along high ways confirmatory route markers are generally fixed

a. before the crossing on the left side

b. after the crossing on the left side

c. before the crossing on the right side

d. after the crossing on the right side.

41. The main factor to be considered while preparing a detailed estimate, is

a. Availability of materials

b. Transportation of materials

c. Location of site and local labour charges

d. All the above.

42. A portion of an embankment having a uniform up-gradient 1 in 500 is circular with radius 1000 m of the centre line. It subtends 180° at the centre. If the height of the bank is 1 m at the lower end, and side slopes 2:1, the earth work involved.

a. 26, 000 m3

b. 26, 500 m3

c. 27, 000 m3

d. 27, 500 m3

43. The diameter of a domestic sewer pipe laid at gradient 1 in 100 is recommended

a. 100 mm

b. 150 mm

c. 200 mm

d. 175 mm

44. In long and short wall method of estimation, the length of long wall is the centre to centre distance between the walls and

a. breadth of the wall

b. half breadth of wall on each side

c. one fourth breadth of wall on each side

d. None of these.

45. Residential buildings are treated as

a. light construction

b. heavy construction

c. industrial construction

d. private construction.

46. Pick up the incorrect statement from the following:

a. The various functions under each activity, are shown by one bar on Bar Charts

b. Bar chart establishes the interdependency of one event on another

c. Only approximate percentage of the completed work is reported

d. None of these,

47. Henry Gantt developed Bar charts for planning and scheduling of projects in

a. 1900

b. 1920

c. 1940

d. 1950

48. Various activities of a project, are shown on bar charts by

a. vertical lines

b. horizontal lines

c. dots

d. crosses.

49. The length of a runway under standard atmospheric conditions is 1800 m. If the actual reduced level of the site is 1200 m, the design length of the runway is

a. 2360 m

b. 2460 m

c. 2560 m

d. 2660 m

50. The fuse large includes

a. passengers chamber

b. pilot's cabin

c. tail of aircraft

d. all the above.