***Sub Engineer***

1. The latitude of a traverse leg is obtained by multiplying its length by

a. tangent of its reduced bearing

b. sign of its reduced bearing

c. cosine of its reduced bearing

d. cosecant of its reduced bearing.

2. Volume of the earth work may be calculated by

a. end areas

b. Prismoidal formula

c. Trapezoidal

d. all the above.

3. Hydrographic surveys deal with the mapping of

a. large water bodies

b. heavenly bodies

c. mountaineous region

d. canal system

4. The boundary of water of a still lake, represents

a. level surface

b. horizontal surface

c. contour line

d. a concave surface.

5. The minimum percentage of silica, alumina and ferric oxide in lime for white washing, is

a. 15

b. 10

c. 5

d. 0.

6. Pick up the correct statement from the following:

a. The bulking of fine sand is more than that of coarse sand

b. If the percentage content of moisture exceeds 10%, increase in bulk of sand starts increasing

c. The volume of fully saturated sand equals that of dry sand

d. All the above.

7. In   the   method   of   condensation polymerization,

a. low-molecular substances are removed from the high molecular substance

b. the reaction proceeds with an evolution of ammonia

c. the reaction proceeds with an evolution of hydrogen chloride

d. all the above.

8. Portland cement manufactured from pure white chalk and clay but free from iron-oxide, is known as

a. quick setting cement

b. rapid hardening cement

c. white cement

d. low heat Portland cement.

9. A cantilever of length 2 cm and depth 10 cm tapers in plan from a width 24 cm to zero at its free end. If the modulus of elasticity of the material is 0.2 x 106 N/mm2, the deflection of the free end, is

a. 3 mm

b. 4 mm

c. 5 mm

d. 6 mm

10. The tangential component of stress on an plane inclined θ° to the direction of the force, may be obtained by multiplying the normal stress by

a. cos θ

b. tan θ

c. sin θ cos θ

d. sin2θ

11. For structural analysis, Maxwell's reciprocal theorem can be applied to :

a. plastic structures

b. elastic structures

c. symmetrical structures

d. all the above.

12. The maximum twisting moment a shaft can resist, is the product of the permissible shear stress and

a. moment of inertia

b. polar moment of inertia

c. polar modulus

d. modulus of rigidly.

13. If velocities of fluid particles vary from point to point in magnitude and direction, as well as from instant to instant, the flow is said to be

a. laminar

b. turbulent flow

c. uniform flow

d. non-uniform flow.

14. Atmospheric pressure is equal to water column head of

a. 9.81 m

b. 5.0 m

c. 10.30 m

d. 7.5 m.

15. Cappoletti weir is a

a. rectangular weir whose length is kept 3 times the height of the water above sill

b. triangular weir whose notch angle is 90°

c. trapezoidal weir, whose sides slope 1 horizontal to 2 verticals

d. a combination of rectangular and triangular weirs.

16. In Chezy's formula V = C mi

a. V is the mean velocity of flow

b. m is the hydraulic mean depth

c. i is the loss of head per unit length of pipe

d. All the above.

17. Through a point in a loaded soil mass, there exists n typical planes mutually orthogonal on which the stress is wholly normal and no shear stress acts, if n is

a. 1

b. 2

c. 3

d. 4

18. Pick up the correct statement from the following:

a. When stress decreases, void, ratio decreases

b. When stress decreases, coefficient of permeability decreases

c. When stress decreases, coefficient of volume change decreases

d. When stress decreases void ratio, co-effi-cients of per-meability and volume change decrease.

19. An unsaturated 100 cm3 sample of soil weighs 190 g. If its dried weight is 160 g, water content of the soil, is

a. 0.188

b. 0.288

c. 0.388

d. 0.488

20. The active earth pressure of a soil is proportional to (where φ is the angle of friction of the soil)

a. tan (45° - φ)

b. tan2 (45° + φ/2)

c. tan2 (45° - φ/2)

d. tan (45° + φ)

21. If At is the gross area of steel in tension, d is the effective depth of the beam and y is the depth of the centre of gravity of the resultant compression, the moment of resistance M of the beam, is given by

a. M = Atf(d - y)

b. M = Atf(d + y)

c. M =

d. none of these.

22. Pick up the true statement from the following:

a. In the absence of beams, it is easier to install piping

b. In the absence of beams, it is easier to paint

c. A flat slab is capable to withstand concentrated loads

d. All the above.

23. The diameter of main bars in R.C.C. columns, shall not be less than

a. 6 mm

b. 8 mm

c. 10 mm

d. 12 mm

24. The toe projection of foundation slabs is taken

a. as one third of the base

b. as one sixth of overall height of the wall

c. equal to heel slab

d. below ground surface.

25. The brick laid with its breadth parallel to the face of a wall, is known as

a. header

b. stretcher

c. closer

d. none of these.

26. Dado is usually provided in

a. bath rooms

b. living rooms

c. verandah

d. roofs.

27. The increased cohesiveness of concrete, makes it

a. less liable to segregation

b. more liable to segregation

c. more liable to bleeding

d. more liable for surface scaling in frosty weather

28. Pick up the correct proportions of chemical ingredients of cement

a. Lime : Silica : Alumina : Iron oxide : 63 : 22 : 6 : 3

b. Silica : Lime : Alumina : Iron oxide : 63 : 22 : 6 : 3

c. Alumina : Silica : Lime : Iron oxide : 63 : 22 : 6 : 3

d. Iron oxide : Alumina : Silica : Lime : 63 : 22 : 6 : 3

29. Asbestos pipes are

a. highly resistant to corrosion

b. high flexible to accommodate deflection upto 12°

c. very much smooth and hydraulically efficient

d. all the above.

30. Bacterias which can survive with or without free oxygen, are known

a. aerobic bacterias

b. anerobic bacterias

c. facultative bacterias

d. none of these.

31. The gradient of sewers depends upon

a. velocity of flow

b. diameter of the sewer

c. discharge

d. all the above.

32. The design period in years for pumping plants, is

a. 1

b. 2 to 3

c. 3 to 5

d. 5 to 10

33. Bligh's theory of seepage assumes

a. equal weightage to the horizontal and vertical creep

b. more weightage to horizontal creep than vertical creep

c. less weightage to horizontal creep than vertical creep

d. loss of head follows the sine curve.

34. In a Sarda type fall, the rectangular crest, may be used for discharge upto

a. 10 cumecs

b. 14 cumecs

c. 20 cumecs

d. 25 cumecs.

35. An outlet which maintains a constant discharge irrespective of fluctuation in the water levels of the supplying channel or water course, is known as

a. non-modular outlet

b. semi-modular outlet

c. flexible modular outlet

d. right modular outlet.

36. Pick up the correct statement from the following:

a. Duty of canal water is usually calculated at the head discharge of a channel

b. Duty of canal water at the head of a channel and at the outlet of a distributary, differ considerably due to progressive losses of water

c. Duty of canal water at the outlet of a distributary, is generally known as outlet factor or outlet discharge factor

d. All the above.

37. Set-back distance is the distance between

a. road land boundary and building line

b. road land boundary and control line

c. building line and control line

d. road land boundary and control line.

38. Road width 8.8 m of two lane National highways or State highways in mountainous terrain

a. excludes the width of parapet (0.6 m)

b. excludes the width of side drain (0.6 m)

c. excludes the width of parapet and side drain

d. includes the width of parapet and side drain

39. The total length of a valley formed by two gradients - 3% and + 2% curve between the two tangent points to provide a rate of change of centrifugal acceleration 0.6 m/sec2, for a design speed 100 km ph, is

a. 16.0 m

b. 42.3 m

c. 84.6 m

d. none of these.

40. Stability of hill slopes depends upon

a. angle of the slope

b. geological conditions

c. ground water conditions

d. all the above.

41. The weight of an item is measured correct to nearest

a. 0.50 kg

b. 0.75 kg

c. 1.00 kg

d. 5 kg

42. The assumption on which the trapezoidal formula for volumes is based, is

a. The end sections are parallel planes

b. The mid-area of a pyramid is half the average area of the ends

c. The volume of the prismoidal is over-es-timted and hence a prismoidal correction is applied

d. All the above.

43. The cross-sections for a highway is taken at

a. 30 metres apart

b. intermediate points having abrupt change in gradient

c. the starting end points of the curves

d. All the above.

44. For the construction of buildings, the subheads of the estimate are

a. Brickwork, Stone work, Roofing

b. Brickwork Flooring, Wood work, Steel work

c. Plastering or pointing, finishing, water supply and sanitary work

d. All the above.

45. The time by which activity completion time can be delayed without affecting the start of succeeding activities, is known as

a. duration

b. total flat

c. free float

d. interfering float.

46. The performance of a specific task in CPM, is known

a. Dummy

b. Event

c. Activity

d. Contract.

47. Completion of an activity on CPM network diagram, is generally known

a. Event

b. Node

c. Connector

d. All the above.

48. A critical ratio scheduling

a. determines the status of each activity

b. adjusts automatically changes in activity progress

c. is a dynamic system

d. none of these.

49. The height of the pilot's eye above the runway surface is assumed

a. 1 m

b. 3 m

c. 4 m

d. 5 m

50. The coordinates of the ends of a runway are (5000, 5000) and (8000, 7000). The co-ordinates of an other runway are (4600, 5100) and (7000, 5300). The co-ordinates of the A.R.P. are

a. (6500, 6000)

b. (5800, 5200)

c. (61500, 5600)

d. (8000, 7000)