***Sub Engineer***

1. In quadrantal bearing system, back bearing of a line may be obtained from its forward bearing, by

a. adding 180°, if the given bearing is less than 180°

b. subtracting 180°, if the given bearing, is more than 180°

c. changing the cardinal points, i.e. substituting N for S and E for W and vice-versa

d. none of these.

2. The power of a lens

a. is reciprocal of its focal length

b. is positive if it is a convex lens

c. is negative if it is a concave lens

d. is measured in diopter.

3. Closed contours of decreasing values towards their centre, represent

a. a hill

b. a depression

c. a saddle or pass

d. a river bed.

4. Short offsets are measured with

a. an ordinary chain

b. an invar tape

c. a metallic tape

d. a steel tape.

5. Teak wood is suitable for

a. sports articles

b. furnitures

c. railway sleepers

d. all the above.

6. Permanent magnets are made of high carbon steel and

a. 15% of cobalt

b. 20% of cobalt

c. 35% of cobalt

d. 45% of cobalt.

7. Wrought iron is used for

a. structural works in beams

b. small sized water pipes

c. columns and struts

d. none to these.

8. The normal curing period for lime mortar, is:

a. 3 days

b. 7 days

c. 10 days

d. 14 days.

9. The assumption in the theory of bending of beams, is :

a. material is isotropic

b. Young's modulus is same in tension as well as in compression

c. each layer is independent to expand or to contract

d. all the above.

10.  is the equation of Euler's crippling load, if

a. both the ends are fixed

b. both the ends are hinged

c. one end is fixed and other end is free

d. one end is fixed and other end is hinged.

11. The ratio of the flexural strengths of two square beams one placed with its two sides horizontal and the other placed with one diagonal vertical, diagonal, is

a. 2

b. 2

c. 5

d. 7

12. A column is said to be of medium size if its slenderness ratio is between

a. 32 and 120

b. 120 and 160

c. 160 and 180

d. 180 and 200

13. The equation  for the viscosity, is suggested

a. Newton

b. Chezy

c. Bezin

d. Helmholtz.

14. The value of momentum correction factor (β) for a laminar flow through a circular pipe, is

a. 2/3

b. 3/4

c. 4/3

d. 3/2

15. A stepped notch is a combination of

a. rectangular notches of different sizes

b. triangular notches of different sizes

c. rectangular and triangular notches

d. all the above.

16. The ratio of the maximum height to which a jet inclined through θ rises to its total horizontal span is

a.  sin θ

b.  cos θ

c.  tan θ

d.  cot θ.

17. According to IS : 2720 - 1965, the composition of a dispersing solution used in pipette analysis for determining the size of particles, is

a. sodium-hexametaphosphate 33 g, sodium carbonate 7 g and distilled water one litre

b. sodium-hexametaphosphate 7 g, sodium carbonate 33 g and distilled water one litre

c. sodium-hexametaphosphate 23 g, sodium carbonate 17 g and distilled water one litre

d. none of these.

18. The fundamental equation of specific gravity (G), dry density (γd), unit weight of water (γω) and void ratio (e), is

a. e =

b. G =

c. γd =

d. γω =

19. The ratio of volume of air voids to the volume of total voids, is known as

a. air content

b. percentage air voids

c. percentage voids

d. porosity.

20. If the coefficients of volume change and compressibility of a soil sample are respectively 6.75 x 10-2 and 3 x 10-2, the void ratio of the soil sample, is

a. 1.15

b. 1.20

c. 1.25

d. 1.30

21. On an absolutely rigid foundation base, the pressure will

a. be more at the edges of the foundation

b. be uniform

c. not be uniform

d. be zero at the centre of the foundation.

22. The maximum ratio of span to depth of a slab simply supported and spanning in one direction, is

a. 25

b. 30

c. 20

d. 15

23. If H is the overall height of a retaining wall retaining a surcharge, the width of the base slab usually provided, is

a. 0.4 H

b. 0.5 H

c. 0.6 H

d. 0.7 H

24. If the length of an intermediate span of a continuous slab is 5m, the length of the end span is kept

a. 4.5 m

b. 4.0 m

c. 3.5 m

d. 3.0 m

25. Dutch bond is a modification of

a. English bond

b. stretcher bond

c. header bond

d. single Flemish bond.

26. The 9 cm x 9 cm side of a brick as seen in the wall face, is generally known as

a. face

b. front

c. header

d. side.

27. Pick up the incorrect statement from the following:

a. Admixtures accelerate hydration

b. Admixtures make concrete water proof

c. Admixtures make concrete acid proof

d. Admixtures give high strength

28. Too wet concrete may cause

a. excessive laitance

b. segregation

c. lower density

d. all the above.

29. For a city developed haphazardly, the layout of distribution pipes preferred to, is

a. ring system

b. radial system

c. grid iron system

d. dead end system.

30. P.V.C. pipes can withstand pressure bead of water upto

a. 50 m

b. 75 m

c. 100 m

d. 125 m.

31. Pick up the correct statement from the following :

a. Manhole covers are made circular for the convenience of the cleaning staff

b. A manhole is classified as deep manhole if its depth is more than 1.5 m

c. A manhole is classified as shallow manhole if its depth is. less than 0.9 m

d. All the above.

32. The sewage treatment in septic tanks is due to

a. anaerobic decomposition

b. aerobic decomposition

c. parasitic decomposition

d. none of these.

33. According to Khosla, the exist gradient of surface flow

a. depends upon the b/d ratio

b. is independent of the b/d ratio

c. is independent of the depths of d/s cut off walls

d. none of these.

34. An outlet is said to be proportional if its flexibility, is

a. zero

b. less than one

c. more than one

d. one

35. The structure constructed to allow drainage water to flow under pressure through an inverted syphon below a canal, is called

a. super passage

b. aqueduct

c. super-aqueduct

d. syphon aqueduct.

36. If straight sides of a triangular section of a lined canal with circular bottom of radius R, make an angle θ with horizontal, the area of its cross-section, is

a. R(θ + cot θ)

b. R2(θ + tan θ)

c. R2(θ + cot θ)

d. R(θ + tan θ)

37. The efficiency of the brakes of a vehicle depends upon

a. condition of road surface

b. condition of the tyres

c. presence of the show moisture

d. all the above.

38. The full width of land acquired before finalising a highway, alignment is known

a. width of formation

b. right of way

c. carriage way

d. roadway.

39. If the radii of a compound curve and a reverse curve are respectively the same, the length of common tangent

a. of compound curve will be more

b. of reverse curve will be more

c. of both curves will be equal

d. none of these.

40. The type of curves generally provided on highways, is

a. critical curve

b. transition curve

c. vertical curve

d. all the above.

41. The area is measured correct to the nearest

a. 0.01 sqm

b. 0.02 sqm

c. 0.03 sqm

d. 0.04 sqm

42. The measurement is made in square metre in case of

a. R.C.C. structure

b. Hollow concrete block wall

c. Concrete fencing posts

d. None of these.

43. Pick up the correct statement from the following:

a. The earth work of cutting in trenches or borrow pits in fairly uniform ground is measured with the help of average depths of the dead men

b. The earth work in trenches or borrow pits in irregular ground is measured by taking the difference in levels before and after completion of work

c. The earth work in trenches or borrow pits, where neither a nor b is feasible, are measured from the fillings after deduction of voids

d. All the above.

44. The rate of an item of work depends on

a. Specifications of materials

b. Proportion of mortar

c. Method of construction

d. All the above.

45. The main principle of an organisation, is

a. cohegency

b. effective control at all levels

c. delegation of authority

d. all the above.

46. Henry Gantt developed Bar charts for planning and scheduling of projects in

a. 1900

b. 1920

c. 1940

d. 1950

47. The estimated time required to perform an activity, is known as

a. event

b. dummy

c. duration

d. float.

48. The technique for establishing and maintaining priorities among the various jobs of a project, is known

a. Event flow scheduling technique

b. Critical ratio scheduling

c. Slotting technique for scheduling

d. Short interval scheduling.

49. According to I.C.A.O. the recommended length of air ports is decided on

a. sea level elevation

b. standard sea level temperature (15°C)

c. effective gradient percentage

d. all the above.

50. The air is blowing at a speed of 75 knots in the direction of the aircraft flying at a ground speed of 775 knots. The air speed of the aircraft, is

a. 775 knots

b. 75 knots

c. 850 knots

d. 675 knots