***Sub Engineer***

1. Stadia techeometry was discovered by James Watt in the year.

a. 1670

b. 1770

c. 1870

d. 1900.

2. The line of sight is kept as high above ground surface as possible to minimise the error in the observed angles due to

a. shimmering

b. horizontal refraction

c. vertical refraction

d. both shimmering and horizontal refraction.

3. If the long chord and tangent length of a circular curve of radius R are equal the angle of deflection, is

a. 60°

b. 90°

c. 120°

d. 150°.

4. Pick up the wrong statement from the following :

a. the diaphragm is placed between the eyepiece and the objective but nearer to the later

b. the outer component of the objective is a double-convex lens of crown glass

c. the inner component of the objective is a flint glass, convexo-concave

d. all the above.

5. Based on flow quality, the sequence of pipes is

a. A.C. pipes, G.I. pipes, C.I. pipes, PVC pipes

b. C.I. pipes, G.I. pipes, A.C. pipes, PVC pipes

c. C.I. pipes, G.I. pipes, PVC pipes, A.C. pipes

d. None of these.

6. Good quality stones must

a. be durable

b. be free from clay

c. resist action of acids

d. all the above.

7. Name the type of cement from the following for canal linings :

a. sulphate resisting cement

b. rapid hardening cement

c. quick setting cement

d. pozzuolana cement.

8. The wedging is adopted for quarrying costly stratified rock such as

a. marble

b. limestone

c. sandstone

d. All the above.

9. If ΣH and ΣV are the algebraic sums of the forces resolved horizontally and vertically respectively, and ΣM is the algebraic sum of the moments of forces about any point, for the equilibrium of the body acted upon

a. ΣH = 0

b. ΣV = 0

c. ΣM = 0

d. all the above.

10. The shape factor of standard rolled beam section varies from

a. 1.10 to 1.20

b. 1.20 to 1.30

c. 1.30 to 1.40

d. 1.40 to 1.50

11. The stress in the wall of a cylinder in a direction normal to its longitudinal axis, due to a force acting along the circumference, is known as

a. longitudinal stress

b. hoop stress

c. circumferential stress

d. ultimate stress.

12. The section modulus of a rectangular light beam 25 metres long is 12.500 cm3. The beam is simply supported at its ends and carries a longitudinal axial tensile load of 10 tonnes in addition to a point load of 4 tonnes at the centre. The maximum stress in the bottom most fibre at the mid span section, is

a. 13.33 kg/cm2 compressive

b. 26.67 kg/cm2 tensile

c. 26.67 kg/cm2 compressive

d. none of these.

13. Cappoletti weir is a

a. rectangular weir whose length is kept 3 times the height of the water above sill

b. triangular weir whose notch angle is 90°

c. trapezoidal weir, whose sides slope 1 horizontal to 2 verticals

d. a combination of rectangular and triangular weirs.

14. Pick up the correct statement from the following :

a. When the length of the tube is equal to diameter of the internal mouth piece, the jet of liquid comes out without touching the sides of the tube

b. When the length of the tube is three times the diameter of the internal mouth piece, the jet diameter is equal to diameter of the tube.

c. both (a) and (b)

d. Neither (a) nor (b).

15. Chezy's formula is used to determine

a. head loss due to friction in pipe

b. velocity of flow in pipe

c. velocity of flow in open channels

d. none of these.

16. If the pressure at the inlet of a pipe is 90 kg/cm2 and pressure drop over the pipe line is 10 kg/cm2, the efficiency of transmission, is

a. 66.6%

b. 77.7%

c. 55.5%

d. 88.8%.

17. The passive earth pressure of a soil, is proportional to (where φ is the angle of internal friction of the soil.)

a. tan (45° - φ)

b. tan (45° + φ)

c. tan2 (45° - φ

d. tan2 (45° + φ)

18. In a purely cohesive soil, the critical centre lies at the intersection of

a. perpendicular drawn at l/3rd slope from toe and the locus of the centre

b. perpendicular drawn at 2/3rd slope from toe and the locus of the centre

c. directional angles

d. none of these.

19. The capillary rise of water

a. depends upon the force responsible

b. increases as the size of the soil particles increases

c. decreases as the size of the soil particles decreases

d. is less in wet soil than in dry soil.

20. If W1, W2, W3 and W4 are the sequential weights obtained during observations in pycnometer method for determining water content, the formula to be used, is

a. W =

b. W =

c. W =

d. W =

21. The advantage of reinforced concrete, is due to

a. fire-resisting and durability

b. economy because of less maintenance cost

c. moulding in any desired shape

d. All the above.

22. If depth of slab is 10 cm, width of web 30 cm, depth of web 50 cm, centre to centre distance of beams 3 m, effective span of beams 6 m, the effective flange width of the beam, is

a. 200 cm

b. 300 cm

c. 150 cm

d. 100 cm

23. According to load factor method, the permissible load W on a short column reinforced with longitudinal bars and lateral stirrups, is

a. Stress in concrete x area of concrete

b. Stress in steel x area of steel

c. Stress in concrete x area of concrete + Stress in steel x area of steel

d. None of these.

24. The maximum ratio of span to depth of a slab simply supported and spanning in two directions, is

a. 30

b. 35

c. 40

d. 15

25. The concrete slump recommended for columns, is

a. 25 to 50 mm

b. 25 to 75 mm

c. 75 to 125 mm

d. 50 to 100 mm.

26. A pointed arch which forms isosceles or equilateral triangle, is generally known as

a. three centred arch

b. two centred arch

c. Lancet arch

d. Bull's eye arch.

27. A concrete using an air entrained cement

a. has more resistance to weathering

b. is more plastic and workable

c. is free from segregation and bleeding

d. all the above.

28. The cement whose strength is a little lower than the ordinary cement during the first three months but attains afterwards the same strength, is known as

a. low-heat Portland cement

b. rapid hardening Portland cement

c. Portland blast slag cement

d. Portland pozzolana cement

29. For the prediction of future population of a city, the factor to be considered, is

a. births

b. deaths

c. migrants

d. all the above.

30. Pick up the correct statement from the following :

a. Excessive use of lime may kill the bacterias

b. Excessive lime when added to water, raises its pH value

c. When lime raises pH value of water to about 9.5, 100% bacterias are removed

d. All the above.

31. Pick up the correct statement from the following :

a. Thermophilic organisms digest the sludge if the temperature ranges from 40° to 60°C

b. Mesophilic organisms digest the sludge if the temperature is between 25° and 40°C

c. 90% of digestion takes place in 30 days at 29°C, the optimum mesophilic temperature

d. All the above.

32. The angle subtended by the surface of sewer water with partial flow, at sewer centre is 120°, the depth of sewerage is

a. 25 cm

b. 40 cm

c. 50 cm

d. 60 cm.

33. If A is the area of the surface, x is the depth of its C.G. from the surface of the water and ω is the density of water, then

a. total pressure on the surface is equal to ω x

b. depth of the point at which total pressure acts is equal to its moment of inertia divided by A x

c. depth of the centre of pressure is 2/3H vertically below the surface

d. all the above.

34. A hydraulic structure is designed to withstand

a. seepage forces

b. hydraulic jump

c. hydraulic pressure

d. all the above.

35. A fall in a canal bed is generally provided, if

a. ground slope exceeds the designed bed slope

b. designed bed slope exceeds the ground slope

c. ground slope is practically the same as the designed bed slope

d. none of these.

36. For cereal crops the most commonly adopted method of irrigation, is

a. free flowing method

b. check method

c. furrow method

d. sprinkling method.

37. While calculating the sight distances, the driver's eye above road surface, is assumed

a. 100 cm

b. 110 cm

c. 120 cm

d. 150 cm

38. The ability of a driver to stop the vehicle moving with the designed speed, depends upon:

a. brake reaction time

b. efficiency of the brakes

c. frictional resistance between road surface and vehicle

d. all the above.

39. If the number of lanes on the carriageway of a road is more than two, the total width of lane ways is equal to 3.0 m

a. + 0.70 m

b. + 0.80 xn

c. + 0.90 m

d. + 1.50 m

40. The distance travelled by revolving the wheel of a vehicle more than its circumferential movement, is known as

a. slip

b. skid

c. neither (a) nor (b)

d. both (a) and (b)

41. The excavation exceeding 1.5 m in width and 10 sq.m in plan area with a depth not exceeding 30 cm, is termed as

a. Excavation

b. Surface dressing

c. Cutting

d. Surface excavation.

42. The expected out turn of 12 mm plastering with cement mortar is

a. 4.0 sq m

b. 6.0 sq m

c. 8.0 sq m

d. 10 sq m

43. The item of steel work which is measured in sq.m, is

a. Rolling shutters

b. Steel doors

c. Ventilators and glazing.

d. All the above.

44. The measurement is not made in square metres in case of

a. D.P.C. (Damp proof course)

b. Form works

c. Concrete Jaffries

d. R.C. Chhajja.

45. Critical path lies along the activities having total float

a. positive

b. negative

c. zero

d. same.

46. For the execution of a project, a contractor is

a. a person

b. a firm

c. an agency

d. all the above.

47. CPM is

a. is built of activities oriented programme

b. is based on time estimate

c. is used for repetitive works

d. all the above.

48. Modular co-ordination of construction means proper

a. planning

b. designing

c. execution

d. all the above.

49. Total correction for elevation, temperature and gradient for a runway should not be more than

a. 20%

b. 25%

c. 30%

d. 35%

50. If the width of the approach area near the runway end is 150 m, the width of the approach area at a distance of 3 kilometres from runway end will be

a. 1500 m

b. 1200 m

c. 1000 m

d. 800 m