***Sub Engineer***

1. If α is the angle between the polar ray and the tangent at the point of commencement of a lemniscate curve, the equation of the curve, is

a. l = ksin 2α

b. l = ksin 3α

c. l = ktan 2α

d. l = kcos 2α

2. Contour lines of different elevations can unite to form one line, only in the case of

a. a vertical cliff

b. a saddle

c. a water shed line

d. a hill top.

3. A uniform slope was measured by the method of stepping. If the difference in level between two points is 1.8 m and the slope distance between them is 15 m, the error is approximately equal to

a. cumulative, + 0.11 m

b. compensating, ± 0.11 m

c. cumulative, - 0.11 m

d. none of these

4. The vertical angle between longitudinal axis of a freely suspended magnetic needle and a horizontal line at its pivot, is known

a. declination

b. azimuth

c. dip

d. bearing.

5. The most commonly used synthetic abrasive is

a. aluminium carbide

b. boric acid

c. silicon

d. All of these.

6. Lime stones are generally known as

a. aqueous rocks

b. sedimentary rocks

c. stratified rocks

d. all the above.

7. Asbestos is

a. an incombustible fire proof material

b. an organic substance

c. a bad insulator for sound and heat

d. all the above.

8. The coefficient of hardness of stones used in road work should be greater than

a. 10

b. 12

c. 15

d. 17.

9. A masonry dam (density = 20, 000 N/m3) 6 m high, one metre wide at the top and 4 m wide at the base, has vertical water face. The minimum stress at the base of the dam when the reservoir is full, will be

a. 75 N/m2

b. 750 N/m2

c. 7500 N/m2

d. 75000 N/m2

10. A simply supported beam which carries a uniformly distributed load has two equal overhangs. To have maximum B.M. produced in the beam least possible, the ratio of the length of the overhang to the total length of the beam, is

a. 0.207

b. 0.307

c. 0.407

d. 0.508

11. The tensile force required to cause an elongation of 0.045 mm in a steel rod of 1000 mm length and 12 mm diameter, is (where E = 2 x 106 kg/cm2)

a. 166 kg

b. 102 kg

c. 204 kg

d. 74 kg

12. For a beam having fixed ends, the unknown element of the reactions, is

a. horizontal components at either end

b. vertical components at either end

c. horizontal component at one end and vertical component at the other

d. horizontal and vertical components at both the ends.

13. In flowing liquids pitot tubes are used measure

a. discharge

b. pressure

c. velocity

d. depth.

14. When a liquid rotates at constant angular velocity about a vertical axis of a rigid body, the pressure

a. varies inversely as the altitude along any vertical line

b. varies as the square of the radial distance

c. decreases as the square of the radial distance

d. none of these.

15. The pressure rise due to water hammer depends upon

a. the length of pipe

b. time taken to close the valve

c. the elasticity of the pipe material

d. All of above.

16. For a most economical trapezoidal open channel, the half of the top width must be equal to

a. the bed width

b. one sloping side

c. the depth of flow

d. None of these.

17. Back fill with a sloping surface exerts a total active pressure Pa on the wall of height H and acts at

a. H/4 above the base parallel to base

b. H/2 above the base parallel to base

c. H/3 above the base parallel to base

d. H/5 above the base parallel to base.

18. The property of a soil which permits water to percolate through it, is called

a. moisture content

b. permeability

c. capillarity

d. none of these.

19. The neutral stress in a soil mass is

a. force per neutral area

b. force per effective area

c. stess taken up by the pore water

d. stress taken up by solid particles.

20. Pick up the correct statement from the following:

a. At critical hydraulic gradient, the saturated sand becomes quick

b. The critical gradient depends on the void ratio and the specific gravity

c. The quick sand occurs more in fine sand and silt than coarse material

d. All the above.

21. The weight of reinforced concrete, is generally taken as

a. 2300 kg/m3

b. 2400 kg/m3

c. 2500 kg/m3

d. 2800 kg/m3

22. A circular slab subjected to external loading, deflects to form a

a. semi-hemisphere

b. ellipsoid

c. parabolloid

d. none of these.

23. If the width of the foundation for two equal columns is restricted, the shape of the footing generally adopted, is

a. square

b. rectangular

c. trapezoidal

d. triangular.

24. In a singly reinforced beam, the effective depth is measured from its compression edge to

a. tensile edge

b. tensile reinforcement

c. neutral axis of the beam

d. longitudinal central axis.

25. Vertical construction joints are provided where the shearing forces are minimum in the case of

a. slabs

b. beams

c. girders

d. all of these.

26. In verandah floors outward slope is

a. 1 in 50

b. 1 in 60

c. 1 in 70

d. 1 in 100.

27. The bulk density of aggregates, depends upon

a. shape

b. grading

c. compaction

d. all the above.

28. To ensure constant moisture content in aggregates

a. height of each aggregate pile should not exceed 1.50 m

b. aggregate pile should be left for 24 hours before aggregates are used

c. conical heaps of aggregates should be avoided to prevent moisture variation

d. all the above.

29. The head against which the motor works for lifting water, is

a. maximum depression head

b. velocity head

c. frictional losses in the delivery pipe

d. all the above.

30. The strainer type tube well, is unsuitable for

a. coarse gravels

b. fine sandy strata

c. clean gravels

d. none of these.

31. In detritus tanks,

a. flow velocity is kept 0.09 m/sec

b. detention period is kept 3 to 4 minutes

c. organic and inorganic materials are separated

d. all the above.

32. The most efficient cross section of sewers in a separate sewerage system is

a. parabolic

b. circular

c. rectangular

d. new egged.

33. Pick up the correct statement from the following

a. The capacity of escapes should not be less than the capacity of the canal at its location

b. The escapes must lead the surplus water to natural drainages

c. The escapes are aligned to take advantage of contours of lower values

d. All the above.

34. The saturation line is the line up to which banks get saturated after the canal runs for some time. The saturation gradient in ordinary loam soil, is generally

a. 3 : 1

b. 4 : 1

c. 5 : 1

d. 6 : 1

35. Pick up the correct statement from the following

a. In a level crossing, a crest with its top at the canal F.S.L. is provided across the drainage at its up-stream junction with canal

b. In a level crossing a regulator is provided across the drainage at its down-stream

c. In a level crossing, a cross regulator is provided on the canal below the crossing

d. All the above.

36. If straight sides of a triangular section of a lined canal with circular bottom of radius R, make an angle θ with horizontal, the area of its cross-section, is

a. R(θ + cot θ)

b. R2(θ + tan θ)

c. R2(θ + cot θ)

d. R(θ + tan θ)

37. During last phase of the reconnaissance, details of the grade line is recorded on 2 metre poles to indicate

a. direction of the proposed alignment

b. distance between the previous and forward pegs

c. relative elevations of pegs

d. all the above.

38. Newly constructed pavement with ordinary Portland cement attains its working strength after

a. 14 days

b. 21 days

c. 28 days

d. 42 days

39. If present A.D.T. is 5000 vehicles and annual increase is 10%, the average future flow after 5 years will be

a. 6050 vehicles

b. 7050 vehicles

c. 8050 vehicles

d. 9050 vehicles

40. In retaining and breast walls, weep holes are provided at

a. 50 cm vertical height and 50 cm centre to centre horizontally

b. 100 cm vertical height and 100 cm centre to centre horizontally

c. 100 cm vertical height and 120 cm centre to centre horizontally

d. 120 cm vertical height and 100 cm centre to centre horizontally.

41. The order of booking dimensions is

a. Length, breadth, height

b. Breadth, length, height

c. Height, breadth, length

d. None of these.

42. Pick up the correct statement from the following:

a. The diameter of the pipes is the nominal diameter of internal bore

b. All pipes are measured along the centre line of the pipes in metres

c. Lead caulked joints are enumerated separately

d. All the above.

43. The measurement is not made in square metres in case of

a. D.P.C. (Damp proof course)

b. Form works

c. Concrete Jaffries

d. R.C. Chhajja.

44. While estimating the qualities for the construction of a building, the correct metric unit is

a. Cubic metre for area

b. Square metres for volume

c. Kilogram for weight

d. Litre for capacity

45. If t is the duration of an activity, t1 is the latest finish possible moment of its preceding activity and t2 is the earliest start possible moment, the independent float of the activity is

a. (t1 - t2) - t

b. t - (t1 - t2)

c. (t1 + t2) - t

d. t + (t1 - t2)

46. The performance of a specific task in CPM, is known

a. Dummy

b. Event

c. Activity

d. Contract.

47. The object of technical planning, is

a. preparation of estimates

b. initiating the procurement action of resources

c. taking remedial action for likely bottleneck in the execution

d. all the above.

48. Pick up the correct statement from the following:

a. The difference of latest occurence time and earliest expected time, is called slack

b. The activities connecting the events having zero slack, lie on the critical path

c. The critical path consumes the maximum time

d. All the above.

49. The air is blowing at a speed of 75 knots in the direction of the aircraft flying at a ground speed of 775 knots. The air speed of the aircraft, is

a. 775 knots

b. 75 knots

c. 850 knots

d. 675 knots

50. The length of a runway under standard atmospheric conditions is 1800 m. If the actual reduced level of the site is 1200 m, the design length of the runway is

a. 2360 m

b. 2460 m

c. 2560 m

d. 2660 m