***Sub Engineer***

1. The 'fix' of a plane table from three known points, is good, if

a. middle station is nearest

b. middle station is farthest

c. either the right or left station is nearest

d. none of these.

2. While working on a plane table, the correct rule is :

a. Draw continuous lines from all instrument stations

b. Draw short rays sufficient to contain the points sought

c. Intersection should be obtained by actually drawing second rays

d. Take maximum number of sights as possible from each station to distant objects.

3. The distance between steps for measuring down hill to obtain better accuracy

a. decreases with decrease of slope

b. increases with increase of slope

c. decreases with increase of slope

d. decreases with decrease of weight of the chain.

4. Prismatic compass is considered more accurate than a surveyor's compass, because

a. it is provided with a sliding glass in the object vane

b. its graduations are in whole circle bearings

c. it is provided with a prism to facilitate reading of its graduated circle

d. both (c) and (d).

5. During pudding

a. carbon is converted into carbonic acid gas

b. silicon forms a slag

c. metal is heated by the burning of gases

d. all the above.

6. Magnese steels

a. possess high electrical resistance

b. possess low coefficient of expansion

c. are used for the manufacture of rails

d. all the above.

7. Plastics are compounds of carbon with element

a. hydrogen

b. nitrogen

c. oxygen

d. All of these.

8. Oil varnish generally consists of

a. synthetic resin and spirit

b. oil, wax and resin

c. resin, oil and turpentine

d. spirit, oil and wax.

9. A steel rod 1 metre long having square cross section is pulled under a tensile load of 8 tonnes. The extension in the rod was 1 mm only. If Esteel = 2 x 106 kg/cm2, the side of the rod, is

a. 1 cm

b. 1.5 cm

c. 2 cm

d. 2.5 cm

10. The yield moment of a cross section is defined as the moment that will just produce the yield stress in

a. the outer most fibre of the section

b. the inner most fibre of the section

c. the neutral fibre of the section

d. the fibre everywhere

11. Shear force for a cantilever carrying a uniformly distributed load over its length, is

a. triangle

b. rectangle

c. parabola

d. cubic parabola.

12. A cantilever beam rectangular in cross-section is subjected to an isolated load at its free end. If the width of the beam is doubled, the deflection of the free end will be changed in the ratio of

a. 1/8

b. 1/2

c. 2

d. 3

13. Though angle of deviation of liquid is more in internal mouth piece, the contraction of the jet, is

a. more in the internal mouth piece

b. less in the internal mouth piece

c. equal to external mouth piece

d. none of these.

14. In pipe lines, a surge tank is provided

a. to relieve the pressure due to water hammer

b. to provide additional water head

c. to overflow the pipe line when suddenly closed

d. to remove the frictional loss in pipe.

15. The Empirical formula  for discharge over large rectangular weirs, is known as

a. Francis formula

b. Bazin formula

c. Rehbook formula

d. Kutter's formula.

16. For a most economical rectangular channel, the width of the channel must be

a. equal to depth of flow

b. twice the depth of flow

c. half the depth of flow

d. None of these.

17. Stoke's law does not hold good if the size of particle is smaller than

a. 0.0002 mm

b. 0.002 mm

c. 0.02 mm

d. 0.2 mm

18. The phreatic line in an earth dam may be

a. elliptical

b. parabolic

c. a straight line

d. all the above.

19. The specific gravity of quartz, is

a. 2.65

b. 2.72

c. 2.85

d. 2.90

20. For determing the specific gravity of soil solids, using a pycnometer of 500 cc., the following data is available : Weight of dry empty pycnometer = 125 g Weight of dried soil and pycnometer = 500 g Weight of dried soil and distilled = 850 g water filled in pycnometer up to top The specific gravity of soil soilds, is

a. 2.0

b. 2.25

c. 2.50

d. 2.75

21. The Young's modulus of elasticity of steel, is

a. 150 KN/mm2

b. 200 KN/mm2

c. 250 KN/mm2

d. 275 KN/mm2

22. The minimum cube strength of concrete used for a prestressed member, is

a. 150 kg/cm2

b. 250 kg/cm2

c. 350 kg/cm2

d. 400 kg/cm2

23. The effective width of a column strip of a flat slab, is

a. half the width of the panel

b. radius of the column

c. diameter of the column

d. none of these.

24. If the loading on a prestressed rectangular beam, is uniformly distributed, the tendon to be provided should be .

a. parabolic with convexity downward

b. parabolic with convexity upward

c. straight above centroidal axis

d. none of these.

25. The stone whose crushing strength is least, is

a. chalk

b. marble

c. slate

d. sand stone.

26. The size of a floor tile commonly used, is

a. 15 cm x 15 cm x 1.8 cm

b. 20 cm x 20 cm x 2 cm

c. 22.5 x 22.5 cm x 2.2 cm

d. all the above.

27. Pick up the correct statement from the following:

a. Sand obtained from flooded pits, need not be washed before use

b. Sea shore sand contains chlorides which cause effloresance

c. The chloride in sea shore sand and shingle may cause corrosion of reinforcement if the concrete is porous

d. All the above.

28. If the various concrete ingredients i.e. cement, sand and aggregates are in the ratio of 1:3:6, the grade of concrete, is

a. M 100

b. M 150

c. M 200

d. M 250

29. The maximum permissible hardness for public supplies is

a. 105 mg/litre

b. 115 mg/litre

c. 125 mg/litre

d. 150 mg/litre.

30. The coefficient of permeability of soils, is generally expressed in

a. cm/sec

b. cm/minute

c. cm2/sec

d. cm2/minute.

31. Setting out the alignments of sewers may start from

a. city

b. out-fall

c. tail end

d. any point.

32. In septic tanks, decomposition of organic bacteria, is done by

a. anaerobic bacteria

b. aerobic bacteria

c. both types of bacterias

d. none of these.

33. For a unique design of a channel by Kennedy's theory

a. its breadth must only be known

b. its depth must only be known

c. its breadth and depth ratio must only be known

d. all the above.

34. For the design of major hydraulic structures on the canals, the method generally preferred to, is based on

a. Bligh's theory

b. Electrical analogy method

c. The relaxation method

d. Khosla's method of independent variables.

35. Lane's weighted creep theory assumes

a. double weightage to horizontal creep and one weightage to vertical creep

b. triple weightage to horizontal creep and one weightage to vertical creep

c. triple weightage to vertical creep and one weightage to horizontal creep

d. double weightage to vertical creep and one weightage to horizontal creep.

36. The velocity of drainage water in the barrels of a syphon-aqueduct, is normally limited to

a. 1 to 2 m per second

b. 2 to 3 m per second

c. 3 to 4 m per second

d. 4 to 5 m per second.

37. Hill roads are

a. provided camber along spur curves

b. provided camber along valley curves

c. seldom provided a camber

d. None of these.

38. If present A.D.T. is 5000 vehicles and annual increase is 10%, the average future flow after 5 years will be

a. 6050 vehicles

b. 7050 vehicles

c. 8050 vehicles

d. 9050 vehicles

39. Width of the shoulders of carriage way is generally kept

a. 125 cm

b. 150 cm

c. 200 cm

d. 250 cm

40. To prevent a head-on-collision of vehicles travelling in opposite directions along four-lane roads

a. physical dividers are provided

b. area dividers are provided

c. medians of wide area are provided

d. none of these.

41. The brick work is not measured in cu m in case of

a. One or more than one brick wall

b. Brick work in arches

c. Reinforced brick work

d. Half brick wall.

42. The floor area includes the area of the balcony up to

a. 100%

b. 75%

c. 50%

d. 25%

43. Pick up the incorrect statement from the following:

a. No deduction is made for the openings upto 0.1 sq.m

b. No deduction is made for volumes occupied by pipes, not exceeding 100 sq.cm in cross-section

c. No deductions are made for the ends of dissimilar structures up to 500 sq.cm cross-sectional area

d. None of these.

44. In the mid-section formula

a. The area of mid-sections is calculated by using mean depth

b. The volume of the earth work is calculated by multiplying the mid-section area by the distance between the two sections

c. The volume of the earth work is calculated by multiplying the mid-section area by the distance between the two original sections

d. (a), (b) and (c) of the above.

45. The time by which activity completion time can be delayed without affecting the start of succeeding activities, is known as

a. duration

b. total flat

c. free float

d. interfering float.

46. Time and progress chart of a construction, is also known as

a. Gantt chart

b. Modified Mile stone chart

c. Critical path method chart

d. All the above.

47. While scheduling a project by C.P.M.

a. required time for each activity is established

b. sequence of various activities is made according to their importance

c. net work is drawn by connecting the activities and the events

d. All the above.

48. Which one of the following represents an activity

a. curing of concrete

b. setting of question paper

c. preparation of breakfast

d. all the above.

49. The coordinates of the ends of a runway are (5000, 5000) and (8000, 7000). The co-ordinates of an other runway are (4600, 5100) and (7000, 5300). The co-ordinates of the A.R.P. are

a. (6500, 6000)

b. (5800, 5200)

c. (61500, 5600)

d. (8000, 7000)

50. For night landing, the thresholds are lighted

a. green

b. red

c. white

d. yellow