***Sub Engineer***

1. The back staff reading on a B.M. of R.L. 500.000 m is 2.685 m. If foresight reading on a point is 1.345 m, the reduced level of the point, is

a. 501.345 m

b. 501.340 m

c. 504.030 m

d. 502.585 m.

2. Removal of parallax, may be achieved by focussing

a. the objective

b. the eye-piece

c. the objective and the eye-piece

d. none of these.

3. Metric chains are generally available in

a. 10 m and 20 m length

b. 15 m and 20 m length

c. 20 m and 30 m length

d. 25 m and 100 m length

4. In levelling operation,

a. The first sight on any change point is a back sight

b. The second sight on any change point is a fore sight

c. The line commences with a fore sight and closes with a back sight

d. The line commences with a back sight and closes with a foresight.

5. For making fly-ash building bricks, the following mix of fly-ash, sand and lime, is

a. 80 : 13 : 7

b. 70 : 20 : 10

c. 60 : 35 : 5

d. none of these.

6. The cracks which extend from bark towards the sap wood in the cross section of a tree, are called

a. radial shakes

b. star shakes

c. heart shakes

d. cup shakes.

7. Brittleness of cold is due to an excess of

a. sulphur

b. carbon

c. phosphorus

d. silicon.

8. For the manufacture of Portland cement, the proportions of raw materials used, are

a. lime 63% ; silica 22% ; other ingredients 15%

b. lime 22% ; silica 63% ; other ingredients 15%

c. silica 40% ; lime 40% ; other ingredients 20%

d. silica 70% ; lime 20% ; other ingredients 10%.

9. A simply supported beam which carries a uniformly distributed load has two equal overhangs. To have maximum B.M. produced in the beam least possible, the ratio of the length of the overhang to the total length of the beam, is

a. 0.207

b. 0.307

c. 0.407

d. 0.508

10. The normal and tangential components of stress on an inclined plane through θ° to the direction of the force, will be equal if θ is

a. 45°

b. 30°

c. 60°

d. 90°

11. The property by which a body returns to its original shape after removal of the force, is called

a. plasticity

b. elasticity

c. ductility

d. malleability.

12. Columns of given length, cross-section and material have different values of buckling loads for different end conditions. The strongest column is one whose

a. both ends are hinged or pin jointed

b. one end is fixed and the other end entirely free

c. both the ends are fixed

d. none of the these.

13. For steady flow in open channels, which one of the following does not change :

a. depth of flow

b. velocity of flow

c. rate of flow

d. All of these.

14. If the atmospheric pressure on the surface of an oil tank (sp. gr. 0.8) is 0.1 kg/cm2, the pressure at a depth of 2.5 m, is

a. 2 metres of water

b. 3 metres of water

c. 3.5 metres of water

d. 4.0 metres of water.

15. Total head of a liquid particle in motion is the sum of

a. potential head and kinetic head

b. kinetic head and pressure head

c. potential head and pressure head

d. potential head, kinetic head and pressure head.

16. The hydrostatic force acts through

a. centre of pressure

b. centre of top edge

c. centre of bottom edge

d. metacentre.

17. Pick up the correct statement from the following:

a. An unconfined compression test is a special case of triaxial compression test

b. An unconfined compression test is a special case of direct shear test

c. The confining pressure is maximum during an unconfined compression test

d. The cylindrical specimen of a soil is subjected to major principal stress till it fails due to shearing along the plane of the failure.

18. Coulomb's wedge theory assumes that

a. slip surface is the plane which passes through the heel of the wall

b. sliding wedge itself acts as a rigid body and the value of earth pressure is obtained by considering the limiting equilibrium of the wedge

c. position and direction of the resultant earth pressure, are known

d. all the above.

19. Pick up the correct statement from the following:

a. The line which shows moisture content dry density relation for soil containing a constant percentage of air voids, is known as air void line

b. The weight of hammer used for compaction test is 25 kg

c. The free fall of hammer for compaction is 30.5 cm

d. All the above.

20. A moist soil sample weighing 108 g has a volume of 60 cc. If water content is 25% and value of G = 2.52, the void ratio is

a. 0.55

b. 0.65

c. 0.75

d. 0.80

21. For a ribbed slab

a. clear spacing between ribs shall not be greater than 4.5 cm

b. width of the rib shall not be less than 7.5 cm

c. overall depth of the slab shall not exceed four times the breadth of the rib

d. all the above.

22. In favourable circumstances a 15 cm concrete cube after 28 days, attains a maximum crushing strength

a. 200 kg/cm2

b. 300 kg/cm2

c. 400 kg/cm2

d. 500 kg/cm2

23. The length of lap in tension reinforcement should not be less than the bar diameter x (actual tension / four times the permissible average bond stress) if it is more than

a. 18 bar diameters

b. 24 bar diameters

c. 30 bar diameters

d. 36 bar diameters

24. For a number of columns constructed in a rcjw, the type of foundation provided, is

a. raft

b. strap

c. strip

d. a combination of the above.

25. The taper of precast concrete pile should not be more than

a. 1 cm per metre length

b. 2 cm per metre length

c. 4 cm per metre length

d. 5 cm per metre length.

26. For heavy embankments and dams, of height h, the depth of exploration of soil should not be less than

a. h/4

b. 1/2 h

c. h

d. 2 h.

27. If the depth of moist sand in a cylinder is 15 cm and the depth of the sand when fully inundated with water is 12 cm, the bulking of the moist sand, is

a. 12%

b. 15%

c. 20%

d. 25%.

28. Expansion joints are provided if the length of concrete structures exceeds

a. 15 m

b. 15 m

c. 35 m

d. 45 m

29. The efficiency of a pumping set, is generally assumed

a. 55%

b. 60%

c. 65%

d. 70%.

30. If the specific capacity of a well is 0.3183 x 10-3 per sec, the discharge from a well of 4 m diameter under a depression head of 4 m, is

a. 10 litres/sec

b. 12 litres/sec

c. 14 litres/sec

d. 16 litres/sec.

31. For drains up to 15 cumecs, the depth d and width B are related by

a. d = 0.2 B

b. d = 0.5 B

c. B = 0.2 d

d. B = 0.5 d.

32. In a sludge tank, the gas mainly produced, is

a. Nitrogen

b. Hydrogen

c. Carbon dioxide

d. Methane.

33. Disposal of extra excavated earth of canals, is utilised to provide a spoil bank on

a. left side

b. right side

c. both sides

d. all the above.

34. For a standing crop, the consumptive use of water is equal to the depth of water

a. transpired by the crop

b. evaporated by the crop

c. transpired and evaporated by the crop

d. used by the crop in transpiration, evaporation and also the quantity of water evaporated from adjacent soil.

35. The ratio of the rate of change of discharge of an outlet to the rate of change in level of water surface in a distributary at its normal depth, is known as

a. efficiency

b. sensitivity

c. flexibility

d. modular limit.

36. When a canal flowing under pressure is carried below a natural drainage such that its F.S.L. does not touch the underside of the supporting structure, the structure so provided, is called

a. syphon

b. aqueduct

c. super passage

d. syphon-aqueduct.

37. For the administration of road transport, a Motor Vehicle Act was enacted in

a. 1934

b. 1939

c. 1947

d. 1950

38. Pick up the incorrect statement from the following. The width of the right-of-way is decided so as to accommodate

a. side slopes

b. horizontal curve

c. vertical curve

d. provision of future widening.

39. For clear distinct vision, images of obstructions should fall on the retina with a cone of

a. 3°

b. 4°

c. 5°

d. 6°

40. The gradients of trace cuts for hilly roads, are kept

a. equal to ruling gradient

b. 10 tO 20 per cent easier than ruling gradients

c. 10 to 20 per cent steeper than ruling gradients

d. none of these.

41. The concrete work for the following part of the building of specified thickness is measured in square metres

a. Floors

b. D.P.C.

c. Wall panels

d. All the above.

42. If tensile stress of a steel rod of diameter D is 1400 kg/cm2 and bond stress is 6 kg/cm2, the required bond length of the rod is

a. 40 D

b. 50 D

c. 53 D

d. 59 D

43. Pick up the correct statement regarding the centre line method of estimating a building

a. Product of the centre line of the walls and area of cross-section of any item, gives total quantity of the item

b. The centre line is worked out separately for different sections of walls of a building

c. The centre line length is reduced by half the layer of main wall joining the partition wall

d. All the above.

44. The expected out turn of half brick partition wall per mason per day is

a. 1.5 m3

b. 2.0 m3

c. 4.0 m2

d. 5.0 m2

45. Which one of the following represents an event ?

a. concrete cured

b. fixing of door

c. plastering of walls

d. selecting sites

46. Frederick W. Taylor introduced a system of working known as

a. line organisation

b. line and staff organisation

c. functional organisation

d. effective organisation.

47. The performance of a specific task in CPM, is known

a. Dummy

b. Event

c. Activity

d. Contract.

48. Optimistic time, most likely time and pessimistic times for the activities of a network in the given figure are written above their arrows. If the contractual obligation time for the project is 75, the latest occurrence time for the event 2, is

a. 20

b. 25

c. 35

d. 15

49. Effective length of a runway is the distance between

a. ends of the runway

b. point of intersection of the obstruction clearance line and the extended plane of the runway surface, and the other end of the runway

c. point of intersection of the glide path and the extended plane of the runway surface and the other end of the runway

d. ends of the clear way on either side.

50. To cope up high temperature of 196°C, the taxi ways and aprons are constructed with

a. rubberised tar concrete

b. plain concrete

c. epoxy asphalt concrete

d. all the above.