***Sub Engineer***

1. The whole circle bearing of a line is 290°. Its reduced bearing is

a. N 20° E

b. N 20° W

c. N 70° W

d. S 70° E

2. Accurate measurement of deflection angles with a transit not properly adjusted may be made by

a. setting the vernier A at zero at back station and then plunging the telescope

b. setting the vernier A at zero at back station and turning the instrument to the forward station

c. taking two back sights one with the telescope normal and the other with telescope inverted

d. none of these.

3. While surveying a plot of land by plane tabling, the field observations

a. and plotting proceed simultaneously

b. and plotting do not proceed simultaneously

c. and recorded in field books to be plotted later

d. all the above.

4. In a telescope the object glass of focal length 14 cm, is located at 20 cm from the diaphragm. The focussing lens is midway between them when a staff 16.50 m away is focussed. The focal length of the focussing lens, is

a. 5.24 cm

b. 6.24 cm

c. 7.24 cm

d. 8.24 cm

5. Acrylic sheets

a. are generally unaffected by most household detergents

b. possess the light transmission rate of 93%

c. are available in various shapes

d. all the above.

6. Stones used for rubble masonry must be

a. soft

b. hard

c. light

d. heavy.

7. Forge pig may be converted to wrought iron by

a. rolling

b. pudding

c. shingling

d. refining.

8. Priming consists of

a. one part of white lead, 8 parts of chalk and four parts of twice boiled linseed oil

b. 8 parts of white lead, one part of chalk and four parts of twice boiled linseed oil

c. one part of white lead, 8 parts of chalk and one part of linseed oil

d. none to these.

9. Maximum strain theory for the failure of a material at the elastic limit, is known as

a. St. Venant's theory

b. Rankine's theory

c. Haig's theory

d. Von Mises's theory.

10. The horizontal thrust on the ends of a two hinged semicircular arch of radius R carrying

a. a uniformly distributed load ω per unit run over its right half span, is

b. a uniformly distributed load ω per unit run over its entire span is

c. a distributed load varying from zero at the left end to ω per unit horizontal run at the right end, is

d. all the above.

11. If the shear force along a section of a beam is zero, the bending moment at the section is

a. maximum

b. minimum

c. average of maximum-minimum

d. none of these.

12. In rectangular columns (cross-section b x h), the core is a

a. square of length b/2

b. rhombus of length h/2

c. rhombus of diagonals b/3 and h/3

d. none of the these.

13. Pick up the correct statement from the following :

a. The fluids which follow  are known as Newtonian fluids

b. The fluids which do not follow the linear relationship between shear and rate of strain are known as non Newtonian fluids

c. The substances which flow after yield strains, are known as plastics

d. all the above.

14. A pitot tube is used to measure

a. pressure

b. difference in pressure

c. velocity of flow

d. none of these.

15. To avoid vapourisation, pipe lines are laid over the ridge so that these are above the hydraulic gradient line, not more than

a. 2.4 m

b. 6.4 m

c. 10.0 m

d. 5.0 m.

16. For a long pipe, the head loss

a. at the entrance is ignored

b. at the outlet is ignored

c. at the entrance and outlet both are ignored

d. due to friction is ignored.

17. The maximum value of effective stress in the past divided by the present value, is defined as over consolidation ratio (OCR). The O.C.R. of an over consolidated clay is

a. less than 1

b. 1

c. more than 1

d. None of these.

18. The ultimate bearing capacity of a soil, is

a. total load on the bearing area

b. safe load on the bearing area

c. load at which soil fails

d. load at which soil consolidates.

19. Which one of the following statements is true ?

a. Clays are more porous than sands

b. Pressure of organic matter in a soil decreases the bearing capacity of the soil

c. Aluminous cement is used for foundations in soils with chemical deposits

d. All the above.

20. The ratio of volume of air voids to the volume of total voids, is known as

a. air content

b. percentage air voids

c. percentage voids

d. porosity.

21. Pick up the incorrect statement from the following: Tensile reinforcement bars of a rectangular beam

a. are curtailed if not required to resist the bending moment

b. are bent up at suitable places to serve as shear reinforcement

c. are bent down at suitable places to serve as shear reinforcement

d. are maintained at bottom to provide at least local bond stress.

22. If permissible working stresses in steel and concrete are respectively 1400 kg/cm2 and 80 kg/cm2 and modular ratio is 18, in a beam reinforced in tension side and of width 30 cm and having effective depth 46 cm, the lever arms of the section, is

a. 37 cm

b. 38 cm

c. 39 cm

d. 40 cm

23. The pitch of the main bars in a simply supported slab, should not exceed its effective depth by

a. four times

b. five times

c. six times

d. seven times.

24. A singly reinforced concrete beam of 25 cm width and 70 cm effective depth is provided with 18.75 cm2 steel. If the modular ratio (m) is 15, the depth of the neutral axis, is

a. 25 cm

b. 30 cm

c. 35 cm

d. 40 cm

25. The minimum distance between the centres of bulb of diameter du, of a multi under reamed piles, is

a. 1.25 du

b. 1.5 du

c. 1.75 du

d. 2 du.

26. Pick up the correct statement from the following :

a. the sand is kept moist at the time of placing and tamping

b. the top one metre of the pile is filled up with cement concrete to provide a cap for the filled up sand

c. sand piles are generally used under column loads

d. all of the above.

27. If the effective working time is 7 hours and per batch time of concrete is 3 minutes, the output of a concrete mixer of 150 litre capacity, is

a. 15, 900 litres

b. 16, 900 litres

c. 17, 900 litres

d. 18, 900 litres

28. The process of proper and accurate measurement of concrete ingredients for uniformity of proportion, is known

a. curing

b. mixing

c. batching

d. none of these.

29. Turbidity of water is expressed

a. in ppm

b. in numbers in an arbitrary scale

c. by pH value

d. by colour code.

30. Alum is a

a. coagulant

b. flocculant

c. catalyst

d. disinfectant.

31. The suitable cross-section of sewers to carry combined flow, is

a. circular

b. egg shaped

c. rectangular

d. horse shoe shaped.

32. The detention period for plain sedimentation water tanks, is usually

a. 4 to 8 hours

b. 8 to 16 hours

c. 16 to 24 hours

d. 24 to 36 hours.

33. Finally formed berms in canals are provided for

a. protection of banks erosion by the waves

b. control of seepage losses

c. strengthening of banks

d. all the above.

34. The Lacey's regime velocity is proportional to

a. R1/2 S3/4

b. Q3/4 S1/3

c. R3/4 S1/3

d. R2/3 S1/2

35. Pick up the correct statement from the following:

a. Duty of canal water is usually calculated at the head discharge of a channel

b. Duty of canal water at the head of a channel and at the outlet of a distributary, differ considerably due to progressive losses of water

c. Duty of canal water at the outlet of a distributary, is generally known as outlet factor or outlet discharge factor

d. All the above.

36. Pick up the correct statement from the following:

a. Static level of water in an open well indicates the position of the water table

b. Saturated pores of a soil in the capillary zone above water table have negative pressure

c. At water table, hydrostatic pressure is zero

d. All the above.

37. The first stage of deciding the alignment of a hill road, is

a. reconnaissance

b. detailed survey

c. trace-out

d. preliminary survey.

38. For the administration of road transport, a Motor Vehicle Act was enacted in

a. 1934

b. 1939

c. 1947

d. 1950

39. Roadway width for a National highways and State highways (two-lanes) is

a. 12 m

b. 9 m

c. 9.5 m

d. 15 m

40. Deviation of the alignment of a trace cut may be permitted in areas involving

a. land slides

b. sand dunes

c. dens

d. none of these.

41. Anti-siphonage pipe is connected to

a. Main soil pipe

b. Bottom of P trap W.C.

c. Top of P trap W.C.

d. Side of water closet.

42. The trap which is provided to disconnect the house drain from the street sewer is called

a. Intercepting trap

b. Interception manhole

c. Intercepterchamber

d. All the above.

43. Pick up the correct statement regarding the centre line method of estimating a building

a. Product of the centre line of the walls and area of cross-section of any item, gives total quantity of the item

b. The centre line is worked out separately for different sections of walls of a building

c. The centre line length is reduced by half the layer of main wall joining the partition wall

d. All the above.

44. The total length of a cranked bar through a distance (d) at 45° in case of a beam of effective length L, is

a. L + 0.42 d

b. L + 2 x 0.42 d

c. L - 0.42 d

d. L - 2 x 0.4 d

45. Pick up the correct statement from the following:

a. Backward pass is used for calculating the latest occurence time

b. Maximum value of earliest expected time is used if there are more than one value of any event

c. Minimum value of latest occurrence time is used if there are more than one value of any event

d. All the above.

46. Pick up the PERT event from the following:

a. Digging of foundation completed

b. Laying of concrete started

c. Laying of concrete completed

d. All the above.

47. Various activities of a project, are shown on bar charts by

a. vertical lines

b. horizontal lines

c. dots

d. crosses.

48. Modular co-ordination of construction means proper

a. planning

b. designing

c. execution

d. all the above.

49. Effective length of a runway is the distance between

a. ends of the runway

b. point of intersection of the obstruction clearance line and the extended plane of the runway surface, and the other end of the runway

c. point of intersection of the glide path and the extended plane of the runway surface and the other end of the runway

d. ends of the clear way on either side.

50. Beaufort scale is used to determine

a. strength of winds

b. direction of winds

c. height of air-crafts

d. none of these.